| Forest Carbon Partnership Facility (FCPF) Carbon Fund ER Monitoring Report (ER-MR) |  |
| --- | --- |
| ER Program Name and Country: | People and Forests- A Sustainable Forest Management -Based Emission Reduction Program in the Terai Arc Landscape, Nepal. |
| Reporting Period covered in this report: | 01-01-2022 to 31-12-2024 |
| Number of FCPF ERs: | 2,538,991 |
| Quantity of ERs allocated to the Uncertainty Buffer: | 412,171 |
| Quantity of ERs allocated to the Pooled Reversal Buffer: | 483,616 |
| Number of FCPF ERs from enhanced Removals through afforestation/reforestation (A/R) | NA |
| Date of Submission: | 03-04-2026 |
| Version | 1.0 |

WORLD BANK DISCLAIMER The boundaries, colors, denominations, and other information shown on any map in ER-MR does not imply on the part of the World Bank any legal judgment on the legal status of the territory or the endorsement or acceptance of such boundaries. The Facility Management Team and the REDD Country Participant shall make this document publicly available, in accordance with the World Bank Access to Information Policy and the FCPF Disclosure Guidance.

Purpose of the ER-MR

ER Programs that have been included in the portfolio of the FCPF Carbon Fund shall implement the ER Program and report on performance, in particular ERs generated. By completing and submitting the ER Monitoring Report, a REDD Country Participant or its authorized entity officially reports on its performance to the Carbon Fund.

The FCPF Glossary of Terms provides definitions of specific terms used in the Methodological Framework, Buffer Guidelines and other requirements. Unless otherwise defined in this ER-MR template, any capitalized term used in this ER-MR template shall have the same meaning ascribed to such term in the FCPF Glossary of Terms.

Guidance on completing the ER-MR

All sections of the ER-MR shall be completed. If sections of the ER-MR are not applicable, explicitly state that the section is “Intentionally left blank” and provide an explanation why this section is not applicable.

All instructional text, including this section, must be deleted before submitting the final ER-MR to the FCPF Facility Management Team. Failure to remove instructions may result in a request for resubmission.

Font of the body text shall be Calibri 10 black regular (non-italic) font.

Provide definitions (for those not already defined within the FCPF Glossary of Terms) of key terms that are used and use these key terms and their variables consistently, using the same abbreviations, formats, subscripts, etc. If the ER–MR contains equations, please number all equations and define all variables used in these equations, with units indicated.

The presentation of values in the ER-MR, including those used for the calculation of Emission Reductions, should be in international standard format e.g., 1,000 representing one thousand and 1.0 representing one. Please use units defined and/or mentioned by the International System Units (SI units – refer to http://www.bipm.fr/enus/3_SI/si.html) unless the MF or the IPCC Guidelines indicate otherwise (e.g., tons vs Mg).

REDD Country Participants should note that if the Reporting Period does not coincide with the beginning and end of a natural year, it shall apply the Guidelines on the application of the MF Number 3 on reporting periods. In this case, net ERs shall be estimated for the Monitoring Period and they shall be allocated to the Reporting Period pro-rata on the number of months. In the template Monitoring Report refers to the period used for monitoring ERs, while Reporting Period refers to the period defined in the ERPA and for which ERs are paid for.

REDD Country Participants should also note that if Technical Corrections to the Reference Level (RL) have been applied in accordance with the Guidelines on the application of the methodological framework number 2 on technical corrections, then the technically corrected RL shall be reported in Annex 4 and will be subject to Validation by the Validation and Verification Body.

Table of Contents

1	Implementation and operation of the ER Program during the Reporting Period	10

1.1	Implementation status of the ER Program and changes compared to the ER-PD	10

1.2	Update on major drivers and lessons learned	23

1.3	Methodological deviations	25

2	System for measurement, monitoring and reporting emissions and removals occurring within the monitoring period	26

2.1	Forest Monitoring System	26

2.2	Updates to the monitoring approach	30

2.3	Measurement, monitoring and reporting approach	31

2.3.1	Line Diagram	41

2.3.2	Calculation	41

3	Data and parameters	49

3.1	Fixed Data and Parameters	49

3.2	Monitored Data and Parameters	67

4	Quantification of Emission Reductions	73

4.1	ER Program Reference level for the Monitoring / Reporting Period covered in this report	73

4.2	Estimation of Emissions by Sources and Removals by Sinks included in the ER Program’s scope	73

4.3	Calculation of Emission Reductions	74

4.4	  Identification, assessment and addressing sources of Uncertainty	79

4.5	Uncertainty of the estimate of Emission Reductions	90

Parameters and assumptions used in the Monte Carlo method	90

Quantification of the Uncertainty of the estimate of Emission Reductions	93

4.6	Sensitivity analysis and identification of areas of improvement of MRV system	93

5	Transfer of Title to ERs	96

5.1	Ability to transfer title	96

5.2	Implementation and operation of Program and Projects Data Management System	98

5.3	Implementation and operation of ER Transaction Registry	99

5.4	ERs transferred to other entities or other schemes	100

6	Reversals	105

6.1	Occurrence of major events or changes in ER Program circumstances that might have led to the Reversals during the Reporting Period compared to the previous Reporting Period(s     	105

6.2	Quantification of Reversals during the Reporting Period	105

6.3	Quantification of Pooled Reversal Buffer replenishments	106

6.4	Reversal Risk assessment	107

7	Emission Reductions available for transfer to the Carbon Fund	111

Annex 2: Information on the implementation of the Benefit-Sharing Plan	113

Annex 3: Information on the generation and/or enhancement of priority Non-Carbon Benefits	119

Annex 5: DETAILED DESCRIPTION of the application of the Reversal risk assessment tool	123

LIST OF ACRONYMS

| ACOFUN | Association of Collaborative Forest Users, Nepal |
| --- | --- |
| AD | Activity Data |
| AEPC AFOLU | Alternative Energy Promotion Center Agriculture Forestry and Other Land Use |
| AGB | Above-ground Biomass |
| BGB | Below-ground Biomass |
| BSM | Benefit Sharing Mechanism |
| BSP CA | Benefit Sharing Plan Conservation Area |
| CATS | Carbon Assets Tracking System |
| CBFM CBO CCBA | Community Based Forest Management Community Based Organization Climate, Community and Biodiversity Alliance |
| CCDC-SMA | Continuous Change Detection and Classification – Spectral Mixture Analysis |
| CCSP | Concentric Circular Sample Plot |
| CEO | Collect Earth Online |
| CF | Carbon Fund |
| CFUG | Community Forest User Group |
| CI | Confidence Interval |
| CODED | Continuous Degradation Detection |
| CSO | Civil Society Organization |
| dbh | Diameter at Breast Height |
| DFMP | District Forest Management Plan |
| DFO | Division Forest Office |
| DFRS | Department of Forest Research and Survey |
| DNPWC EF | Department of National Parks and Wildlife Conservation Emission Factors |
| EIA | Environmental Impact Assessment |
| ER | Emission Reduction |
| ERMR | Emission Reduction Monitoring Report |
| ERPA | Emission Reduction Payment Agreement |
| ERPD | Emission Reduction Program Document |
| ESMF | Environmental and Social Management Framework |
| ESMP | Environmental and social management plan |
| FAO | Food and Agricultural Organization |
| FCPF | Forest Carbon Partnership Facility |
| FDF | Forest Development Fund |
| FECOFUN FenFIT FGRM | Federation of Community Forest Users, Nepal Federation of Forest Based Industry and Trade, Nepal Feedback and Grievance Redress Mechanism |
| FMT | Fund Management Team |
| FPIC | Free, Prior and Informed Consent |
| FRA | Forest Resource Assessment |
| FREL FRL | Forest reference Emission Level Forest Reference Level |
| FRTC | Forest Research and Training Centre |
| FSCMD FUG | Forest Survey and Carbon Monitoring Division Forest User Group |
| GEE GESI | Google Earth Engine Gender, Equality, and Social Inclusion |
| GFOI | Global Forest Observations Initiative |
| GHG | Greenhouse Gases |
| GoN | Government of Nepal |
| GRM | Grievance redressal mechanism |
| ha | hectares |
| HR | High Resolution |
| ICS | Improved Cook Stove |
| ID | Inventory Design |
| IEE ILO | Initial Environmental Examination International Labour Organization |
| IP | Indigenous People |
| IPCC | Inter-governmental Panel on Climate Change |
| IPLC | Indigenous Peoples and Local Communities |
| IPO | Indigenous People Organization |
| IPP | Indigenous Peoples Plan |
| LC | Land Cover(s) |
| LHF LiDAR LPG | Leasehold Forestry Light Detection and Ranging Liquefied Petroleum Gas |
| MC MGD | Monte Carlo Methods Guidance Document |
| MoFE MoITC | Ministry of Forests and Environment Ministry of Industry, Tourism, Forests and Environment |
| MoITFE | Ministry of Industry, Tourism, Forests and Environment (Province level) |
| MoLCPA | Ministry of Land Management, Cooperatives and Poverty Alleviation |
| MRV MSPA | Measurement, Reporting and Verification Morphological Spatial Pattern Analysis |
| MTDD | Multivariate Time-series Disturbance Detection |
| MuAN NEFIN | Municipal Association of Nepal Nepal Federation of Indigenous Nationalities |
| NP NFDI NDVI NDWI NFD | National Park Normalized Degradation Fraction Index Normalized Difference Vegetation Index Normalized Difference Water Index National Forest Database |
| NFI | National Forest Inventory |
| NFIS | National Forest Information System |
| NFMIS NGO | National Forest Monitoring and Information System Non-Governmental Organization |
| NLCMS | National Land Cover Monitoring System |
| NRC | National REDD+ Center |
| NRCC | National REDD+ Coordination Committee |
| NRSC NTFP OC | National REDD+ Steering Committee Non-Timber Forest Production Operation Committee |
| PA PDF PRCC | Protected Area Probability Density Function Provincial REDD+ Coordination Committee |
| PPE PSP QA/QC | Personal Protective Equipment Permanent Sampling Plots Quality Assurance and Quality Control |
| REDD IC RL | REDD Implementation Centre Reference Level |
| RP | Reference Period |
| RWG | REDD Working Group |
| SAVI | Soil-Adjusted Vegetation Index |
| ScFM SD SE | Scientific forest management Standard Deviation Standard Error |
| SESA | Strategic environmental and social assessment |
| SFM | Sustainable Forest Management |
| SIS SOC | Safeguards Information System Soil Organic Carbon |
| SoI SU SWIR | Summary of information (on safeguards) Sampling Unit Short Wavelength InfraRed |
| TAL TCC UNDRIP | Terai Arc Landscape Tree Canopy Cover United Nations Declaration on the Rights of Indigenous Peoples |
| USGS | United States Geological Survey |
| VHR | Very High Resolution |
| WB | The World Bank |

# Implementation and operation of the ER Program during the Reporting Period

## Implementation status of the ER Program and changes compared to the ER-PD

The Government of Nepal (GoN) has Nepal has received a payment of $9.4 million from the World Bank’s Forest Carbon Partnership Facility (FCPF) for reducing approximately 1.88 million tons of carbon dioxide under its REDD+ Emission Reductions (ER) Program in the Terai Arc Landscape, as the results-based payments for the verified emissions reduction credits generated during the 1st reporting period (June 22, 2018 - December 31, 2021). Seven strategic interventions stipulated in the Emission Reduction Program Document (ER-PD) of the Emission Reduction (ER) Program, People and Forests:  A Sustainable Forest Management-Based Emission Reduction Program in the Terai Arc Landscape, Nepal, has been implemented in the 13 districts of the Terai Arc Landscape under the Carbon Fund of the FCPF. Implementation of the ER Program interventions continued for the 2nd reporting period (January 1, 2022 - December 31, 2024) in the 13 districts of the Terai Arc Landscape, where the government aimed to further enhance its emissions reduction strategies.

The ER-PD, which received approval in 2018, is strategically designed to achieve over 80% of its emission reduction credits through interventions centered on sustainable forest management. This comprehensive approach encompasses several key initiatives aimed at enhancing forest stewardship. One of the primary strategies involves improving management practices within existing communities and collaborative forests, drawing upon traditional and customary methods that have proven effective over time. Additionally, the program seeks to localize forest governance by empowering community and collaborative forest user groups through the transfer of national forests, thereby fostering a sense of ownership and responsibility among local stakeholders. The initiative also emphasizes the expansion of private sector forestry operations, which will be facilitated by better access to extension services and financial resources, ultimately promoting sustainable economic growth in the region.

Furthermore, the ER-PD aims to broaden access to alternative energy solutions, such as biogas and improved cookstoves, which are essential for reducing reliance on traditional biomass fuels. The program also focuses on scaling up pro-poor leasehold forestry, ensuring that marginalized communities benefit from forest resources. Another critical aspect of the ER-PD is the enhancement of integrated land-use planning, which is intended to mitigate forest conversion linked to infrastructure development. Lastly, the initiative includes efforts to strengthen the management capacity of protected areas, which not only contributes to carbon reduction but also generates significant non-carbon benefits, such as the conservation of biodiversity. Through these multifaceted strategies, the ER-PD aims to establish a comprehensive framework for forest management that prioritizes sustainability and resilience. This initiative seeks to tackle the underlying causes of deforestation and forest degradation, which are critical factors contributing to increased emissions from forested areas. By addressing these drivers, the framework not only aims to mitigate forest-based emissions but also strives to deliver a range of non-carbon benefits that enhance the overall health of forest ecosystems. These benefits may include improved biodiversity, enhanced water quality, and increased opportunities for local communities to engage in sustainable practices.

On February 24, 2021, GoN and the World Bank signed the Emission Reductions Payment Agreement (ERPA), which outlines two reporting periods for performance-based payments. The agreement stipulates the transfer of 9,000,000 emission reduction (ER) units (Tons of CO2 e) to the FCPF over a total duration of 6.53 years, at the rate of USD 5.00credit, with of the verified emission reductions credit, with the first reporting period spanning from June 22, 2018, to December 31, 2021, and the second period from January 1, 2022, to December 31, 2024. Nepal has already received results-based payments for the verified Emission Reductions Credits from the first reporting period. This Emission Reduction Monitoring Report (ER-MR) focuses on the performance of the ER program interventions during the second reporting period of three years, following the FCPF ER monitoring report template, version 3.2, released in April 2026.

The design and key assumptions of the ER program remained consistent throughout the reporting period, with the program continuing to implement the seven interventions outlined in the ER-PD.

    - Progress on the actions and interventions under the ER Program

Seven ER Program interventions have been implemented in the 13 ER Program districts of the TAL. These are:

Intervention 1 (Improvement of Management Practices through SFM Principles) is aligned with the Strategy 1 (Reduce carbon emissions, enhance forest carbon stocks, and improve the supply of forest products) of the National REDD+ Strategy (NRS), 2018-2022, and is focused on enhancing management practices within the existing community and collaborative forests, as well as the government managed block forests, which are suitable for production oriented active Sustainable Forest Management (SFM) to boost timber and fuelwood production, as well as biomass, while addressing local needs. The intervention involves implementing the shelterwood silvicultural system in which trees are removed in a series of cuts designed to achieve a new stand under the shelter of remaining trees. The REDD Implementation Centre has supported the development of SFM plans (in most cases) which are executed by Division Forest Offices. Key activities include silvicultural treatments, and protective measures against illegal logging and grazing, supported by stringent law enforcement and community monitoring teams. Additionally, the initiative promotes forest plantation on open spaces and public lands, alongside improved methods for harvesting and utilizing forest resources.

Intervention 2 (Transferring government managed forests to the CBFM Regimes) focused on decentralizing forest governance by transferring government-managed forests to community and collaborative forest user groups under the Forest Act and Forest Regulations.  This initiative is closely tied to the first intervention, as the forest management operational plans (based on the SFM principles) should have been developed and approved before handing over the forests to the CBFM regimes.

Intervention 3, which aims to expand private forestry, is closely aligned with Strategy 3 of the NRS for 2018-2022. This strategy highlights the significance of promoting both private and public forestry initiatives to improve sustainable forest management and enhance carbon sequestration. By encouraging private landowners to adopt responsible forest management practices, the intervention not only supports the economic growth of the country through increased supply of forest products for related industries but also strives to establish a network of well-managed forests that function as effective carbon sinks

Intervention 4 focuses on broadening access to alternative energy sources, specifically through the implementation of biogas systems and the distribution of improved cookstoves. This is aligned with Strategy 8 of the NRS, 2018-2022 (Increase access to sustainable, affordable, and reliable alternative energy). The primary objective of this initiative is to mitigate the environmental impact associated with the collection of wood fuel from forests, which often leads to degradation of forests and increased greenhouse gas emissions. By promoting biogas, which harnesses organic waste for energy production, and improved cookstoves that enhance fuel efficiency, this intervention seeks to provide sustainable energy solutions that not only reduce reliance on traditional biomass but also contribute to cleaner air and healthier living conditions for communities. In addition to environmental benefits, expanding access to these alternative energy sources can have significant socio-economic implications. The adoption of biogas technology can empower local communities by providing a reliable energy source for cooking and heating, thereby reducing the time and effort spent on fuel collection. Improved cookstoves, designed to burn fuel more efficiently, can lower fuel costs and decrease indoor air pollution, which is a major health risk in many households.

Intervention 5 focuses on the expansion of pro-poor leasehold forestry (LHF) in the ER Program Area, a vital component of the Community-based Forest Management Regimes. This innovative program has garnered recognition for its significant impact on enhancing the livelihoods and employment prospects of rural communities, particularly among the economically disadvantaged. By granting local communities the right to manage and utilize forest resources, the LHF initiative not only empowers these groups but also fosters sustainable practices that contribute to the overall health of forest ecosystems. As a result, the program plays a crucial role in improving forest conditions and helps mitigate forest-based emissions.

Intervention 6 (Improve integrated land use plans to reduce forest conversion associated with infrastructure development) is closely linked to the development of infrastructure and is in accordance with Strategy 4 of the National REDD+ Strategy for the period 2018-2022, which emphasizes the promotion of land use across various physiographic regions. The primary aim of this intervention is to mitigate deforestation by fostering integrated land use planning and its execution within the infrastructure development projects in the districts participating in the Emission Reduction (ER) program. Local governments within the ER Program area, including municipalities and rural municipalities, have developed land-use plans to guide sustainable land management and infrastructure development. These plans provide an important framework for balancing development needs with environmental conservation objectives. By integrating land-use planning into local development processes, the initiative aims to minimize deforestation and forest degradation while promoting sustainable land management practices that generate long-term ecological, social, and economic benefits for local communities. The list of local governments in ER Program Area with Integrated Land Use Planning during the reporting period is shown below:

| District | Municipality/Rural Municipality | Fiscal Year |
| --- | --- | --- |
| Rupandehi | Siddharthanagar Municipality | 2022/23 |
|  | Mayadevi Rural municipality | 2021/22 |
|  | Marchawari Rural Municipality | 2022/23 |
|  | Lumbini Sanskritik Municipality | 2022/23 |
| Banke | Rapti Sonari Rural Municipality | 2022/23 |
| Nawalpur | Devchuli Municipality | 2021/22 |
| Chitwan | Ratnanagar municipality | 2024/25 |
| Dang | Rajpur Rural municipality | 2022/23 |
|  | Rapti municipality | 2022/23 |
| Kapilvastu | Bijayanagar Rural municipality | 2022/23 |
| Kanchanpur | Laljhadi Rural Municipality | 2022/23 |
| Parasi | Ramgram Municipality | 2022/23 |
| Kailali | Lamkichuha Municipality | 2022/23 |
|  | Mohanyal Rural municipality | 2022/23 |

Intervention 7, which focuses on enhancing the management of protected areas, aligns closely with Strategy 2 of the National REDD+ Strategy for 2018-2022, aimed at increasing the non-carbon benefits of forest ecosystems. The ER program encompasses six protected areas, including five National Parks and one Conservation Area, collectively spanning 341,997 hectares. The buffer zones surrounding these National Parks extend over 210,617 hectares across 39 local government jurisdictions, managed under the Buffer Zone Community Forest Users Group, which are also the recognized beneficiaries of the results-based payments of the ER-Program. Given that carbon stocks in these protected areas significantly exceed those found in other management types, it is crucial to bolster their management to preserve and enhance forest carbon stocks. These protected areas, managed by the Department of National Parks and Wildlife Conservation (DNPWC), benefit from stringent protection measures that have historically shielded them from deforestation and degradation. Their inclusion in the ER Program highlights their vital contributions to carbon sequestration, ecosystem services, and biodiversity conservation. However, due to the National Park and Wildlife Conservation Act of 1973, which restricts active management within National Parks, any emission reductions achieved in these areas are ineligible for results-based payments.

Progress on the actions and interventions under the ER Program for the 2nd reporting period is provided in Table 1.

Table 1. ER Program progress on actions and interventions for the 2nd reporting period

| ER-PD Target | Progress during the 1st reporting period | Progress during the 2nd reporting period | Remarks/Data source |
| --- | --- | --- | --- |
| Intervention 1: Improve management practices in the existing community, and collaborative forests building on the traditional and customary practices. |  |  |  |
| 336,069 ha | 154,766 ha (total) Community Forests: 94,236 ha. Collaborative Forests: 52,515 ha Block Forests: 8,015 ha | 217,851.481 ha (total) Community Forests: 178251.19 ha. Collaborative Forests: 39600.291 ha |  |
| Intervention 2. Localize forest governance through transfer of National Forests to Community and Collaborative Forest User Groups |  |  |  |
| 200,937 ha | 12,107 ha handed over to the communities. Community Forest: 9,454 ha Collaborative Forest: 2,653 ha | Community forest: 8577.415 ha |  |
| Intervention 3. Expand private sector forestry operations through improved access to extension services and finance |  |  |  |
| 30,141 ha | 190 new private forests of 114 ha registered. | 50 new private forests of 17.42 ha registered |  |
| Intervention 4a. Expand access to alternative energy with biogas |  |  |  |
| 60,000 units | 2,382 units | 3674 units |  |
| Intervention 4b. Expand access to alternative energy with improved cookstoves |  |  |  |
| 60,000 units | 3,728 units | 431 units |  |
| Intervention 5. Scale up pro-poor leasehold forestry |  |  |  |
| 12,056 ha | 10.49 ha forest registered as the Leasehold forest | 74.25 ha forest registered as leasehold forest |  |
| Intervention 6. Improve integrated land use plans to reduce forest conversion associated with infrastructure development |  |  |  |
| 9,000 ha | Land use plans developed in 44 (out of 144) local government jurisdictions in the ER program area | Land use planning in 14 (out of 144) local government jurisdictions in ER program area |  |
| Intervention 7. Improve management of existing Protected Areas (PAs) |  |  |  |
| 6 PAs | 155 ha of grassland managed (Chitwan and Bardia National Parks) 600 ha of new grassland constructed in Chitwan National Park | 3600 ha of grassland managed, 136 number of water hole maintenance, (Chitwan national park, Bardiya national park, Banke national park, Shuklaphanta national park, Parsa National Park and Krishnasar Conservation area) |  |

1.1.2 Update on the strategy to mitigate and/or minimize potential Displacement

Following the approval of National REDD Strategy (NRS), 2018-2022, a range of forestry and cross-sectoral policy and legislative instruments were developed to enhance the governance of forest resources in Nepal. Key documents include the National Forest Policy (2019), the National Agroforestry Policy (2019), and the Forest Act (2019) along with its accompanying Regulations (2022). These instruments are crucial for the forestry sector and are directly linked to the objectives of REDD+. Additionally, the Environment Protection Act (2019) and its Regulations (2020), as well as various Provincial Forest Acts and Regulations, play significant roles in shaping the legal framework that supports sustainable forest management. On the cross-sectoral front, policies such as the National Climate Change Policy (2019), the Industrial Enterprise Act (2020), the Land Use Act (2019) and its Regulations (2022), and the Lands Act (8th Amendment, 2019) are pertinent as they address the utilization of forest land for diverse purposes, with several provisions directly impacting REDD+ initiatives and forest carbon strategies.

The Environment Protection Act (2019) and the Forest Act (2019), both of which include measures aimed at reducing or minimizing emissions displacement during the implementation of Emission Reduction (ER) Programs. The Environment Protection Act was officially implemented in 2020 following the approval of the Environment Protection Regulations in the same year. Likewise, the Forest Act came into effect after the Forest Regulations were sanctioned in 2022. Notably, Article 28 of the Environment Protection Act empowers the Government of Nepal to engage in carbon trading, aligning with international treaties aimed at reducing emissions and enhancing carbon stocks. The Forest Act further delineates provisions for forest carbon trading and the equitable distribution of benefits derived from such activities, specifically outlined in Article 44 (a) and (b) of Section 13, which facilitates the establishment of the Forest Development Fund (FDF).

In addition to these foundational laws, the Forest Regulation (2022) elaborates on the processes for developing emission reduction programs, as detailed in Rule 107. It also addresses the Free, Prior, and Informed Consent (FPIC) process in Rule 107 (6) and outlines the distribution of carbon benefits through the decision-making framework of the FDF, guided by the benefit-sharing plan articulated in Rules 107 (7-8) and 115. The Government of Nepal has demonstrated a strong commitment to REDD+, as highlighted in the budget speech for the fiscal year 2023/24, 2024/25, and 2025/26, which emphasizes the continuation of carbon trading initiatives through REDD+ and other emission reduction strategies throughout the nation. This commitment underscores the government's recognition of the importance of sustainable forest management and climate change mitigation efforts.

In 2020, Nepal further advanced its commitment to climate action by developing the Second Nationally Determined Contribution (NDC) and its Implementation Plan for the period 2023-2030. This strategic framework outlines the country's emissions reduction targets and the methodologies to achieve them, reflecting a comprehensive approach to addressing climate change. Additionally, Nepal's Long-term Strategy for Net-zero Emissions, established in 2021, underscores the nation’s dedication to sustainable development and environmental stewardship. These initiatives not only align with global climate agreements but also demonstrate Nepal's proactive stance in contributing to international efforts to mitigate climate change, particularly through forest conservation and sustainable land management practices.

The Ministry of Forests and Environment (MoFE), along with its various divisions, departments, and centers, as well as the provincial ministries of forests and environment and Division Forest Offices (DFOs), play a crucial role in managing the risks associated with emissions displacement during the implementation of REDD+ programs. These entities are tasked with taking proactive measures to mitigate potential negative impacts that may arise from such initiatives. Additionally, Community-Based Forest Management (CBFM) groups are also integral to this process, as they are responsible for executing relevant policies and strategies within their respective jurisdictions to effectively address the risks of emission displacement.

Moreover, the Forest Research and Training Centre (FRTC) holds the responsibility for the ongoing monitoring and identification of any instances of emissions displacement that may occur. The MoFE, in collaboration with the REDD Implementation Centre (REDD IC), is charged with the development of comprehensive policies and strategies aimed at assessing the risks associated with emissions displacement. This collaborative effort ensures that appropriate measures are taken to address these risks, thereby promoting sustainable forest management and environmental protection in the context of REDD+ initiatives.

Nepal has several policies and laws relevant to reducing the risk of displacement, for example the National Forest Policy (2019) and Forests Act (2019), which strictly prohibit conversion of forest to non-forest land use system in all types of forests, which will help reduce the risk of displacement of emissions. REDD IC has been integrating into consideration of the risk of displacement in the design, prioritization, implementation, and regular assessment of REDD+ policies and programs. For example, the NRS 2018-2022 has various national scale actions on forest monitoring, which will also help reduce the risk of displacement of emissions. These include:

- Increase supply of sustainably harvested timber and timber products with improved distribution mechanisms. (Action 1.8)

- Promote agroforestry in public land such as canals, roadside areas, marginal lands, and riverbanks through regulatory frameworks and incentive mechanisms, with participation of poor, women and marginalized households. (Action 3.3)

- Promote implementation of the Land Use Policy (2015), particularly provisions related to the forestry sector, and update zoning and mapping of forest land use regularly. (Action 4.1)

- Strengthen enforcement and monitoring capacity of district level land encroachment control committees and law enforcement agencies to reclaim illegally occupied forest lands. (Action 4.5)

- Control cross-border illegal trade of forest products through intercountry cooperation with Indian and Chinese authorities. (Action 9.7)

- Avoid forest areas in infrastructure development and resettlement and make compulsory provisions for tree planting to compensate for forest areas being cleared. (Action 11.5)

- Establish and maintain a National Forest Monitoring System with robust measurement, monitoring, reporting and verification mechanisms (Action 11.6) and establish a well-functioning Forest Management Information System under the NFMS. (Action 12.3).

- Enhance national capability with investment, technology, and human resources to conduct forest resource surveys and inventories. (Action 12.1)

- Strengthen community-based monitoring systems with identified monitoring indicators in community-based forest management. (Action 12.5)

- Establish spatially explicit information systems on land use potential, allocations, and potential conflicts/complementarity with REDD+ strategies. (Action 12.6)

Some other measures related to REDD+ design, implementation and monitoring that will reduce the risk of emissions displacement include:

- Nepal’s FRL is compiled at national level, which is advantageous to avoid internal displacement of emissions and to ensure that the impact of national policies and measures can be properly assessed.

- The NRS also includes strategies or REDD+ actions that are undertaken nationally, which are linked to national scale policies and measures (such as CBFM and alternative energy promotion).

- Forest monitoring and evaluation of forest related emissions and changes in forest carbon stock at national, provincial, regional, and local levels by mobilizing experts, forest bureaucracy, and local communities, in order to detect displacement

- Nepal established a National Forest Monitoring System (NFMS), comprising the National Land Cover Monitoring System and NFIS. FRTC is the national MRV agency and conducts forest cover monitoring, and the national forest inventory, and provides activity data and emission/removal factors. The NFMS provides information that is transparent, consistent over time, suitable for measuring, reporting and verifying anthropogenic forest-related emissions by sources, and removals by sinks, forest carbon stocks, and forest-area changes; including the information on the displacement of the emissions if any.

Revised drivers of deforestation and forest degradation and risk of emissions displacement are discussed in Section 1.2

    - Effectiveness of the organizational arrangements and involvement of partner agencies

The institutional framework for decision-making and implementation of the ER program under the REDD+ in Nepal is notably robust and effective. The National REDD+ Steering Committee (NRSC), a 25-member body led by the Minister for Forests and Environment, which serves as the primary policy-making institution for REDD+ initiatives in the country. Complementing this high-level committee is the National REDD+ Coordination Committee (NRCC), established in February 2020 and chaired by the Secretary of the Ministry of Forests and Environment (MoFE). This 25-member committee focuses on addressing technical aspects of REDD+ implementation, ensuring that decisions are informed by expert knowledge and practical considerations. Additionally, the REDD Implementation Centre (REDD IC), which evolved from the former REDD-Forestry and Climate Change Cell established in 2009, plays a crucial role in coordinating and facilitating REDD+ activities nationwide.

To further enhance the effectiveness of REDD+ initiatives, dedicated REDD+ desks have been established within provincial forest directorates. These desks are tasked with coordinating and facilitating REDD+-related activities at both provincial and district levels, thereby ensuring a comprehensive approach to implementation. Moreover, the establishment of multi-stakeholder forums, along with alliances of Indigenous Peoples (IPs) and Civil Society Organizations (CSOs), provides essential platforms for sharing experiences, discussing progress, and gathering feedback throughout the REDD+ implementation process. This inclusive approach not only fosters collaboration among various stakeholders but also enhances transparency and accountability in the execution of REDD+ strategies, ultimately contributing to the program's success in addressing deforestation and promoting sustainable forest management in the ER Program area.

National REDD+ Steering Committee (NRSC)

The NRSC serves as the principal authority at the national level, tasked with establishing comprehensive policy guidance, fostering collaboration among various agencies involved in REDD+ initiatives, and supervising operational protocols. Chaired by the Honorable Minister of Forests and Environment, the NRSC comprises representatives from government bodies, non-governmental organizations, academic institutions, networks and associations of Indigenous Peoples and Local Communities, as well as independent experts, ensuring a diverse range of perspectives and expertise (see Table 2). Typically, the NRSC convenes annually, although additional meetings may be scheduled as necessary to address pressing issues or developments in the REDD+ framework.

Table 2. Composition of National REDD+ Steering Committee

| S.N. | Designation/Organization | Position |
| --- | --- | --- |
| 1 | Honorable Minister, Ministry of Forests and Environment | Chairperson |
| 2 | Honorable Member (Forestry and Environment Portfolio), National Planning Commission | Member |
| 3 | Secretary, OPMCM (Provincial Coordination Portfolio) | Member |
| 4 | Secretary, Ministry of Forests and Environment | Member |
| 5 | Secretary, Ministry of Finance | Member |
| 6 | Secretary, Ministry of Agriculture and Livestock Development | Member |
| 7 | Secretary, Ministry of Energy, Water Resources and Irrigation | Member |
| 8 | Secretary, Ministry of Industry, Commerce and Supply | Member |
| 9 | Secretary, Ministry of Land Management, Cooperatives and Poverty Alleviation | Member |
| 10 | Secretary, Ministry of Physical Infrastructure and Transport | Member |
| 11 | Secretary, National Natural Resources and Fiscal Commission | Member |
| 12 | Two Professors of Forestry/Environmental Science (at least one woman) nominated by the MoFE | Member |
| 13 | Chairperson, Federation of Community Forest Users Group Nepal (FECOFUN) | Member |
| 14 | Chairperson, Association of Collaborative Forest Users, Nepal (ACOFUN) | Member |
| 15 | Chairperson, Nepal Federation of Indigenous Nationalities (NEFIN) | Member |
| 16 | President, Federation of Forest Based Industry and Trade, Nepal (FenFIT) | Member |
| 17 | Chairperson, Municipal Association of Nepal (MuAN) | Member |
| 18 | Chairperson, National Association of Rural Municipalities in Nepal | Member |
| 19 | Two representatives from Women and Dalit-related networks/associations nominated by the Ministry of Forests and Environment | Member |
| 20 | Two independent experts (at least one woman) nominated by the MoFE | Member |
| 21 | Gender Focal Person, Ministry of Forests and Environment | Member |
| 22 | Chief, REDD Implementation Centre | Member Secretary |

National REDD+ Coordination Committee (NRCC)

The NRCC, led by the Secretary of the Ministry of Forests and Environment (MoFE), holds significant responsibilities primarily focused on technical decision-making. This includes the endorsement of various study and research documents, overseeing the implementation and monitoring of REDD+ programs, and proposing agendas for the National REDD+ Steering Committee (NRSC) meetings. The structure of the NRCC is detailed in Table 3, which outlines its composition. Typically, the NRCC convenes at least biannually, although additional meetings may be scheduled as necessary to address pressing issues or developments in the field.

Table 3. Composition of National REDD+ Coordination Committee (NRCC)

| SN | Designation/Organization | Position |
| --- | --- | --- |
| 1 | Secretary, Ministry of Forests and Environment | Chairperson |
| 2 | Joint Secretary, Planning, Monitoring and Coordination Division | Member |
| 3 | Joint Secretary, Climate Change Management Division | Member |
| 4 | Director General, Department of Forests and Soil Conservation | Member |
| 5 | Director General, Department of National Parks and Wildlife Conservation | Member |
| 6 | Director General, Forest Research and Training Centre | Member |
| 7 | Secretary of seven Provincial Ministry responsible for forests, environment, and climate change | Member |
| 8 | Representative, Federation of Community Forest Users Group Nepal (FECOFUN) | Member |
| 9 | Representative, Association of Collaborative Forest Users, Nepal (ACOFUN) | Member |
| 10 | Representative, Nepal Federation of Indigenous Nationalities (NEFIN) | Member |
| 11 | Representative, Federation of Forest Based Industry and Trade, Nepal (FenFIT) | Member |
| 12 | Representative, Municipal Association of Nepal (MuAN) | Member |
| 13 | Representative, National Association of Rural Municipalities in Nepal | Member |
| 14 | Two representatives from Women and Dalit-related networks/associations nominated by the Ministry of Forest Environment | Member |
| 15 | Two independent experts (at least one woman) nominated by the Ministry of Forest Environment | Member |
| 16 | Gender Focal Person, Ministry of Forests and Environment | Member |
| 17 | Chief, REDD Implementation Centre | Member Secretary |

REDD Implementation Center (REDD IC)

The Ministry of Forests and Environment (MoFE) has established the REDD Implementation Centre (REDD IC) as a specialized institution tasked with the coordination and facilitation of REDD+ initiatives throughout the nation. Under the leadership of the Joint Secretary of the MoFE, the REDD IC serves as the primary government agency responsible for overseeing and directing all activities related to REDD+. Its comprehensive mandate includes leading the planning and execution of REDD+ related programs, ensuring effective collaboration among federal, provincial, and local agencies, and establishing a national safeguards information system. Additionally, the center is designated as the national authority for carbon management, tasked with the regular updating of greenhouse gas emissions data derived from forest activities and maintaining the Forest Reference Emission Level (FREL) in accordance with international obligations.

Beyond these foundational responsibilities, the REDD IC plays a crucial role in coordinating with the Forest Research and Training Centre (FRTC) to develop a credible Measurement, Reporting, and Verification (MRV) system, which is essential for the functionality of the National Forest Information System. The center is also responsible for creating a benefit-sharing mechanism for Emission Reduction (ER) programs and facilitating capacity-building efforts for forest officials and stakeholders at various administrative levels. Furthermore, the REDD IC conducts research related to REDD+ and, under the guidance of the MoFE, seeks financing opportunities to implement programs aimed at addressing the drivers of deforestation and degradation. Regular reporting to the MoFE, the National REDD Coordination Committee (NRCC), and the National REDD Steering Committee (NRSC) ensures that all activities are aligned with national objectives and international commitments.

Provincial REDD+ Coordination Committee (PRCC)

In accordance with the Constitution of Nepal, the management of forests is designated as a unique responsibility of the provincial government, which places the provincial forest agencies at the forefront of executing REDD+ initiatives. Each province has established a Provincial REDD+ Coordination Committee (PRCC), consisting of 21 members, which operates under the auspices of the Provincial Ministry responsible for forest, environment, and climate change. This committee plays a pivotal role in encouraging provinces to formulate their own provincial-level Action Plans for REDD+. The structure and composition of the PRCC are detailed in Table 8, and the committee is expected to convene at least twice a year, or more frequently, if necessary, to fulfill its obligations effectively.

The PRCC is tasked with several critical responsibilities that are essential for the successful implementation of REDD+ activities within the province. These include facilitating the execution and monitoring of the National REDD+ Strategy, providing guidance for the development of the REDD+ action plan, and overseeing the preparation and implementation of REDD+ initiatives by various entities such as forest offices and community-based forestry user groups. Additionally, the committee is responsible for advising relevant agencies on the technical and institutional support required for effective REDD+ implementation, as well as ensuring that accurate data and information related to REDD+ activities, including insights from the national forest monitoring system, are made available to stakeholders. Table 4 provides the composition of the PRCC.

Table 4. Composition of the Provincial REDD+ Coordination Committee (PRCC)

| SN | Designation/Organization | Position |
| --- | --- | --- |
| 1 | Secretary, Ministry responsible for Forests, Environment and Climate Change | Chairperson |
| 2 | Division Chief (Environment and Climate Change) | Member |
| 3 | Division Chief (Planning and Monitoring) | Member |
| 4 | Chief, Forest Research and Training Centre | Member |
| 5 | Under Secretary/Officer, Office of Chief Minister and Council of Ministers | Member |
| 6 | Under Secretary/Officer, Ministry responsible for Finance | Member |
| 7 | Under Secretary/Officer, Provincial Policy and Planning Commission | Member |
| 8 | Under Secretary/Officer, Ministry responsible for agriculture | Member |
| 9 | Under Secretary/Officer, Ministry responsible for energy | Member |
| 10 | Under Secretary/Officer, Ministry responsible for industry | Member |
| 11 | Representative, Provincial FECOFUN | Member |
| 12 | Representative, Provincial ACOFUN | Member |
| 13 | Representative, Provincial NEFIN | Member |
| 14 | Representative, Provincial FenFIT | Member |
| 15 | Representative, Nepal Federation of Indigenous Nationalities |  |
| 16 | Two representatives from Local Government | Member |
| 17 | Two representatives from Women and Dalit-related networks/associations nominated by the Ministry | Member |
| 18 | Two independent experts (at least one woman) nominated by the Ministry | Member |
| 19 | Director, Province Forest Directorate | Member Secretary |

REDD+ Institutional Arrangement at Local/Community Level

The REDD+ focal persons are established at the Division Forest Office and within protected area offices, where they facilitate the on-ground implementation of REDD+ strategies. At the community level, the responsibility for carrying out these activities falls to Community-Based Forest Management (CBFM) groups, including Community Forest User Groups and Collaborative Forest Management User Groups. These groups operate according to their specific forest operational and management plans, ensuring that the REDD+ activities align with local needs and ecological sustainability goals.

Institutional Arrangement for NFMS and MRV

The Forest Research and Training Centre (FRTC) operates as a key government agency under the Ministry of Forests and Environment, tasked with the essential role of conducting forestry research and survey activities across the nation. The primary objectives of the FRTC encompass a range of critical functions, including the execution of national-level forest inventories and the establishment of a comprehensive forest resource database. Additionally, the Centre is responsible for forest cover mapping and the preparation of detailed forest maps, alongside providing forestry training and extension services aimed at enhancing the knowledge and skills of stakeholders involved in forest management.

Within the FRTC, the Forest Survey and Carbon Monitoring Division plays a pivotal role by generating valuable statistical information and acquiring remote sensing data. This division is instrumental in producing essential products that facilitate forest inventory and carbon measurement, which are vital for planning forestry development at various administrative levels; national, provincial, and district. The federal FRTC oversees the operation of the National Forest Monitoring System (NFMS) and Measurement, Reporting, and Verification (MRV) process in close collaboration with the REDD IC and provincial forest authorities.

Figures 1 show the institutional arrangement for REDD+ implementation provided in the National REDD+ Strategy (2025-2034);

Figure 1.  Institutional Arrangement for REDD+ Implementation (in National REDD+ Strategy, 2025-2034)

    - Updates on the assumptions in the financial plan and any changes in circumstances that positively or negatively affect the financial plan and the implementation of the ER Program

The ER-PD (Annex 1) presents a detailed financial framework designed to facilitate the execution of the interventions proposed by the ER Program, which addresses the urgent challenges of deforestation and forest degradation while simultaneously promoting carbon sequestration. Initially, the estimated cost for implementing the key interventions identified during the ER-PD's development phase in 2018 was around US$ 184 million over a ten-year period. This financial plan spans the entire decade, but it is noteworthy that the anticipated total implementation costs for the first six years, concluding in 2024, were projected at US$ 123 million. The expectation was that the first results-based payment would be received by 2022, providing essential funding to support the activities of the ER Program until 2024. Furthermore, the government, through the Ministry of Forests and Environment (MoFE), is anticipated to contribute approximately US$ 21 million over the ten-year duration to bolster the implementation of the ER Program.

Significant financial support for the ER Program was expected from a variety of initiatives, including the President Chure Terai Madhesh Conservation Program, the World Bank-supported Forests for Prosperity Project under the Forest Investment Program of the Climate Investment Fund, and the long-term Terai Arc Landscape Program, which is funded by the World Wildlife Fund (WWF). These initiatives were projected to allocate considerable budgets to help achieve the objectives of the ER Program. Additionally, community and collaborative forests involved in the ER Program were anticipated to play a vital role, as Community-Based Forest Management (CBFM) groups are required to invest a minimum of 25% of their earnings into forest management activities. However, due to the absence of dedicated external funding for the implementation of the ER Program, most activities have relied on government budgets, the Forests for Prosperity Project, WWF's backing for the Terai Arc Landscape Strategy 2015-2025, and various initiatives led by the Zoological Society of London (ZSL), WWF etc., particularly in the National Parks and buffer zone community forests that are part of the ER Program area.

## Update on major drivers and lessons learned

The preliminary analysis during the preparation of Nepal’s R-PP in 2010 indicated that the drivers of deforestation and forest degradation are diverse, complex, and differ across various physiographic regions, which are always changing.  The analysis identified nine direct drivers and several indirect drivers, including socio-economic factors such as population increase, poverty, land scarcity and economic growth. After reviewing different studies and receiving input through consultations, the NRS (2018 -2022) outlined nine major drivers of deforestation and forest degradation with 10 underlying causes.

The ER-PD of the FCPF ER Program identified six major drivers and 10 underlying causes [for the ER-Program area] and assessed their potential impact on displacement risks and exploring strategies for mitigation. This evaluation classifies displacement risks into high, medium, or low categories. Now, major drivers of deforestation and forest degradation in the ER Program area and their potential impact on displacement have changed. New National REDD+ Strategy (2025-2034) has provided 8 direct drivers and various associated underlying causes. New drivers of deforestation and forestation as well as their potential displacement risk and mitigation measures are provided in Table 5.

Table 5. Updated drivers of deforestation and forest degradation for the ER Program area.

| SN | Direct Drivers | Magnitude | Risk of displacement |
| --- | --- | --- | --- |
| 1 | Unregulated and haphazard infrastructure construction, especially rural road construction | High (1) | Low: Infrastructure planning is typically designed to serve a given area and is therefore not at risk of displacement outside the ER Program Area. Where changes to designs do occur of some infrastructure; they are still within the same district boundary and within the ER Program accounting area. There is therefore minimal risk of displacement due to infrastructure development displacement. |
|  | Unplanned infrastructure development projects (e.g., Hydropower dams, transmission lines, and cement factors | low (3) |  |
| 2 | Frequent forest fires | High (1) | Low: The chances of displacement due to forest fire cannot occur, as forest fires in Nepal, unlike in many parts of the world, are generally started by people. Therefore, the displacement from forest fires is estimated to be very low. |
| 3 | Agriculture expansion (Encroachment for subsistence agriculture) | High (1) | Low: Migration within Nepal has been occurred from north to South, with potential encroachment in Terai forests for agriculture expansion. However, now encroachment for agriculture expansion is almost none because of out-migration is more common with Nepali men seeking labor opportunities in the Middle East in construction and infrastructure development. |
| 4 | Forest encroachment for human re-settlement (in the name of land-less family) | Medium (2) |  |
|  | Unplanned urbanization (rapid expansion) | High (1) | Low: Rapid expansion of unplanned urbanization does not have direct link to the emissions displacement. |
| 5 | The rapid spread of invasive alien species | High (1) | Low: No direct link to the displacement of the emissions. |
| 6 | Unsustainable and illegal logging and harvesting. | Medium (2) | Medium: The demand for timber in Nepal and the Terai far exceeds the current sustainable supply of timber. Timber trade in Nepal includes both inter-district and cross-border flows, with much of Nepal’s domestic demand being met through international sources or from domestic, illegal supply. Post-earthquake recovery is expected to sustain the high demand for construction timber through the lifetime of the ER Program. Curbing illegal timber extraction in the TAL could therefore cause additional pressure on forests outside the project area or promote illegal timber extraction within the project boundary. Counterbalancing this is the often-local nature of timber extraction in the Terai to serve the needs of local districts and communities. We have therefore ranked the risk of displacement of timber extraction as medium. |
|  | Unsustainable fuelwood extraction | Low (3) | Low: Now people are using less wood fuel for their energy needs. |
| 7 | Uncontrolled and unregulated livestock grazing | Medium (2) | Low: Grazing typically occurs within the vicinity of villages. Furthermore, now livestock population has significantly decreased and Cattle from the lowlands will not go as far as the highlands to graze thus displacement from grazing can be ranked as low. Nevertheless, some displacements could occur partially in border areas but will be very few |
| 8 | Unsustainable mining and Excavation (Stone, sand, and boulder) | Medium (2) | Low: Mining and excavation are mostly occurring in the reverbed-not the core forest areas, where actual ER Program interventions are implemented. |

## Methodological deviations

No changes to the methodology are suggested to address either material or immaterial errors or misstatements that could affect the assessment of the confirmed Reference Level and the validated Emission Reductions.

# System for measurement, monitoring and reporting emissions and removals occurring within the monitoring period

## Forest Monitoring System

Nepal has established a comprehensive and systematic forest monitoring system. Forest cover monitoring and periodic national forest inventory have been functioning well under the Forest Research and Training Centre (FRTC), Ministry of Forests and Environment. Permanent sample plots for national forest inventory have been established and measured at an interval of five years. Under the broader theme of National Land Cover Monitoring System (NLCMS), forest cover monitoring and mapping using satellite images are carried out on an annual basis. Furthermore, for precise estimation of volume and biomass, a program to develop allometric equations of 16 major tree species of the country has been initiated in 2022. The Forest Survey and Carbon Monitoring Division (FSCMD) of FRTC is the focal institution for forest monitoring systems. The organizational structure of the Division is presented in Figure 2-1. Though the ER-MR does not report emission from soil carbon pool, soil monitoring is part of National Forest Inventory (NFI) and thus, the soil section is part of the FSCMD.

Figure 2-1: Organizational Structure of Forest Survey and Carbon Monitoring Division in FRTC.

On the national scale, forest survey and monitoring require engagement of federal, provincial (state), and local (divisional) and community level actors and agencies. The relationship of these organizations is illustrated in Figure 2-2. Until REDD + Registry is fully established/operationalized, it has been agreed that the Government of Nepal deploys the World Bank’s Carbon Assets Tracking System (CATS).

Figure 2-2: Multi-Level Structure for Forest Inventory and Carbon Monitoring.

2.1.1 	The selection and management of GHG related data and information

The Forest Research and Training Centre (FRTC) and the REDD Implementation Centre are responsible for generating data for the Land Use, Land-Use Change, and Forestry (LULUCF) component of the national greenhouse gas (GHG) inventory. In the context of the Emission Reductions Monitoring Report (ERMR), only carbon dioxide (CO₂) is considered as the relevant GHG. To address this, FRTC has operationalized the National Land Cover Monitoring System (NLCMS), which provides annual updates on land cover changes across the country. Additionally, the five-year National Forest Inventory (NFI) feeds plot-level carbon density data into the system, which is used to calculate emission factors for different strata, including deforestation, forest degradation, and gain/loss areas.

The focus on CO₂ in the ERMR is aligned with the REDD+ framework, which prioritizes CO₂ emissions and removals from forests due to their significant role in climate change mitigation. While other GHGs, such as methane (CH₄) and nitrous oxide (N₂O), are not explicitly addressed in the ERMR, this approach ensures robust and consistent reporting on forest-related carbon fluxes, which are the primary contributors under the LULUCF sector.

The periodic Forest Resource Assessment (FRA) / National Forest Inventory (NFI) of Nepal produces the estimates on Emission Factors and generates information on tons of biomass (and carbon) stored per hectare of forest. Nepal has conducted three national forest inventories: NFI (1987-1998), FRA (2010-2014), and FRA (2016-2021). The FRA (2010-2014) produced the Emission Factors used for the FREL submitted to the UNFCCC in 2017 (Reference level mentioned in section 8 (page 120) of ERPD.

FRA (2010–2014) was designed to carry out national level forest resource assessment for providing comprehensive and up-to-date national-level forest resource information to support forest policy formulation, national-level forestry sector decision-making and international reporting. FRA Nepal implemented multi-source forest resources inventory by using high-resolution satellite imagery, field inventory as well as other existing data sources such as digital elevation model and national topographic maps. Categorization of land cover followed in FRA Nepal is based on current international practices of FAO which is also adopted by Intergovernmental Panel on Climate Change (IPCC) for greenhouse gases (GHG) emission estimation and reporting. The inventory design was largely based on the principle adopted for NFI (1999) developed by Kleinn (1994). Two-phase systematic cluster sampling was adopted for field measurement.

In order to have more accurate estimates for the Terai Arc Landscape (TAL) area, the plots located in this area were used to generate TAL-specific Emission Factors.

The Inventory design (ID) for national forest inventory has been created. The ID is based on two-phase sampling with stratification and, in case of TAL area with LiDAR working areas, the three-phase sampling with stratification has also been done.  Different sampling intensity and design has been applied for each stratum depending on a priori information and results from the visual interpretation of satellite imageries per stratum. However, the strata for calculation of results are five physiographic zones. Both High Resolution (HR) and Very High Resolution (VHR) satellite images are used for the first phase of the sampling to classify clusters and sample points of forest coverage and to assess the accessibility of sample plots. Based on this, sample plots for the second phase of the sampling (field inventory work) are defined. The second phase of the sampling refers to the actual clusters and permanent sample plots that were measured in the field.

2.1.2 	Processes for collecting, processing, consolidating and reporting GHG data and information

National Forest Inventory (Forest Resource Assessment) data has been collected as per the FRA Field Manual, 2022 approved by FRTC. The field manual contains detailed methodology of the Inventory Design and field measurement procedures.

The Forest Survey and Carbon Measurement section of FRTC validates the field data. In doing so, it deploys a validated method, stipulated in the Manual on Data Analysis and Results Generation (2021), which is available at the FRTC website.  The FRTC will operationalize the National Forest Monitoring Portal which will store and manage GHG related data and information.

2.1.3 	Systems and processes that ensure the accuracy of the data and information

The overall National Forest Inventory (NFI) Forest Resource Assessment (FRA) data collection procedure has always been accompanied by various levels of Quality Assurance and Quality Control (QA/QC) to ensure that the recorded data and followed procedures are reliable and meet minimum measurement standards. A refresher’s training on NFI before field data collection, hot checks by FRTC officers during the field measurement, re-measurement of 10 % forest plots to assure that collected data has error less than 5 %, QA/QC for lab measurements and data entry & archiving, etc. are some of the tools under overall system and process that ensure the accuracy of the data and information.

Accordingly, the forest inventory system is well established. Periodic forest cover monitoring system using remotely sensed data (satellite images) has been put in place. The FRTC will operationalize the National Forest Monitoring Portal which will incorporate design and maintenance of the Forest Monitoring System.

Systems and processes that support the Forest Monitoring System, including Standard Operating Procedures and QA/QC procedures. A well-established National Land Cover Monitoring System is functioning in Nepal under Google Earth Engine (GEE) platform.  Forest Resource Assessment manual is in operation for conducting Forest Resource Assessment (FRA) . There is a standard operating procedure for QA/QC process.

Role of communities in the forest monitoring system

Communities play a significant role in Nepal's forest monitoring system through their direct and indirect contributions to forest resource assessment and sustainable forest management. Prior to field measurements, local forest communities are informed about inventory activities and encouraged to participate in the assessment of sample plots. Their involvement is particularly important in facilitating access to plot locations, assisting in species identification, and providing local knowledge on the drivers, causes, characteristics, and impacts of deforestation and forest degradation. Community members play an important role in the early detection and reporting of changes and emerging threats within forest areas.

Community-Based Forest Management (CBFM) groups also contribute to monitoring efforts by preparing and submitting annual monitoring reports to the respective Division Forest Offices (DFOs). Although community-generated information is not always explicitly incorporated into national monitoring databases, community participation remains essential for the effective implementation of forest management plans, forest protection, and benefit-sharing mechanisms. Forest management groups offer support during the inventory of the Permanent sample plot (National Forest Inventory) and participate in sharing the benefits accrued from non-carbon benefits.

2.1.4 Use of and consistency with standard technical procedures in the country and the National Forest Monitoring System.

Nepal has developed a system for continuous monitoring of land cover using consistent methodology, following “National Land Cover Monitoring System (NLCMS) of Nepal” (2022), which is available at FRTC website. Forest cover from NLCMS is the one of the input variables for generation of Activity Data. The Activity Data were prepared using ensemble methods. This method includes four algorithms (please refer to Nepal Forest Change Area Estimation Tool), that include CODED (Continuous Degradation Detection), CCDC-SMA (Continuous Change Detection and Classification- Spectral Mixture Analysis), LandTrendr, and MTDD (Multivariate Time-series Disturbance Detection).

## Updates to the monitoring approach

The monitoring approach remains unchanged from that described in the validated version of Annex 4 of the first Monitoring Report, aside from adding additional samples via a sampling design that does not loose information about previous sample distribution. Because the initial stratification map used to estimate activity data for the reference and first monitoring periods (2004-2021) did not account for land use changes such as deforestation, reforestation, and degradation occurring between 2022 and 2024, a partially independent sample was required to estimate activity data for the second monitoring period. However, independent surveys and independent sample units used in successive surveys carry the risk of “double detection.” To address this, in areas previously identified in the first stratification map as deforested or degraded were not used directly for additional sampling. New plots were added only in the new deforestation, degradation, and regeneration areas of the second stratification map. This was done using a sub-strata approach from the original 5 strata. So in the strata where degradation or deforestation was logically possible, given the previous state of the forest (e.g. degradation possible in formerly stable forest but not in formerly deforested areas), and was detected, then a sub-strata was used to distribute samples within these new areas of change.

The final stratification map for 2022-2024, covering the second monitoring period, is an updated version of the reference period map (2004–2021). This update was created using the same four mapping algorithms applied to the original stratification map to detect areas of forest loss, degradation, and regrowth.

Sub-strata for detected changes from 2022 to 2024 were added to the strata in the reference period map, where these changes were logically possible. The monitoring period sub-strata for deforestation/degradation were created only using the Continuous Change Detection and Classification – Spectral Mixture Analysis (CCDC-SMA) algorithm. The sub-strata for forest gain were created by comparing pre-monitoring-period local land-cover maps with a 2024 tree-canopy-cover threshold map. The version of CCDC-SMA used in the reference period was adapted so that, instead of using a local land cover map from the ending year (2024) to distinguish degradation from deforestation, the adaptation used a >20% thresholder Tree Canopy Cover map. This was because no local 2024 land-cover map was yet available at the time of analysis.

The sub-strata for areas of forest gain in the monitoring period were created in a custom-made script in Google Earth Engine. The approach used the local land cover maps created as part of the National Land Cover Monitoring System (NLCMS) for Nepal. These maps have been thoroughly checked for accuracy and are the officially used annual land cover maps for Nepal. No NLCMS map was yet available for the final year of analysis, 2024, so we used a local Tree Canopy Cover (TCC) map produced by ICIMOD. Anywhere where the TCC was greater than 20% in 2024 was considered to be forest. The >20% threshold was selected by local forestry experts using visual comparison of the TCC map with areas of forest, because although >10% TCC is the definition of forest in Nepal, this value in the TCC map did not align with known areas of forest and a more aggressive threshold was required. Using these two input datasets we obtained a 2022-2024 forest gain map by looking for areas where NLCMS said there was no forest for at least the five years prior to the monitoring period and the 2024 >20% TCC map indicated forest was present in 2024.

## Measurement, monitoring and reporting approach

Table 6 provides a systematic and step-by-step description of the measurement and monitoring approach applied for the establishment of the Reference Level and estimating Emissions and Emissions Reductions during the Monitoring and Reporting Period for estimating the emissions and removals from the Sources/Sinks, Carbon Pools, and greenhouse gases selected in the ER-PD.

Table 2-1: Step-by-step description of the monitoring parameter and data integration tools to establish the Reference Level and estimate Emissions and Emissions Reductions during the Monitoring Period for the Carbon Pools and greenhouse gases (GHGs) selected in  Annex 4 of the First Monitoring Report.

| Step | Monitoring parameters and Data Integration tools | Tools and datasets | Description of the measurement and monitoring approach |  |
| --- | --- | --- | --- | --- |
| 1 | Activity Data estimate and associated uncertainty. | GEE repository of all scripts CCDC-SMA: 1_CCDC_SMA_UI_C2 2_ViewExportDegDefMapp 2_ViewExportDegDefMap_noNLCMSavailable (modified CCDC-SMA that works for monitoring period with no end-year landcover map) CODED CODED_Application_v2_localcopy LandTrendr24 1_UI-ImageScreener (optional) 2_LT-Data-Visualization-NepalTool 3_LTMakeLossGainPostprocessed 4_AssembleMap MTDD 1MTDD_app_trainingpoints 2MTDD_app_changemap Map Agreement App 1_MakeAgreementMap_Nepal_nocombo Nepal_Physio_SubStrata_FCPF_monitoring_densification (for monitoring period creation of stratification map with sub-strata) | Nepal Forest change area estimation tool: Documentation on how to use this tool and a compiled set of links to user interfaces of all the tools needed to complete the forest change area estimation for Nepal can be accessed at the following link: . However, these instructions only go up through the reference period methodology. The updated repository of scripts in GEE is here: https://code.earthengine.google.com/?accept_repo=users/cwespestad_SIG/Nepal_WorkshopScripts_2024 1. Forest change mapping: To estimate the area, Nepal employs a sample-based approach. For the sample design, a forest change map spanning from 2004 to 2021 was prepared, then updated for 2022-2024 changes. The following four mapping algorithms that utilize remote sensing imagery, training data points, land cover maps, and time series data analysis were used to map areas experiencing forest loss, degradation, and/or regrowth. - CCDC-SMA: Continuous Change Detection and Classification - Spectral Mixture Analysis (CCDC-SMA) monitors abrupt and gradual forest degradation. - CODED: Continuous Degradation Detection (CODED) algorithm detects forest canopy disturbances and classifies them as degradation or deforestation based on land cover. CODED uses linear spectral unmixing to generate subpixel fractions of spectral endmembers, which are used to calculate a time series of the Normalized Degradation Fraction Index (NDFI). - LandTrendr: The LandTrendr algorithms use simple statistical techniques to simplify a time-series of spectral values into a sequence of connected straight-line segments that capture the overall shape of that pixel’s trajectory while omitting year-to-year noise. The resultant segments can then be examined to select periods where the trajectory displays behaviors of interest such as disturbance or growth. - MTDD: Multi-variate Time-series Disturbance Detection (MTDD) classifies initially forested areas into stable forest, degraded, and deforested by training a random forest classifier with 66 metrics. These metrics are derived from six annual time-series (i.e., NDVI, two SWIR spectral regions, two NDWI indices, and SAVI) which are used to calculate eleven descriptive statistics (i.e., minimum, maximum, range, mean, standard deviation, coefficient of variation, kurtosis, skewness, slope, maximum 5-year slope, and most recent value). Overall MTDD’s process includes five main steps: (1) making annual time series, (2) calculating 11 descriptive statistics for the time series, (3) generating training/validation points, (4) training a random forest classifier, and (5) validating the classification. Each of the mapping algorithms is useful for detecting changes in a slightly different manner. However, all maps are susceptible to bias, which is why the area of map classes from the resulting maps should not be directly used for Activity Data reporting. Each map was visually assessed so any concerning results can be addressed with parameter adjustment as needed. An agreement map generated from the results of all four methods is used to inform sampling of the reference period design. The goal is to ensure that no strata is under sampled. The 1_MakeAgreementMap_Nepal tool (in Map Agreement App folder of GEE repository) is used to combine the maps of the four forest change detection algorithms. Final strata for the reference period for the agreement map and their human-readable labels are 1: "Degradation"; 2: "Deforestation"; 3: "Reforestation"; 4: "Stable Non-Forest"; 5: "Stable Forest". When combining the results of the four algorithms into one map for the reference period, the following logic rules are applied for each pixel: - A GAIN supersedes all other labels. - If an equal number of DEG and LOSS labels occur across the four algorithms, LOSS supersedes. - If the number of DEG labels is more than the number of LOSS labels or DEG is the only type of change detected, a DEG label is given. - If the number of LOSS labels is more than the number of DEG labels or LOSS is the only type of change detected, a LOSS label is given. - A Non-forest label is given only if all four algorithms label it as Non-forest. - A Forest label is given only if all four algorithms label it as Forest. This final stratification map for the monitoring period is a revised version of the reference period (2004-2021) stratification map, which used four mapping algorithms to map areas experiencing forest loss, degradation, and/or regrowth. Sub-strata for detected changes within 2022-2024 were added to the strata in the reference period map where these changes were logically possible. The monitoring period sub-strata for deforestation/degradation were created only using the Continuous Change Detection and Classification – Spectral Mixture Analysis (CCDC-SMA) algorithm. The sub-strata for forest gain were created by comparing pre-monitoring period local land cover maps with a thresholded tree canopy cover map for 2024. The version of CCDC-SMA used in the reference period was adapted so that instead of using a local land cover map from the ending year (2024) to distinguish degradation from deforestation, the adaptation used a >20% thresholded Tree Canopy Cover map. This was because no local 2024 land cover map was yet available at the time of analysis. The sub-strata for areas of forest gain in the monitoring period was created in a custom-made script in Google Earth Engine. The approach used the local land cover maps created as part of the National Land Cover Monitoring System (NLCMS) for Nepal. These maps have been thoroughly checked for accuracy and are the officially used annual land cover maps for Nepal. No NLCMS map was yet available for the final year of analysis, 2024, so we used a local Tree Canopy Cover (TCC) map produced by ICIMOD. Anywhere where the TCC was greater than 20% in 2024 was considered to be forest. The >20% threshold was selected by local forestry experts using visual comparison of the TCC map with areas of forest, because although >10% TCC is the definition of forest in Nepal, this value in the TCC map did not align with known areas of forest and a more aggressive threshold was required. Using these two input datasets we obtained a 2022-2024 forest gain map by looking for areas where NLCMS said there was no forest for at least the five years prior to the monitoring period and the 2024 >20% TCC map indicated forest was present in 2024. Script for monitoring period creation of the stratification map with sub-strata for densification of points in areas of monitoring period change: Nepal_Physio_SubStrata_FCPF_monitoring_densification (https://code.earthengine.google.com/09e619e58415b82ec939827090a24a61) |  |
|  |  | Agreement map preparation 1_MakeAgreementMap_Nepal26 (Agreement Map in Google Drive folder) Nepal_Physio_SubStrata_FCPF_monitoring_densification (for monitoring period creation of stratification map with sub-strata) Forest Change Agreement Map (reference period) Area available in each stratum Spreadsheet for Sample Size and Distribution Design | 2. Sample design: A sample-based approach is used to complete area estimation. This approach is preferred over pixel-counting methods because all maps have errors. Sample based approaches create unbiased estimates of area and allows calculation of the uncertainty of each estimate. The final stratification map is defined into 9 strata, as defined below. Final strata definitions: - Degradation (1) = in the reference period more algorithms detected degradation than loss, and GAIN is not detected - Deforestation (2) = in the reference period more algorithms detected LOSS than DEG or an equal number of algorithms detected LOSS and DEG, and GAIN is not detected - Reforestation (3) = in the reference period one or two algorithms labeled the pixel as GAIN, even if others detected LOSS or DEG - Stable Non-Forest (4) = in the reference period all algorithms labeled pixel as stable Non-forest - Stable Forest (5) = in the reference period all algorithms labeled pixel as stable Forest - SF to Monitoring Degradation (6): Reference period stable forest with Monitoring Period Degradation - DEG to Monitoring Deforestation (7): Reference period degraded forest with Monitoring Period Deforestation - SF to Monitoring Deforestation (8): Reference period stable forest with Monitoring Period Deforestation - NF to Monitoring Gain (9): Reference period stable non-forest with Monitoring Period Forest Gain) 4. Because the initial stratification map used to estimate activity data for the reference and first monitoring periods (2004-2021) did not account for land use changes such as deforestation, reforestation, and degradation occurring between 2022 and 2024, a partially independent sample was required to estimate activity data for the second monitoring period. However, independent surveys and independent sample units used in successive surveys carry the risk of “double detection.” To address this, in areas previously identified in the first stratification map as deforested or degraded, were not directly used for additional sampling. . New plots were added only in the new deforestation, degradation, and regeneration areas of the second stratification map [1]. Stratas 5-9 are sub-stratas from the original 4 (e.g. “SF to Monitoring Degradation (6)” is a substrata of the reference period Stable Forest strata (5) where new Degradation events from the monitoring period have been detected). It is important not to merge former stratas when doing sampling, so the weights of the reference period sampling design can be easily managed. This is why Monitoring period Deforestation is kept as separate stratas (7 and 8), depending on whether the Deforestation was detected in formerly degraded or stable forest. The distribution of the sample of plots for the second monitoring period was also based on the relative area available in each stratum in the revised stratification map, the human resources available for interpretations, and a target standard error. The linked spreadsheet in tools columns contains equations needed to calculate the ideal sample size to hopefully achieve the target standard error. A total of 1,961 points were selected via stratified random sampling to be used for sample-based area estimation. For the smaller strata, a minimum of 110 points was required (the same minimum sample size per strata used for the reference period, giving more than ample points to meet the Central Limit Theorem minimum requirement for statistical validity). It is important to clarify that the 1,961 points used to calculate activity data for the second reporting period include 1,521 of the 1,522 points used for the reference and first reporting periods. It was necessary to include the original sampling plots to monitor degradation and deforestation in the permanent and secondary forest by tracking land use at each plot. To better capture land-use change (deforestation and reforestation) and degradation that occurred during 2022–2024, an additional 440 points were incorporated into the original sampling effort via the new sub-strata (5-9). [1] See 2.3 Temporal tracking of land use in the context of temporary versus permanent sample units by Espejo, A. and Hamilton, R. In Good practices in sample-based area estimation https://www.researchgate.net/profile/Randy-Hamilton/publication/378655524_Good_practices_in_sample-based_area_estimation/links/65e28136e7670d36abeb48c0/Good-practices-in-sample-based-area-estimation.pdf |  |
|  |  | Nepal’s CEO institution Interpretation key | 3. Reference data collection (completed in CEO): To estimate emissions from deforestation, carbon enhancement removals, and forest degradation emissions, reference data were collected through visual imagery interpretation and time series analysis of 1,961 sampling plots in CEO. The sampling points were visually interpreted for the same period that the forest change map was created (2004 to 2024). However, to identify the age of forests in order to differentiate between secondary and permanent forests, an additional pre-period was examined. The time period of examination was divided into four subperiods with distinct sets of survey questions: 1984-2003, 2004-2014, 2015-2017, 2018-2021 and 2022-2024 (see Figure 2-3). The canopy cover was visually evaluated in permanent forest only for the years 2003/2004, 2014/2015, 2017/2018, 2021, and 2024. |  |
|  |  |  | i. | Generating a CEO project from a template: FRTC created a template to collect land-use change and degradation reference data in Collect Earth Online (CEO31) for the following periods: Pre-reference period (t0) - 1983-2003, Reference Period (t1) – 2004-2014, First monitoring period (t2) – 2015-2017 and Second monitoring period (t3) – 2018-2021 and Third monitoring period (t4) – 2022-2024 |
|  |  |  | ii. | Sampling unit: The Sampling Unit (SU) is a 70 x 70 meter plot. Inside the SU, a 3x3 points sub-grid (9 points) was created to estimate forest canopy cover percentage within each sampling unit. Negative changes in this value were used to indicate whether a degradation event had occurred |
|  |  |  | iii. | Number of Sampling Units: A total of 1,961 sampling points, selected via stratified random sampling, were visually assessed. |
|  |  |  | iv. | Interpretation key: Nepal produced an interpretation key that should be reused and updated as needed. The key was used to increase consistency across the team of interpreters. The interpretation key includes the following land use categories: - Forest - Natural Secondary Forest - Forest Plantations - Shaded cropland - Grassland - Settlements - Unshaded Cropland (TCC 10% or less) - Other lands |
|  |  |  | v. |  |
|  |  | Reference data compilation R-script CompiledData_CEO | 4. | Reference data compilation: The data collected in the CEO was compiled in R for each period ( t0, t1, t2, t3 and t4) to obtain necessary information that was then used to estimate the Activity Data |
|  |  |  | i. | Deforestation Activity Data - tx_disturbance_type_subcat: type of forest loss and gain, stable forest, and stable non-forest. - Non.forest.land.use.type.in.[year of interest] Non forest land use type in the period. - Number.of.tree.covered.samples.[year of interest]: Number from the 9-point grid of sample points within the plot that are covered by tree canopies. - pl_monitoring_adjusted_label: Agreement map strata. - tx_type_final: Land use / land cover type - tx_yr_secondaryforest_establ: year of secondary forest establishment |
|  |  |  | ii. | Forest gain Activity Data - tx_disturbance_type_subcat: type of forest loss and gain, stable forest, and stable non-forest. - pl_monitoring_adjusted_label: Agreement map strata. |
|  |  |  | iii. | Degradation Activity Data - tx_type_final: Land use / land cover type - tx_numbretrees: canopy cover - pl_monitoring_adjusted_label: Agreement map strata |
|  |  | (Please read this file “READ” before accessing it) | 7. Area and uncertainty estimation: Nepal adopts a sample-based methodology to estimate the Activity Data for deforestation, forest gain, and degradation. All 1,961 samples were used to calculate area estimates and their associated uncertainties. The estimation of Activity Data was done using the stratified random estimator based on the formulas described by Cochran (1977). Estimates are made for each of the land use categories considered (10 classes) and in terms of changes from one period to another, representing a total of 27 effective combinations (Deforestation 15 , Forest Gain 3, and Degradation 9). Number Transition Initial Final 1 Primary Forest Lost Intact Forest Grasslands 2 Primary Forest Lost Intact Forest Other Land 3 Primary Forest Lost Intact Forest Settlements 4 Primary Forest Lost Intact Forest Unshaded Cropland (TCC 10% or less) 5 Primary Forest Lost Degraded Forest Grasslands 6 Primary Forest Lost Degraded Forest Other Land 7 Primary Forest Lost Degraded Forest Settlements 8 Primary Forest Lost Degraded Forest Unshaded Cropland (TCC 10% or less) 9 Primary Forest Lost Very Degraded Forest Grasslands 10 Primary Forest Lost Very Degraded Forest Other Land 11 Primary Forest Lost Very Degraded Forest Settlements 12 Primary Forest Lost Very Degraded Forest Unshaded Cropland (TCC 10% or less) 13 Secondary Forest Lost Secondary forest Other Land 14 Secondary Forest Lost Secondary forest Unshaded Cropland (TCC 10% or less) 15 Secondary Forest Lost Secondary forest Grasslands 16 Stable Forest Inctact forest Inctact forest 17 Stable Forest Degraded forest Degraded forest 18 Stable Forest Very degraded forest Very degraded forest 19 Degradation Inctact forest Degraded forest 20 Degradation Inctact forest Very degraded forest 21 Degradation Degraded forest Very degraded forest 22 Enhancement Degraded forest Inctact forest 23 Enhancement Very degraded forest Inctact forest 24 Enhancement Very degraded forest Degraded forest 25 Forest Gain Non-Forest Natural secondary forest gain 26 Forest Gain Non-Forest plantation forest gain 27 Forest Gain Non-Forest shaded cropland gain With Cochran (1977) formulas it is not possibles to calculate a standard deviation (SD) for the activity data estimate. Instead, the sampling error calculation is based on the standard error (SE), which quantifies the precision of the estimates based on the sample data. In Monte Carlo simulations de SD is used to represent the variability of input parameters. Initially, because no SD was available for the activity data, the country used the 90th percentile confidence interval and the normal probability density function (PDF). Later on, the Monte Carlo simulation was updated to utilize the standard error (SE) in the normal PDF functions when no standard deviation (SD) was available. Estimates and associated uncertainties are produced in the Activity Data Tool (Nepal_TAL_AD_tool_2sdERMR.xlsx) for each combination considering the stratification applied. The Activity Data tool comprises various spreadsheets that estimate different types of Activity Data. These include the Dataset that is used to estimate sample-based Activity Data (Reference_Data sheet), as well as spreadsheets for estimating Activity Data for deforestation (Deforestation sheet), forest restoration (Forest_gain sheet), and area of change in canopy cover (loss and gain) in permanent forest lands (Degradation sheet). To ensure accurate Activity Data estimates, material errors are controlled through specific mechanisms in the estimation spreadsheets. This includes matching forest type sampling points with sample size to prevent double counting in the sample-based Activity Data estimate. The accuracy of deforested, forest gain, and degraded areas are checked in cells Deforestation: E26-G38/E56-G68, Forest_gain: D17-F29, and Degradation: D23-F35 respectively. Before reporting AD values, a quality assurance/quality control procedure is conducted to verify that all these cells are labeled “Ok”. |  |
| 2 | Forest regrowth removal rates and forest carbon densities calculation, including the uncertainty estimate. | NFI dataset | 1. National Forest Inventory: The biomass estimates used for the ER monitoring report are Tier 2 (country specific data) and has been derived from the National Forest Inventory-Forest Resource Assessment (NFI-FRA). The NFI-FRA involved remeasurement of the permanent sample plots established by the FRA Nepal Project (2010-2014) in addition to an additional number of plots established and measured using the same methodology. The Inventory Design adopted was based largely on methods developed by Kleinn (1994) and finalized by the DFRS/FRA 2010-2014. Nepal is conducting NFI by re-measuring the permanent sample plots at an interval of every five years. One of the important characteristics of NFI in Nepal is hidden permanent sample plots leaving “no marks” above ground. Instead, the plots are georeferenced and plot centers consist of metal pegs inserted a few inches below the ground level. The reason behind hidden plots in NFI is to maintain consistency in anthropogenic activities and forest products used by local people both inside and outside the plots. These characteristics of NFI plots of Nepal might even aid to control leakage of GHG emission. The detailed methodology adopted for sample selection is presented in DFRS, 2014. NFI data from 622 permanent sample plots located within the Emission Reduction Program area were derived. i. Inventory / Sample plot design and data collection: The Concentric Circular Sample Plot (CCSP) design was adopted as used by the FRA Nepal Project (2010-2014). Each sample plot had four concentric circles of different radii (see Figure below), which were used to measure trees with different DBH as follows: ▪ trees having 30 cm DBH or more enumerated within a 20 m radius plot (area: 1256.6 m2) ▪ trees having 20-29.9 cm DBH enumerated within a 15 m radius plot (area:706.9 m2) ▪ trees having 10-19.9 cm DBH enumerated within an 8 m radius plot (area:201.0 m2) ▪ trees having 5-9.9 cm DBH enumerated within a 4 m radius plot (area: 50.3 m2) Layout of the concentric circular plot with other sub-plots Other subplots were established to assess forest attributes other than trees, such as dead woods and disturbances, seedlings, saplings, shrubs, and herbs, etc. ii. Volume and Biomass estimation: Tree stem volumes and biomass were estimated using standard methodology with national allometric equations adopted since NFI / FRA 2010-2014. Details provided in the manual. iii. Quality assurance of forest inventory data: Use of periodically revised field manual, training to field crews and regular monitoring and feedback were some of the measures applied to maintain the quality of the inventory results. For the statistical analysis to check for the quality of the results, over 10% of the total PSPs measured were systematically selected (with a random start) and re-measured. Details can be referred to: https://frtc.gov.np/uploads/files/1_%20QAQC_manual.pdf Furthermore, standard protocols and manuals on modeling of required parameters e.g. diameter-height modeling & taper function curve, calculation of volume and biomass using the allometric models, and error estimation were developed under supervision of the experts from Finnish Forest Research Institute (METLA, now LUKE Finland) during the FRA 2010-2014. |  |
|  |  | Nepal’s CEO institution | 2. Land use change analysis of the NFI permanent plots stratification for carbon densities, and removal rate estimate. To ensure consistency, the Emission Factors (EF) have been aligned with the estimates of land-use transitions area (AD). To achieve this, the same time series analysis and data collection methods that were used in CEO were replicated for the NFI permanent plot’s locations. The NFI plots have been classified as Non-forest land use (grassland, other land, unshaded cropland), Permanent Forest, or Secondary Forests. Additionally, the canopy cover of Permanent Forest plots was evaluated to determine whether they were intact (7-9 points), degraded (4-6 points), or very degraded forest (1-3 points). |  |
|  |  | CarbonDensitiesTools.xlsx (Please read this file “READ” before accessing it) | 3. Carbon densities and removal rates calculation: Nepal developed a calculation tool (CarbonDensitiesTools.xlsx), to estimate carbon densities for both forest and non-forest areas based on the NFI plots dataset. This tool also facilitates the determination of forest regrowth removal rates. Confidence intervals and errors are computed based on the number of sampling plots and standard deviation within each respective land use type or removal rate: - Natural Forest carbon densities calculation: The carbon densities of natural forests categorized as intact, degraded, and very degraded were estimated using the first measurement from NFI's 388 plots (pl_total_bio_mrv). - Non-Forest carbon densities calculation: The determination of average carbon densities for non-forest lands was based on 21 NFI plots, which provided biomass estimates for grassland, other land, and unshaded cropland. These estimates were obtained during the initial measurement phase of the NFI (pl_total_bio_mspa). - Forest regrowth removal rates estimate: The forest regrowth removal rate calculation is based on a sample of sixteen NFI plots established in secondary forests. Two biomass measurements were taken in these plots, and the difference in biomass over the years between measurements (pl_yr) was used to estimate the average removal rate. |  |
| 3 | Emission and removals calculation | (Please read this file “READ” before accessing it) | To calculate the Emission Reductions of the Nepal Emission Reduction Program, an Excel tool named Nepal_TAL_Integration_tool_V6_2sdERMR.xlsx is used. This tool generates estimates for emissions and removals, along with their associated uncertainties, for both the reference and reporting periods. The estimates are generated for Deforestation, Carbon Enhancement, and Degradation - the three REDD+ activities involved in the carbon accounting of the program. - Calculation of emissions and removals: The Parameters and Model sheet generate estimates for Emissions and Removals. These estimates are calculated using Activity Data and Carbon Density tools. - Emission Reductions calculation: Results sheet generates estimates of Emission Reductions for the Reporting Period (June 22, 2018 – December 31, 2021). These estimates are calculated using the Parameters and Model sheet calculations. - Emission Reductions available for transfer to the Carbon Fund: The Section 8 sheet of Nepal_TAL_fcpf_er-mr_template_tool_2ndERMR_revised.xlsm computes the available ER for transfer in accordance with Section 8 of the ER monitoring report. |  |
| 4 | Emission reduction uncertainty estimates and sensitivity analysis. | NEPAL_TAL_Integration_tool_MC_2ERMR.xlsx NEPAL_TAL_Integration_tool_SensitivityAnaly sis_2ERMR.xlsx (Please read this file “READ” before accessing it) | Nepal has developed two distinct Excel tools utilizing the ER calculation tool (NEPAL_TAL_Integration_tool.xlsx). The first one, NEPAL_TAL_Integration_tool_MC.xlsx, is designed to carry out Monte Carlo simulations and estimate the uncertainty of the ER calculation. The second tool, NEPAL_TAL_Integration_tool_SensitivityAnalysis.xlsx, is utilized for sensitivity analysis purposes. |  |

Figure 2-3: A. Reference Period and monitoring periods considered in collecting reference data for AD estimate. B. Activity Data that is required for the estimate of emissions from deforestation, carbon enhancement removals, and emissions from degradation. C. Forest cover type definition (permanent and secondary) based on time-series analysis.

A

B

C

Figures A and B

Figure C

>>

### Line Diagram

Figure 2-4 presents the emissions reductions calculation workflow during the monitoring period. Activities listed under steps 1 to 4 are all performed by the Forest Research and Training Centre (FRTC).

Figure 2-4: Line Diagram- Emissions reduction calculation workflow

### Calculation

Emission reduction calculation (𝐸𝑅𝐸𝑅𝑃,):

To determine GHG emission reductions, the IPCC methods and equations were used over the monitoring period.

𝐸𝑅𝐸𝑅𝑃, = 𝑅𝐿𝑡 − 𝐺𝐻𝐺𝑡   	Equation 1

| Where: |  |  |
| --- | --- | --- |
| 𝐸𝑅𝐸𝑅𝑃 | = | Emission Reductions under the ER Program in the Reporting Period; tCO2e*year-1. |
| 𝑅𝐿𝑅𝑃 | = | Net emissions of the Reference Level over the Reference Period; tCO2e/yr. This is sourced from Annex 4 to the ER Monitoring Report and equations are provided below. |
| 𝐺𝐻𝐺𝑡 | = | Monitored gross emissions from deforestation during the Reporting Period; tCO2e; |
| 𝑇 | = | Number of years during the reporting period; dimensionless. |

Reference Level (𝑅𝐿𝑅𝑃)

Net emissions of over Reference Period (𝑅𝐿𝑅𝑃) are estimated as the sum of annual change in total biomass carbon stocks (deforestation and degradation), and annual removals (∆𝐶𝐵𝑡).

| RLRP=tRP ∆CLURP,i,tRP | Equation 2 |
| --- | --- |

Where:

| ∆CLURP,i,t | = | Balance of emissions during the Reference Period in the Accounting Area of the ER Program that corresponds to the sum of annual change in carbon stocks and removals for each REDD+ activity i at year t; tCO2*year-1. |
| --- | --- | --- |
| RP | = | Reference period; years. |

Annual change in total biomass carbon stocks forest land converted to another land-use category (∆CBdefo,t)

Emissions from deforestation were estimated based on the Deforestation Sheet of Activity Data tool following the 2006 IPCC Guidelines, the annual change in total biomass carbon stocks forest land converted to other land-use category (∆CBdefo,t) would be estimated through the following equation:

| ∆CBdefo,t=∆CG+∆CCONVERSION-∆CL | Equation 3 |
| --- | --- |

Where:

| ∆CBdefo,t | Annual change in carbon stocks in biomass on land converted to other land-use category, in tones C yr-1; |
| --- | --- |
| ∆CG | Annual increase in carbon stocks in biomass due to growth on land converted to another land-use category, in tones C yr-1; |
| ∆CCONVERSION | Initial change in carbon stocks in biomass on land converted to other land-use category, in tones C yr-1; and |
| ∆CL | Annual decrease in biomass carbon stocks due to losses from harvesting, fuel wood gathering and disturbances on land converted to other land-use categories, in tones C yr-1. |

Following the recommendations set in chapter 2.2.1 of the GFOI Methods Guidance Document for applying IPCC Guidelines and guidance in the context of REDD+, the above equation will be simplified and it will be assumed that: a) the annual change in carbon stocks in biomass (∆CB) is equal to the initial change in carbon stocks (∆CCONVERSION); b) it is assumed that the biomass stocks immediately after conversion are the biomass stocks of the resulting land-use. Therefore, the annual change in carbon stocks would be estimated as follows:

| ∆CBt=∆CCONVERSION ∆CBt=j,i BBefore,j- BAfter,i x CF x4412 × Aj,iRP |  |  |
| --- | --- | --- |
|  | Equation 4 |  |

Where:

| Aj,iRP | Area converted/transited from forest type j to non-forest type i during the Reference Period, in hectares per year. In this case, sixteen forest land conversions are possible: 1 Intact Forest to Grasslands 2 Intact Forest to Other Land 3 Intact Forest to Settlements 4 Intact Forest to Unshaded Cropland (TCC 10% or less) 5 Degraded Forest to Grasslands 6 Degraded Forest to Other Land 7 Degraded Forest to Settlements 8 Degraded Forest to Unshaded Cropland (TCC 10% or less) 9 Very Degraded Forest to Grasslands 10 Very Degraded Forest to Other Land 11 Very Degraded Forest to Settlements 12 Very Degraded Forest to Unshaded Cropland (TCC 10% or less) 13 Secondary natural forest to Grasslands 14 Secondary natural forest to Other Land 15 Secondary natural forest to Settlements 16 Secondary natural forest to Unshaded Cropland (TCC 10% or less) |
| --- | --- |
| BBefore,j | Total biomass of forest type j before conversion/transition, in tons of dry matter per ha. This is equal to the sum of aboveground (AGBBefore,j) and belowground biomass (BGBBefore,j) and it is defined for each forest type. |
| BAfter, i | Total biomass of non-forest type i after conversion, in tons dry matter per ha. This is equal to the sum of aboveground (AGBAfter,i) and belowground biomass (BGBAfter,i) and it is defined for each of the non-forest Land Use categories. |
| CF | Carbon fraction of dry matter in tC per ton dry matter. The value used is: - 0.47 is the default for (sub)tropical forest as per IPCC AFOLU guidelines 2006, Table 4.3. |
| 44/12 | Conversion of C to CO2 |
| R: S | Root-to-shoot ratio (0.44). |
| Aj,iRP | Area converted/transited from forest type j to non-forest type i during the Reference Period, in hectares per year. In this case, sixteen forest land conversions are possible: 1 Intact Forest to Grasslands 2 Intact Forest to Other Land 3 Intact Forest to Settlements 4 Intact Forest to Unshaded Cropland (TCC 10% or less) 5 Degraded Forest to Grasslands 6 Degraded Forest to Other Land 7 Degraded Forest to Settlements 8 Degraded Forest to Unshaded Cropland (TCC 10% or less) 9 Very Degraded Forest to Grasslands 10 Very Degraded Forest to Other Land 11 Very Degraded Forest to Settlements 12 Very Degraded Forest to Unshaded Cropland (TCC 10% or less) 13 Secondary natural forest to Grasslands 14 Secondary natural forest to Other Land 15 Secondary natural forest to Settlements 16 Secondary natural forest to Unshaded Cropland (TCC 10% or less) |

Annual change in carbon stocks in biomass on forestland remaining forestland (∆CBdeg,t)

Following the 2006 IPCC Guidelines, the annual change in carbon stocks in biomass on forestland remaining forestland (∆CBDEG) could be estimated through the Gain-Loss Method or the Stock-Difference Method as described in Chapter 2.3.1.1 of Volume 4 of the 2006 IPCC Guidelines.

| ∆CB=∆CG-∆CL | Equation 5 (Equation 2.7, 2006 IPCC GL) |
| --- | --- |
| ∆CB=(Ct2-Ct1)(t2-t1) | Equation 6 (Equation 2.8 (a), 2006 IPCC GL) |

| ∆CB | Annual change in carbon stocks in biomass for each land sub-category, in tones C yr-1 |
| --- | --- |
| ∆CG | annual increase in carbon stocks due to biomass growth for each land sub-category, considering the total area, tones C yr- |
| ∆CL | annual decrease in carbon stocks due to biomass loss for each land sub-category, considering the total area, tones C yr-1 |
| Ct2 | total carbon in biomass for each land sub-category at time t2, tonnes C |
| Ct1 | total carbon in biomass for each land sub-category at time t1, tonnes C |

Following the recommendations set in chapter 2.2.2 of the GFOI Methods Guidance Document for applying IPCC Guidelines and guidance in the context of REDD+, the above equation will be simplified, and it will be assumed that: a) the annual change in carbon stocks in biomass (∆CB) due to degradation is equal to the annual decrease in carbon stocks; (b) the decrease in carbon stocks occurs the year of conversion. The long-term decrease in carbon stocks indicated in equation (1) of the GFOI MGD is assumed here to be zero. Therefore, considering the GFOI MGD the IPCC equation for forest degradation could be expressed as an Emission Factor time Activity Data as follows:

| ∆CBDEG=j EFj×A(a,b)RP | Equation 7 |
| --- | --- |

Where:

| EFj | Emission Factor for degradation of forest type a to forest type b, tones CO2 ha-1. |
| --- | --- |
| A(a,b)RP | Area of forest type a converted to forest type b (transition denoted by a,b) during the Reference Period, ha yr-1. |

Annual change in carbon stocks in biomass on non-forestland converted in forestland (∆CBreg)

Land converted to forest land CO2 removals has been estimated following the recommendations set in the Guidance Note for accounting of legacy emissions/removals of the FCPF (version 1). When non-forest land is converted to forest land, the removal of CO2 has been estimated based on the assumption that the conversion from non-forest to forest occurs over a conservative default period of 20 years.

The removal estimate takes into account changes in carbon stocks in aboveground and belowground biomass. The changes in total carbon stocks in biomass (removals) during the Reference Period were determined as the sum of the total carbon stocks in biomass of all land units with forest cohorts with ages no longer than 20 years.

The removal rate was calculated using two methods: Opt1) Calculating the biomass increment between measurements taken in NFI plots located in secondary forest, and Opt2) Estimating the Mean Annual Increment (MAI) in biomass. To apply these calculation methods, the NFI plots were evaluated and categorized by their land use type, including non-forestland use, Permanent Forest, or Secondary Forests, along with the date the forest was regenerated.

A total of 16 NFI plots were established in Secondary Forests. However, only 8 had biomass measurements after regeneration to calculate the MAI (Option 2), and only 3 plots had two measurements to obtain the biomass increment (Option 1). The worksheet “Natural Forest reg removal rate” in the Carbon Densities Tool contains the calculations to obtain the two estimates of the removal factor [1]. The MAI removal (8.97 tdm/ha/yr) was ultimately used for the removal calculation because it had a lower estimation error and was consistent with the peer review estimate made by Joshi et al. in 2021 [1].

| Statistics | Biomass increment Opt 1 tdm/ha/yr | MAI Opt 2 tdm/ha/yr |
| --- | --- | --- |
| Average | 7.27 | 9.69 |
| Standard deviation | 5.82 | 8.97 |
| n | 3 | 8 |
| CI | 9.82 | 6.01 |
| ERROR% | 135% | 62% |

[1]The updated version of Carbon densities tool (CarbonDensitiesToolV6.xlsx) can be accessed at the following link: https://docs.google.com/spreadsheets/d/1TkQ_dLmGF9Iz_h0Jx4zGRpiNJZN3UIoC/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true

[2]Joshi, V. C., Negi, V. S., Bisht, D., Sundriyal, R., & Arya, D. (2021). Tree biomass and carbon stock assessment of subtropical and temperate forests in the Central Himalayas, India. Trees, Forests and People, 6, 100147.           .

The net annual CO2 removals are calculated using equation 8. It was determined the changes in the total carbon stocks in biomass (removals) during the Reference Period as the sum of the total carbon stocks in biomass of all land units.

| ∆CBreg=LU=1n RFreg×A(i,j)RP | Equation 8 |
| --- | --- |

Where:

| RFreg | Above and belowground biomass removal rate in new forests [tCO2*ha*year-1]. |
| --- | --- |
| A(j,i)RP | Area of non-forestland i converted to forestland j (transition denoted by i,j) in the Reference Period, ha yr-1. |
| LU | Land unit. |

Monitored emissions (GHGt)

Annual gross GHG emissions over the monitoring period in the Accounting Area (GHGt) are estimated as the sum of annual change in total biomass carbon stocks (∆CBt).

| GHGt=tT ∆CLUMP,i,tT | Equation 9 |
| --- | --- |

Where:

| ∆CLUMP,i,t | = | Balance of emissions during the Monitoring Period in the Accounting Area of the ER Program that corresponds to the sum of annual change in carbon stocks and removals for each of i REDD+ activities at year t; tCO2*year-1. |
| --- | --- | --- |
| T | = | Number of years during the monitoring period; dimensionless. |

Annual change in total biomass carbon stocks forest land converted to another land-use category (∆CBdefo,t)

The annual change in total biomass carbon stocks forest land converted to other land-use category (∆CBdefo,t) would be estimated through Equation 4 above. Making the same assumptions as described above for the RL the change of biomass carbon stocks could be expressed with the following equation:

| ∆CBt=j,i BBefore,j- BAfter,i x CF x4412 × Aj,iMP | Equation 10 |  |
| --- | --- | --- |

Where:

| Aj,iRP | Area converted/transited from forest type j to non-forest type i during the Monitoring Period, in hectares per year. In this case, fiveteen forest land conversions are possible: Number Transition Initial Final 1 Primary Forest Lost Intact Forest Grasslands 2 Primary Forest Lost Intact Forest Other Land 3 Primary Forest Lost Intact Forest Settlements 4 Primary Forest Lost Intact Forest Unshaded Cropland (TCC 10% or less) 5 Primary Forest Lost Degraded Forest Grasslands 6 Primary Forest Lost Degraded Forest Other Land 7 Primary Forest Lost Degraded Forest Settlements 8 Primary Forest Lost Degraded Forest Unshaded Cropland (TCC 10% or less) 9 Primary Forest Lost Very Degraded Forest Grasslands 10 Primary Forest Lost Very Degraded Forest Other Land 11 Primary Forest Lost Very Degraded Forest Settlements 12 Primary Forest Lost Very Degraded Forest Unshaded Cropland (TCC 10% or less) 13 Secondary Forest Lost Secondary forest Other Land 14 Secondary Forest Lost Secondary forest Unshaded Cropland (TCC 10% or less) 15 Secondary Forest Lost Secondary forest Grasslands With Cochran (1977) formulas it is not possibles to calculate a standard deviation (SD) for the activity data estimate. Instead, the sampling error calculation is based on the standard error (SE), which quantifies the precision of the estimates based on the sample data. In Monte Carlo simulations de SD is used to represent the variability of input parameters. Initially, because no SD was available for the activity data, the country used the 90th percentile confidence interval and the normal probability density function (PDF). Later on, the Monte Carlo simulation was updated to utilize the standard error (SE) in the normal PDF functions when no standard deviation (SD) was available. Estimates and associated uncertainties are produced in the Activity Data Tool (Nepal_TAL_AD_tool_2sdERMR.xlsx) for each combination considering the stratification applied. The Activity Data tool comprises various spreadsheets that estimate different types of Activity Data. These include the Dataset that is used to estimate sample-based Activity Data (Reference_Data sheet), as well as spreadsheets for estimating Activity Data for deforestation (Deforestation sheet), forest restoration (Forest_gain sheet), and area of change in canopy cover (loss and gain) in permanent forest lands (Degradation sheet). To ensure accurate Activity Data estimates, material errors are controlled through specific mechanisms in the estimation spreadsheets. This includes matching forest type sampling points with sample size to prevent double counting in the sample-based Activity Data estimate. The accuracy of deforested, forest gain, and degraded areas are checked in cells Deforestation: E26-G38/E56-G68, Forest_gain: D17-F29, and Degradation: D23-F35 respectively. Before reporting AD values, a quality assurance/quality control procedure is conducted to verify that all these cells are labeled “Ok”. |
| --- | --- |
| BBefore,j | Total biomass of forest type j before conversion/transition, in tons of dry matter per ha. This is equal to the sum of aboveground (AGBBefore,j) and belowground biomass (BGBBefore,j) and it is defined for each forest type. |
| BAfter, i | Total biomass of non-forest type i after conversion, in tons dry matter per ha. This is equal to the sum of aboveground (AGBAfter,i) and belowground biomass (BGBAfter,i) and it is defined for each of the non-forest IPCC Land Use categories. |
| CF | Carbon fraction of dry matter in tC per ton dry matter. The value used is: - 0.47 is the default for (sub)tropical forest as per IPCC AFOLU guidelines 2006, Table 4.3. |
| 44/12 | Conversion of C to CO2 |
| R: :S | Root-to-shoot ratio (0.44). |

Annual change in carbon stocks in biomass on forestland remaining forestland (∆CBdeg,t)

The Annual change in carbon stocks in biomass on forestland remaining forestland (∆CBdeg,t) would be estimated through Equation 7 above. Making the same assumptions as described above for the RL the change of biomass carbon stocks could be expressed with the following equation:

| ∆CBDEG=j EFj×A(a,b)MP | Equation 11 |
| --- | --- |

Where:

| EFj | Emission Factor for degradation of forest type a to forest type b, tones CO2 ha-1. |
| --- | --- |
| A(a,b)MP | Area of forest type a converted to forest type b (transition denoted by a, b) during the Monitoring Period, ha yr-1. |

Annual change in carbon stocks in biomass on non-forestland converted in forestland (∆CBreg)

Annual change in carbon stocks in biomass on forestland remaining forestland (∆CBreg) would be estimated through Equation 8 above. Making the same assumptions as described above for the RL the change of biomass carbon stocks could be expressed with the following equation:

| ∆CBreg=LU=1n RFreg×A(i,j)MP | Equation 12 |
| --- | --- |

Where:

| RFreg | Above and belowground biomass removal rate in new forests [tCO2*ha*year-1]. |
| --- | --- |
| A(j,i)MP | Area of non-forestland i converted to forestland j (transition denoted by i,j) in the Monitoring Period, ha yr-1. |
| LU | Land unit. |

>>

# Data and parameters

## Fixed Data and Parameters

| Parameter: | BBefore,j Equation 4; BAfter,i Equation 4 |
| --- | --- |
| Description: | B Before: Total biomass of forest type j before conversion/transition, in tons of dry matter per ha. This is equal to the sum of aboveground (AGB_(Before,j)) and belowground biomass (BGB_(Before,j)) and it is defined for each forest type. B after: Total biomass of non-forest type i after conversion, in tons dry matter per ha. This is equal to the sum of aboveground (AGBAfter,i) and belowground biomass (BGBAfter,i) and it is defined for each of the non-forest Land Use categories. |
| Data unit: | Tonne/ha (dry matter), |
| Source of data or description of the method for developing the data including the spatial level of the data (local, regional, national, international): | The carbon densities used for the ER monitoring report are Tier 2 (country specific data) and has been derived from the latest NFI (FRA) except the removal rates for forest plantation and shaded crops. The NFI (FRA) involved remeasurement in 2022 of the permanent sample plots established by the FRA Nepal Project (2010-2014) including an additional number of plots established and measured using the same methodology. Nepal is conducting NFI by re-measuring the permanent sample plots at an interval of every five years. NFI (FRA) Inventory Design: The Inventory Design adopted was based largely on methods developed by Kleinn (1994) and finalized by the DFRS/FRA 2010-2014 (see Figure below). The detailed methodology adopted for sample selection is presented in DFRS, 2014, link: https://frtc.gov.np/downloadfile/The-TeraiForestsofNepal_1579845265.pdf. NFI data from 622 permanent sample plots located within the ER accounting area were derived (see NFI_dataset sheet in Carbon density calculation tool – CarbonDensitiesTools.xlsx). Inventory Sample plot design and data collection: The Concentric Circular Sample Plot (CCSP) design was adopted as used by the FRA Nepal Project (2010-2014). Each sample plot had four concentric circles of different radii (see Figure below), which were used to measure trees with different DBH as follows: ▪ trees having 30 cm DBH or more enumerated within a 20 m radius plot (area: 1256.6 m2) ▪ trees having 20-29.9 cm DBH enumerated within a 15 m radius plot (area:706.9 m2) ▪ trees having 10-19.9 cm DBH enumerated within an 8 m radius plot (area:201.0 m2) ▪ trees having 5-9.9 cm DBH enumerated within a 4 m radius plot (area: 50.3 m2) Other subplots were established to assess forest attributes other than trees, such as dead woods and disturbances, seedlings, saplings, shrubs, and herbs, etc. Layout of the concentric circular plot with other sub-plots The plots were used for data collection of standing trees (diameter at breast height (dbh) ≥5 cm), which were used in the estimation of the biomass and carbon stocks. Data collected included tree information (bearing, distance from plot center, species code, local name, scientific name, DBH, quality class, crown class and total and crown heights. In addition, data on other important variables like dead woods, disturbances, shrub and small trees, soil characteristics and soil samples, leaf litter and debris, non-wood forest products, epiphytes, parasites, herbaceous plants, bamboo, invasive and alien plant species, forest diseases and pests, etc. have been collected in regular NFI/FRA. One of the important characteristics of NFI in Nepal is hidden permanent sample plots leaving “no marks” above ground. Instead, the plots are georeferenced and plot centers consist of metal pegs inserted a few inches below the ground level. The reason behind hidden plots in NFI is to maintain consistency in anthropogenic activities and forest products use by local people both inside and outside the plots. This characteristics of NFI plots of Nepal might even aid to control leakage of GHG emission. Volume and Biomass estimation: Tree stem volumes and biomass were estimated using standard methodology with national allometric equations adopted since NFI / FRA 2010-2014. To ensure consistency between the Emission Factors and land-use transitions area, the NFI plots were evaluated and categorized according to their land use type, such as non-Forest land use, Permanent Forest, or Secondary Forests, for the current monitoring report. The same time series analysis and data collection methods used in CEO were replicated for the NFI permanent plot locations. Additionally, the canopy cover of Permanent Forest plots was evaluated to determine whether they were intact (7-9 points), degraded (4-6 points), or very degraded forest (1-3 points). Carbon density estimates: The carbon density estimates for forest and non-forest areas are calculated using data from the NFI biomass plots. The NFI gives two measurements of biomass: the first was taken between 2011 and 2013, and the second in 2022. It's important to note that the first measurement was made during the ER-Program Reference Period 2004-2014. The determination of average carbon densities for non-forest lands is based on 21 NFI plots, which provided biomass estimates for grassland, other land, and unshaded cropland. These estimates were obtained during the NFI's initial measurement phase, which was measured between 2011 and 2013. The initial carbon density estimates for natural forests were based on the second measurement. However, these estimates were made after the signing of the ERPA in February 2021. Therefore, the carbon densities of natural forests were recalculated using only the first measurement (2011-2013) [1]. The carbon densities of intact, degraded, and very degraded natural forests were recalculated using the first measurement from NFI's 388 plots. The determination of average carbon densities for non-forest lands was based on 21 NFI plots, which provided biomass estimates for grassland, other land, and unshaded cropland. These estimates were also obtained during the initial measurement phase of the NFI. The plots are classified as unshaded cropland based on experts’ knowledge, experiences, the extent of the forest patch, and analysis of near-temporal images and changes in the land cover. In a conservative approach, Nepal has decided to use the country-specific average value for unshaded crops instead of the IPCC default factor. This is because the country-specific value is larger than the IPCC default value, resulting in a smaller emission factor for the transition from forest to unshaded crops. [1] The updated version of Carbon densities tool (CarbonDensitiesToolV6.xlsx) can be accessed at the following link: https://docs.google.com/spreadsheets/d/17jMImCkjKGJjfAOZnfHItgJkjls9Rktm/edit?usp=share_link&ouid=100991295489415488908&rtpof=true&sd=true |
| Value applied: | Due to the homogeneity of the forest in the Emission Reduction Program accounting area, the whole forest was considered as the same unit for the calculation. Forest type; Average; CI; Unit. Natural intact forest; 202.08; 9.78; tdm/ha. Natural degraded forest; 101.93; 37.93; tdm/ha. Natural very degraded forest; 18.49; 10.68; tdm/ha. Non- Forest Lands; Average; CI; Unit. Grassland; 20.95; 17.96; tdm/ha. Other land; 27.37; 32.32; tdm/ha. Unshaded cropland; 57.54; 28.67; tdm/ha. Note: It was assumed the carbon density of grasslands for Settlements. Land Cover; AGB (tCO2e/ha); BGB (tCO2e/ha); Total Biomass (AGB+BGB). Intact Forest; 348.26; 153.23; 501.49. Degraded Forest; 175.65; 77.29; 252.94. Very Deg Forest; 31.86; 14.02; 45.88. grassland; 36.10; 68.12; 104.22. other land; 47.17; 89.01; 136.18. unshaded cropland; 99.16; 187.11; 286.27. Below-ground biomass for forest lands was estimated using a root-to-shoot ratio of 0.44 (2019 refinement to the 2006 IPCC guidelines for national GHG inventory, Table 4.4 -Subtropical dry Asia, . B > 125 tons/ha). BGB for non-forest lands was estimated using R::S 1.887 (Mokany et al., 2006; Table 2). |
| QA/QC procedures applied | Quality assurance of forest inventory data: Use of periodically revised field manual, training to field members and regular monitoring and feedback were some of the measures applied to maintain the quality of the inventory results. For the statistical analysis to check the quality of the results, over 10% of the total Permanent Sampling Plots measured were systematically selected (with a random start) and re-measured (QAQC manual). Furthermore, standard protocols and manuals on modeling of required parameters e.g., diameter-height modeling & taper function curve, calculation of volume and biomass using the allometric models, and error estimation were developed under supervision of the experts from Finnish Forest Research Institute (METLA, now LUKE Finland) during the FRA 2010-2014. Also, documentation on the assemble of the QAQC protocol and QAQC report of 2022 are available in the QAQC manual. |
| Uncertainty associated with this parameter: | To determine the uncertainty in carbon density, we calculated the half-width of the 90% confidence interval as a percentage of the estimated emissions. This calculation only takes sampling errors into account and does not consider model or allometric errors. The Country used a local volume tree equation to estimate biomass. The volume of the tree, which is further converted into biomass and carbon, is calculated using the allometric equation developed by Sharma and Pukala in 1990. There are more than 21 species of trees with specific parameters, along with two additional groups of species found in lower and higher altitudes, each with their respective parameters. The maximum and minimum standard error percentages of the regression model are 9.9% and 5.8%, respectively. The R2 of the model for every species is higher than 95% (Sharma and Pukala, 1990). The country has not determined whether the uncertainty from this source is lower than the uncertainty from sampling error. Since the country is unable to include this error source in the Monte Carlo simulation due to a lack of covariance table, the sampling uncertainty of carbon density in different land uses based on the NFI dataset has been increased by 10% at a 90% confidence level using the quadrature approach (see %E adj in tables below). The combined error was then included in the Monte Carlo simulation. Forest type; Average; CI; %E; %E adj; n; Std Dev; Unit. Natural intact forest; 202.08; 9.78; 5%; 11%; 367.00; 113.57; tdm/ha. Natural degraded forest; 101.93; 37.93; 37%; 39%; 14.00; 80.15; tdm/ha. Natural very degraded forest; 18.49; 10.68; 58%; 59%; 7.00; 14.54; tdm/ha. Non- Forest Lands; Average; CI; % E; %E adj; n; Std Dev; Unit. Grassland; 20.95; 17.96; 86%; 86%; 10; 30.99; tdm/ha. Other land; 27.37; 32.32; 118%; 118%; 6; 39.28; tdm/ha. Unshaded cropland; 57.54; 28.67; 50%; 51; 5; 30.07; tdm/ha. The table below displays the results of the normality test conducted on carbon densities and removal rate estimations for various land uses in Nepal's ER-Program. The test was based on NFI biomass plots and utilized the Shapiro-Wilk Test calculated with the Real Statistics Resource Pack [1]. All parameters demonstrate a normal distribution, with the exception of Intact Forest and Grasslands. It's worth noting that four out of the seven parameters are calculated using less than eight data points, which makes it challenging to fit a probability density function (PDF) distribution to these parameters. As a result, following the FCPF Guidance note on estimating the uncertainty of Emission Reductions (ERs) using Monte Carlo simulation, the country has opted to use the bootstrapping approach instead of a specific non-normal distribution to sample the values of Carbon Densities for use in the simulation. According to the FCPF guidance, sampling from a dataset has the advantage that no assumptions are needed about the nature of the distribution. If the distribution is not normal, then bootstrapping would be more accurate, unless the data are not representative. The Monte Carlo simulation has been updated [2] and now includes the bootstrapping sampling method for all carbon density parameters except for the Natural Forest Removal Rate. Since the removal rate follows a normal distribution, we used a normal probability density function (PDF) in the Monte Carlo simulation. The bootstrapping sampling was done using Infostat software with the Resampling method (Randomly with replacement). The resampling results for the different carbon density parameters can be found in [4], [5], [6], [7], [8], and [9]. Parameter; n; Shapiro-Wilk Normality test results. W-stat; p-value; alpha; Normal. Natural Forest Removal rate; 8; 0.836001505; 0.068507908; 0.05; Yes. Intact Forest Carbon Density [4]; 367; 0.965272874; 1.18349E-07; 0.05; No. Degraded Forest Carbon Density [5]; 14; 0.942524; 0.451617; 0.05; Yes. Very Degraded Forest Carbon Density [6]; 7; 0.952499; 0.752377; 0.05; Yes. Grassland Carbon Density [7]; 5; 0.728652; 0.001962; 0.05; No. Other Land Carbon Density [8]; 4; 0.722904; 0.010679; 0.05; No. Unshaded Crops Carbon Density [9]; 5; 0.90192; 0.420587; 0.05; Yes. [1] Real Statistics Resource Pack is available at the following link: https://real-statistics.com/free-download/ [2] The updated Monte Carlo simulation tool can be accessed at the following link: https://docs.google.com/spreadsheets/d/1WDYooTWyLeAZpffzrGYRfWWFuJMljGM-/edit?usp=share_link&ouid=100991295489415488908&rtpof=true&sd=true [3] Infostat software is available at the following link: https://www.infostat.com.ar/ [4] Intact Forest bootstrapping https://docs.google.com/spreadsheets/d/1tD49N6OB-5qAPEIeH8y4oPeTRwTQQ0nD/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true [5] Degraded Forest bootstrapping https://docs.google.com/spreadsheets/d/1IWhWH_KQsWlrh3qz_-X-Ur5AH3OliN5e/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true [6] Very Degraded Forest bootstrapping https://docs.google.com/spreadsheets/d/14rMgD0_PZeYLwBO9PXBIWjt_98NtUo1a/edit?usp=drive_link [7] Grassland bootstrapping https://docs.google.com/spreadsheets/d/1AaSR6NDF86Xqn3G67axs2xAAWTfjBhiX/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true [8] Other Land bootstrapping https://docs.google.com/spreadsheets/d/1zNyRyyK-vNAZGeQ5GFJwtvFwQYePdVDq/edit?usp=drive_link [9] Unshaded Crops bootstrapping https://docs.google.com/spreadsheets/d/192L-jqRohxAZiX3R_KLWvPoks4TbGYCJ/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true |
| Any comment: |  |

| Parameter: | RFreg Equation 8 |
| --- | --- |
| Description: | RFreg : Above and belowground biomass removal rate in new forests [tCO2*ha*year-1]. |
| Data unit: | Tonne/ha (dry matter), |
| Source of data or description of the method for developing the data including the spatial level of the data (local, regional, national, international): | The removal rates used for the ER monitoring report are Tier 2 (country specific data) and has been derived from the latest NFI (FRA) except the removal rates for forest plantation and shaded crops. The NFI (FRA) involved remeasurement in 2022 of the permanent sample plots established by the FRA Nepal Project (2010-2014) including an additional number of plots established and measured using the same methodology. Nepal is conducting NFI by re-measuring the permanent sample plots at an interval of every five years. NFI (FRA) Inventory Design: The Inventory Design adopted was based largely on methods developed by Kleinn (1994) and finalized by the DFRS/FRA 2010-2014 (see Figure below). The detailed methodology adopted for sample selection is presented in DFRS, 2014, link: https://frtc.gov.np/downloadfile/The-TeraiForestsofNepal_1579845265.pdf. NFI data from 622 permanent sample plots located within the ER accounting area were derived (see NFI_dataset sheet in Carbon density calculation tool – CarbonDensitiesTools.xlsx). Inventory Sample plot design and data collection: The Concentric Circular Sample Plot (CCSP) design was adopted as used by the FRA Nepal Project (2010-2014). Each sample plot had four concentric circles of different radii (see Figure below), which were used to measure trees with different DBH as follows: ▪ trees having 30 cm DBH or more enumerated within a 20 m radius plot (area: 1256.6 m2) ▪ trees having 20-29.9 cm DBH enumerated within a 15 m radius plot (area:706.9 m2) ▪ trees having 10-19.9 cm DBH enumerated within an 8 m radius plot (area:201.0 m2) ▪ trees having 5-9.9 cm DBH enumerated within a 4 m radius plot (area: 50.3 m2) Other subplots were established to assess forest attributes other than trees, such as dead woods and disturbances, seedlings, saplings, shrubs, and herbs, etc. Layout of the concentric circular plot with other sub-plots The plots were used for data collection of standing trees (diameter at breast height (dbh) ≥5 cm), which were used in the estimation of the biomass and carbon stocks. Data collected included tree information (bearing, distance from plot center, species code, local name, scientific name, DBH, quality class, crown class and total and crown heights. In addition, data on other important variables like dead woods, disturbances, shrub and small trees, soil characteristics and soil samples, leaf litter and debris, non-wood forest products, epiphytes, parasites, herbaceous plants, bamboo, invasive and alien plant species, forest diseases and pests, etc. have been collected in regular NFI/FRA. One of the important characteristics of NFI in Nepal is hidden permanent sample plots leaving “no marks” above ground. Instead, the plots are georeferenced and plot centers consist of metal pegs inserted a few inches below the ground level. The reason behind hidden plots in NFI is to maintain consistency in anthropogenic activities and forest products use by local people both inside and outside the plots. This characteristics of NFI plots of Nepal might even aid to control leakage of GHG emission. Volume and Biomass estimation: Tree stem volumes and biomass were estimated using standard methodology with national allometric equations adopted since NFI / FRA 2010-2014. To ensure consistency between the Emission Factors and land-use transitions area, the NFI plots were evaluated and categorized according to their land use type, such as non-Forest land use, Permanent Forest, or Secondary Forests, for the current monitoring report. The same time series analysis and data collection methods used in CEO were replicated for the NFI permanent plot locations. Additionally, the canopy cover of Permanent Forest plots was evaluated to determine whether they were intact (7-9 points), degraded (4-6 points), or very degraded forest (1-3 points). Removal Factor Calculation: The calculation for the rate of regrowth removal in the forest is based on a sample of sixteen NFI plots set up in secondary forests. Three plots have biomass measurements for both 2022 and 2011-2014, five have measurements only for 2022, and eight have measurements only for 2011-2013. The removal rate was calculated using two methods: Opt1) Calculating the biomass increment between measurements taken in NFI plots located in secondary forest, and Opt2) Estimating the Mean Annual Increment (MAI) in biomass. To apply these calculation methods, the NFI plots were evaluated and categorized by their land use type, including non-forestland use, Permanent Forest, or Secondary Forests, along with the date the forest was regenerated. A total of 16 NFI plots were established in Secondary Forests. However, only 8 had biomass measurements after regeneration to calculate the MAI (Option 2), and only 3 plots had two measurements to obtain the biomass increment (Option 1). The worksheet “Natural Forest reg removal rate” in the Carbon Densities Tool contains the calculations to obtain the two estimates of the removal factor [1]. The MAI removal (9.69 tdm/ha/yr) was ultimately used for the removal calculation because it had a lower estimation error and was consistent with the peer review estimate made by Joshi et al. in 2021 [1]. Statistics; Biomass increment Opt 1 tdm/ha/yr; MAI Opt 2 tdm/ha/yr. Average; 7.50; 9.69. Standard deviation; 5.98; 8.97. n; 3; 8. CI; 10.09; 6.01. ERROR%; 134%; 62%. [1] The updated version of Carbon densities tool (CarbonDensitiesToolV6.xlsx) can be accessed at the following link: https://docs.google.com/spreadsheets/d/17jMImCkjKGJjfAOZnfHItgJkjls9Rktm/edit?usp=share_link&ouid=100991295489415488908&rtpof=true&sd=true |
| Value applied: | In order to calculate the Emission Reductions, the entire forest was treated as a single unit due to its uniformity. The removal rate in new forests is country specific data and has been derived from the NFI (FRA). For Plantation forests and Shaded croplands, removal factors established by the IPCC were utilized. Forest type; Average; CI; Unit. Natural secondary forest gain; -9.69; 6.01; tdm/ha/yr. Plantation forest gain [1]; -13.79; 4.40; tCO2/ha/yr. Shaded cropland gain [2]; -10.23; 2.46; tCO2/ha/yr. [1] Table 4.10 (Updated) ABOVE-GROUND NET BIOMASS GROWTH IN TROPICAL AND SUB-TROPICAL PLANTATION FORESTS (TONNES D.M. HA-1 YR-1). 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Chapter 4: Forest Land. [2] Table 5.2 (Updated) DEFAULT COEFFICIENTS FOR ABOVE- AND BELOW-GROUND BIOMASS IN AGROFORESTRY SYSTEMS CONTAINING PERENNIAL SPECIES. 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Chapter 5: Cropland. |
| QA/QC procedures applied | Quality assurance of forest inventory data: Use of periodically revised field manual, training to field members and regular monitoring and feedback were some of the measures applied to maintain the quality of the inventory results. For the statistical analysis to check the quality of the results, over 10% of the total Permanent Sampling Plots measured were systematically selected (with a random start) and re-measured (QAQC manual). Furthermore, standard protocols and manuals on modeling of required parameters e.g., diameter-height modeling & taper function curve, calculation of volume and biomass using the allometric models, and error estimation were developed under supervision of the experts from Finnish Forest Research Institute (METLA, now LUKE Finland) during the FRA 2010-2014. Also, documentation on the assemble of the QAQC protocol and QAQC report of 2022 are available in the QAQC manual. |
| Uncertainty associated with this parameter: | To determine the uncertainty in removal rates, we calculated the half-width of the 90% confidence interval as a percentage of the estimated emissions. However, for Natural Secondary Forest gain, this calculation only considers sampling errors and does not consider model or allometric errors. In the case of forest plantations, the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Chapter 4 - Forest Land, Table 4.10 does not provide any reference to uncertainty. Therefore, it was assumed that the uncertainty of Natural Secondary Forests gain applies to Plantation Forests as well. When it comes to shaded cropland gain, we used the 2006 IPCC reference to uncertainty in Table 5.2. Forest type; Average; CI; % Error; n; Dev Std; Unit. Natural secondary forest gain; -9.69; 6.01; 62%; 8; 8.97; tdm/ha/yr. Plantation forest gain; -13.79; 4.40[1]; 32%; -; -; tCO2/ha/yr. Shaded cropland gain; -10.23; 2.46[2]; 24%; -; -; tCO2/ha/yr. [1] In the 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Chapter 4 – Forest Land, the Table 4.10 does not have any reference to uncertainty. As a result, it was assumed that the uncertainty of Natural secondary forest for Plantation Forest. [2] Uncertainty indicated in Table 5.2 (Updated) of 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Chapter 5: Cropland. |
| Any comment: |  |

| Parameter: | Activity Data: Aj,iRP Equation 4; A(a,b)RP Equation 7; A(i,j)RP Equation 8. |
| --- | --- |
| Description: | - Deforestation: Area converted/transited from forest type j to non-forest type I during the Reference Period - Degradation: Area of forest type a converted to forest type b (transition denoted by a,b) during the Reference Period, ha yr-1 - Forest gain: Area of non-forestland i converted to forestland j (transition denoted by i,j) in the Reference Period, ha yr-1. |
| Data unit: | hectare |
| Source of data and description of measurement/calculation methods and procedures applied: | Nepal uses a sample-based approach to estimate the Activity Data for Deforestation, Forest gain, and Degradation. This approach ensures unbiased estimates of the area and the error associated with the map. A forest change map spanning from 2004 to 2021 is used for the sample design, and four mapping algorithms are used to map areas experiencing forest loss, degradation, and/or regrowth. The agreement map is used for sample design, and reference data are collected through a time series analysis of 1,522 sampling plots in CEO. Forest change mapping: The following four mapping algorithms that utilize remote sensing imagery, training data points, land cover maps, and time series data analysis were used to map areas experiencing forest loss, degradation, and/or regrowth. - CCDC-SMA: Continuous Change Detection and Classification – Spectral Mixture Analysis (CCDC-SMA) monitors abrupt and gradual forest degradation. - CODED: Continuous Degradation Detection (CODED) algorithm detects forest canopy disturbances and classifies them as degradation or deforestation based on land cover. CODED uses linear spectral unmixing to generate subpixel fractions of spectral endmembers, which are used to calculate a time series of the Normalized Degradation Fraction Index (NDFI). - LandTrendr: The LandTrendr algorithms use simple statistical techniques to simplify a time-series of spectral values into a sequence of connected straight-line segments that capture the overall shape of that pixel’s trajectory while omitting year-to-year noise. The resultant segments can then be examined to select periods where the trajectory displays behaviors of interest such as disturbance or growth. - MTDD: Multi-variate Time-series Disturbance Detection (MTDD) classifies initially forested areas into stable forest, degraded, and deforested by training a random forest classifier with 66 metrics. These metrics are derived from six annual time-series (i.e., NDVI, two SWIR spectral regions, two NDWI indices, and SAVI) which are used to calculate eleven descriptive statistics (i.e., minimum, maximum, range, mean, standard deviation, coefficient of variation, kurtosis, skewness, slope, maximum 5-year slope, and most recent value). Overall MTDD’s process includes five main steps: (1) making annual time series, (2) calculating 11 descriptive statistics for the time series, (3) generating training/validation points, (4) training a random forest classifier, and (5) validating the classification. Sample design: A sample-based approach is used to complete area estimation. This approach is preferred over pixel-counting methods because all maps have errors. Sample based approaches create unbiased estimates of area and the error associated with map. An agreement map generated from the results of all four methods is used for sample design. The goal is to ensure that no strata is under-sampled. The resulting strata is anywhere where 1-4 algorithms agreed there was a certain kind of change event or forest/non-forest remained stable, anywhere the different algorithms labeled different types of change events, anywhere all 4 algorithms labeled non-forest, and anywhere all 4 algorithms labeled forest. Final strata values for the agreement map and their human-readable labels are 1: DEG, 2: LOSS, 3: GAIN, 4: Non-forest, and 5: Forest. The number of points randomly selected depend on the relative area available in each stratum, the human resources available to do interpretations, and a target standard error. A total of 1,522 points were selected via stratified random sampling to be used for sample-based area estimation. Reference data collection (completed in CEO): To estimate emissions from deforestation, carbon enhancement removals, and forest degradation emissions, reference data were collected through a time series analysis of 1,522 sampling plots in CEO. To identify the age of forest gain cohorts and differentiate between secondary and permanent forests, the sampling points were visually interpreted for the same period that the forest change map was created (1983 to 2021). This period was divided into four subperiods: 1984-2003, 2004-2014, 2015-2017, and 2018-2021 (see Figure 4). The canopy cover was visually evaluated in permanent forest only for the years 2003/2004, 2014/2015, 2017/2018, and 2021. - Generating a CEO project from a template: Nepal created a template to collect land-use change and degradation reference data in Collect Earth Online (CEO) for the following periods: Pre-reference period (t0) – 1983-2003, Reference period (t1) – 2004-2014, First monitoring period (t2) – 2015-2017 and Second monitoring period (t3) – 2018-2021. - Sampling unit: The Sampling Unit (SU) is a 70 x 70 meter plot. Inside SU, a 3x3 points sub-grid (9 points) was created to estimate forest canopy cover percentage within each sampling unit. - Number of Sampling Units: A total of 1,522 sampling points, selected via stratified random sampling, were visually assessed. - Interpretation key: Nepal produced an interpretation key that should be reused and updated as needed. The land use categories considered are the following: Forest lands:; Non-forest lands. 1 Intact Forest 2 Degraded Forest Very 3 Degraded Forest 4 Secondary natural forest 5 Plantation Forest 6 Shaded croplands; 7 Grasslands 8 Other lands 9 Settlements 10 Unshaded croplands (tree canopy cover 10% or less). Area and uncertainty estimation: Nepal employs a sample-based approach to estimate the Activity Data for Deforestation, Forest gain and Degradation. All 1,522 samples were used as the basis for calculating area estimates and their uncertainty. The estimation of Activity Data was done using the stratified random estimator based on the formulas described by Cochran (1977). Estimates are made for each of the land use categories considered (10 classes) and in terms of changes from one period to another representing a total of more than 26 effective combinations (Deforestation 14, Forest Gain 3, and Degradation 9). |
| Value applied | Deforestation Initial; Final; 2004-2014. Area (ha); ±90% CI; Area (ha*yr-1). Intact Forest; Grasslands; 391; 639; 35.55. Intact Forest; Other Land; 1,564; 1,261; 142.18. Intact Forest; Settlements; 1,676; 2,199; 152.36. Intact Forest; Unshaded Cropland (TCC 10% or less); 4,818; 3,983; 438.00. Degraded Forest; Grasslands; -; -; -. Degraded Forest; Other Land; 3,506; 3,672; 318.73. Degraded Forest; Settlements; 2,017; 2,748; 183.36. Degraded Forest; Unshaded Cropland (TCC 10% or less); 3,897; 3,726; 354.27. Very Degraded Forest; Grasslands; -; -; -. Very Degraded Forest; Other Land; -; -; -. Very Degraded Forest; Settlements; -; -; -. Very Degraded Forest; Unshaded Cropland (TCC 10% or less); -; -; -. Secondary Natural Forest; Other Land; -; -; -. Forest gain Forest Type; 2004-2014. Area (ha); ±90% CI; Area (ha*yr-1). natural secondary forest gain; 17,136; 8,467; 1,557.82. plantation forest gain; -; -; -. shaded cropland gain; -; -; -. Degradation Initial; Final; 2004-2014. Area (ha); ±90% CI; Area (ha*yr-1). Intact forest; Intact forest; 1,125,747; 22,515; 102 340.64. Degraded forest; Degraded forest; 56,347; 14,799; 5 122.45. Very degraded forest; Very degraded forest; 10,546; 6,585; 958.73. Intact forest; Degraded forest; 5,991; 4,120; 544.64. Intact forest; Very degraded forest; 391; 639; 35.55. Degraded forest; Very degraded forest; 1,627; 2,672; 147.91. Degraded forest; Intact forest; 7,904; 5,553; 718.55. Very degraded forest; Intact forest; -; -; -. Very degraded forest; Degraded forest; 3,254; 3,780; 295.82. |
| QA/QC procedures applied: | Reference data collection: Reference data compilation: To ensure accuracy, the data collected from the CEO was compiled in R for each period of the time series analysis (t0, t1, t2, and t3). The compilation process identifies land-use interpreted points with impossible transitions, and these points are sent back to the interpreted for review, until the compilation process detects no inconsistencies. Area estimate: To ensure accurate Activity Data estimates, material errors are controlled through specific mechanisms in the estimation spreadsheet. This includes matching forest-type sampling points with sample size to prevent double counting in the sample-based Activity Data estimate. The accuracy of deforested, forest gain, and degraded areas are checked in cells Deforestation: I41-N58, Forest_gain: E19-K47, and Degradation: F30-V50 respectively. Before reporting AD values, a quality assurance/quality control procedure is conducted to verify that all these cells are labeled “Ok”. |
| Uncertainty for this parameter: | The determination of the uncertainty Activity Data was not done using the standard deviation (SD) given that the formulas (Cochran, 1977) used to estimate the Activity Data do not allow for the calculation of SD. To determine the uncertainty for Activity Data, initially the country used the calculated the half-width of the 90% confidence interval as a percentage of the estimated emissions and the normal probability density function (PDF). This calculation only takes sampling errors into account and does not consider the interpreter error. Later on, the Monte Carlo simulation was updated to utilize the standard error (SE) in the normal PDF functions when no standard deviation (SD) was available. Deforestation Initial; Final; 2004-2014. Area (ha); ±90% CI; %E. Intact Forest; Grasslands; 391; 639; 164%. Intact Forest; Other Land; 1,564; 1261; 81%. Intact Forest; Settlements; 1,676; 2199; 131%. Intact Forest; Unshaded Cropland (TCC 10% or less); 4,818; 3983; 83%. Degraded Forest; Grasslands; -; -; -. Degraded Forest; Other Land; 3,506; 3672; 105%. Degraded Forest; Settlements; 2,017; 2748; 136%. Degraded Forest; Unshaded Cropland (TCC 10% or less); 3,897; 3726; 96%. Very Degraded Forest; Grasslands; -; -; -. Very Degraded Forest; Other Land; -; -; -. Very Degraded Forest; Settlements; -; -; -. Very Degraded Forest; Unshaded Cropland (TCC 10% or less); -; -; -. secondary natural forest; other land; -; -; -. secondary natural forest; other land; -; -; -. Forest gain Forest Type; 2004-2014. Area (ha); ±90% CI; %E. natural secondary forest gain; 17,136; 8,467; 49%. plantation forest gain; -; -; -. shaded cropland gain; -; -; -. Degradation Initial; Final; 2004-2014. Area (ha); ±90% CI; %E. Intact forest; Intact forest; 1,125,747; 22,515; 2%. Degraded forest; Degraded forest; 56,347; 14,799; 26%. Very degraded forest; Very degraded forest; 10,546; 6,585; 62%. Intact forest; Degraded forest; 5,991; 4,120; 69%. Intact forest; Very degraded forest; 391; 639; 164%. Degraded forest; Very degraded forest; 1,627; 2,672; 164%. Degraded forest; Intact forest; 7,904; 5,553; 70%. Very degraded forest; Intact forest; -; -; -. Very degraded forest; Degraded forest; 3,254; 3,780; 116%. Note: Stable forest areas are reported under “Degradation” for reference, ensuring all possible transitions have been considered in the degradation activity data estimate. |
| Any comment: |  |

## Monitored Data and Parameters

| Parameter: | Activity Data: Aj,iMP Equation 10; A(a,b)MP Equation 11; A(i,j)MP Equation 12. |
| --- | --- |
| Description: | - Deforestation: Area converted/transited from forest type j to non-forest type i during the Monitoring Period - Degradation: Area of forest type a converted to forest type b (transition denoted by a,b) during the Monitoring Period, ha yr-1 - Forest gain: Area of non-forestland i converted to forestland j (transition denoted by i,j) in the Monitoring Period, ha yr-1. |
| Data unit: | hectares per year |
| Value monitored during this Monitoring / Reporting Period: | Deforestation Initial; Final; 2022-2024. Area (ha); ±90% CI; Area (ha*yr-1). Intact Forest; Grasslands; 0; 0; 0. Intact Forest; Other Land; 0; 0; 0. Intact Forest; Settlements; 0; 0; 0. Intact Forest; Unshaded Cropland (TCC 10% or less); 782; 900; 261. Degraded Forest; Grasslands; 0; 0; 0. Degraded Forest; Other Land; 1,607; 2,640; 536. Degraded Forest; Settlements; 0; 0; 0. Degraded Forest; Unshaded Cropland (TCC 10% or less); 0; 0; 0. Very Degraded Forest; Grasslands; 391; 639; 130. Very Degraded Forest; Other Land; 3,250; 3,772; 1,083. Very Degraded Forest; Settlements; 391; 639; 130. Very Degraded Forest; Unshaded Cropland (TCC 10% or less); 2,016; 2,745; 672. secondary natural forest; other land; 0; 0; 0. Forest gain Forest Type; 2022-2024. Area (ha); ±90% CI; Area (ha*yr-1). natural secondary forest gain; 19,156; 8,888; 6,385. plantation forest gain; -; -; -. shaded cropland gain; -; -; -. Degradation Initial; Final; 2022-2024. Area (ha); ±90% CI; Area (ha*yr-1). Intact forest; Degraded forest; 9,907; 5,680; 3,302. Intact forest; Very degraded forest; 0; 0; 0. Degraded forest; Very degraded forest; 0; 0; 0. Degraded forest; Intact forest; 14,233; 7,478; 4,744. Very degraded forest; Intact forest; 0; 0; 0. Very degraded forest; Degraded forest; 3,641; 3,826; 1,214. |
| Source of data and description of measurement/calculation methods and procedures applied: | Nepal uses a sample-based approach to estimate the Activity Data for Deforestation, Forest gain, and Degradation. This approach ensures unbiased estimates of the area and the error associated with the map. A forest change map spanning from 2022 to 2024 is used for the sample design. This stratification map is a revised version of the reference period (2004-2021) stratification map, which used four mapping algorithms to map areas experiencing forest loss, degradation, and/or regrowth. Sub-strata for detected changes within 2022-2024 were added to the strata in the reference period map where these changes were logically possible. The monitoring period sub-strata for deforestation/degradation were created only using the Continuous Change Detection and Classification – Spectral Mixture Analysis (CCDC-SMA) algorithm. The sub-strata for forest gain were created by comparing pre-monitoring period local land cover maps with a thresholded tree canopy cover map for 2024. The final monitoring period stratification map (9-strata) is used for sample design, and reference data are collected through a time series analysis of 1,961 sampling plots in CEO. Forest change mapping: The following four mapping algorithms that utilize remote sensing imagery, training data points, land cover maps, and time series data analysis were used to map areas experiencing forest loss, degradation, and/or regrowth. All four algorithms were used to make the original reference period map. Of these, only CCDC-SMA was used to update the map with changes detected in the monitoring period. This algorithm performed best when run for the short time period of interest, matching local-expert evaluations of reasonable detections that were not a major overestimation or underestimation of forest disturbances. - CCDC-SMA: Continuous Change Detection and Classification – Spectral Mixture Analysis (CCDC-SMA) monitors abrupt and gradual forest degradation. CCDC-SMA looks for deviations of Normalized Degradation Fraction Index (NDFI) from models of typical NDFI variations, in order to detect forest disturbances. — The version of CCDC-SMA used in the reference period was adapted so that instead of using a local land cover map from the ending year (2024) to distinguish degradation from deforestation, the adaptation used a >20% thresholded Tree Canopy Cover map. This was because no local 2024 land cover map was yet available at the time of analysis. - CODED: Continuous Degradation Detection (CODED) algorithm detects forest canopy disturbances and classifies them as degradation or deforestation based on land cover. CODED uses linear spectral unmixing to generate subpixel fractions of spectral endmembers, which are used to calculate a time series of the NDFI. - LandTrendr: The LandTrendr algorithms use simple statistical techniques to simplify a time-series of spectral values into a sequence of connected straight-line segments that capture the overall shape of that pixel’s trajectory while omitting year-to-year noise. The resultant segments can then be examined to select periods where the trajectory displays behaviors of interest such as disturbance or growth. - MTDD: Multi-variate Time-series Disturbance Detection (MTDD) classifies initially forested areas into stable forest, degraded, and deforested by training a random forest classifier with 66 metrics. These metrics are derived from six annual time-series (i.e., NDVI, two SWIR spectral regions, two NDWI indices, and SAVI) which are used to calculate eleven descriptive statistics (i.e., minimum, maximum, range, mean, standard deviation, coefficient of variation, kurtosis, skewness, slope, maximum 5-year slope, and most recent value). Overall MTDD’s process includes five main steps: (1) making annual time series, (2) calculating 11 descriptive statistics for the time series, (3) generating training/validation points, (4) training a random forest classifier, and (5) validating the classification. The sub-strata for areas of forest gain in the monitoring period was created in a custom-made script in Google Earth Engine. The approach used the local land cover maps created as part of the National Land Cover Monitoring System (NLCMS) for Nepal. These maps have been thoroughly checked for accuracy and are the officially used annual land cover maps for Nepal. No NLCMS map was yet available for the final year of analysis, 2024, so we used a local Tree Canopy Cover (TCC) map produced by ICIMOD. Anywhere where the TCC was greater than 20% in 2024 was considered to be forest. The >20% threshold was selected by local forestry experts using visual comparison of the TCC map with areas of forest, because although >10% TCC is the definition of forest in Nepal, this value in the TCC map did not align with known areas of forest and a more aggressive threshold was required. Using these two input datasets we obtained a 2022-2024 forest gain map by looking for areas where NLCMS said there was no forest for at least the five years prior to the monitoring period and the 2024 >20% TCC map indicated forest was present in 2024. In summary the reference period map had five strata (1: Degradation, 2: Deforestation, 3: Reforestation, 4: Stable Non-Forest, 5: Stable Forest) for 2004-2021, created using a comparison of the four forest change detection algorithms. This map was updated to include sub-strata for changes identified in 2022-2024 using CCDC-SMA and NLCMS/TCC maps. The four new sub-strata were (6: Reference period stable forest with Monitoring Period Degradation, 7: Reference period degraded forest with Monitoring Period Deforestation, 8: Reference period stable forest with Monitoring Period Deforestation, and 9: Reference period stable non-forest with Monitoring Period Forest Gain). Sample design: A sample-based approach is used to complete area estimation. This approach is preferred over pixel-counting methods because all maps have errors. Sample based approaches create unbiased estimates of area and the error associated with your map. Final strata values for the monitoring stratification map and their human-readable labels are 1: "Degradation"; 2: "Deforestation"; 3: "Reforestation"; 4: "Stable Non-Forest"; 5: "Stable Forest"; 6: "SF to Monitoring Degradation"; 7: "DEG to Monitoring Deforestation"; 8: "SF to Monitoring Deforestation" and 9: "NF to Monitoring Gain". We note that these new sub-strata were entirely contained within a previous strata and did not cross stratas, allowing us to still accurately use samples from earlier analyses and still know their weight within the sampling design. The strata were created in Google Earth Engine using rules to force abiding by this constraint, via defining the strata number by overlaying the detected changes on the previous strata. The number of points randomly selected depend on the relative area available in each stratum, the human resources available to do interpretations, and a target standard error. A total of 1,961 points were randomly selected, with a specified number from that total within each strata, to be used for sample-based area estimation. For the strata with smaller areas a minimum number of 110 points was required regardless of the lesser number they would have been allocated if the distribution was done on a purely proportional basis. This is following the approach recommended by Olofsson et al. (2014 & 2020) for small strata sample allocation. It is important to clarify that the 1,961 points used to calculate activity data for the second reporting period include 1,521 points that were also used for the reference and first reporting periods. To better capture land-use change (deforestation and reforestation) and degradation that occurred during 2022–2024, an additional 440 points (110 points for each of the new sub-strata) were incorporated into the original sampling effort. Reference data collection (completed in CEO): To estimate emissions from deforestation, carbon enhancement removals, and forest degradation emissions, reference data were collected through a time series analysis of 1,961 sampling plots in CEO. To identify the age of forest gain cohorts and differentiate between secondary and permanent forests, the sampling points were visually interpreted for the same period that the forest change map was created (2022 to 2024), but also looking back to 20 years to 2001, to determine whether the forest in 2022 was permanent (undisturbed for 20-years) or secondary (had been disturbed or lost sometime within the previous 20 years). The points used originally the reference period were evaluated back to 1983 in this same manner. The total period of interpretation was divided into four subperiods: 1984-2003, 2004-2014. 2015-2017, 2018-2021, and 2022-2024 (see Figure 4). The canopy cover was visually evaluated in permanent forest only for the years 2003/2004, 2014/2015, 2017/2018, 2021 and 2024. - Generating a CEO project from a template: Nepal created a template to collect land-use change and degradation reference data in Collect Earth Online (CEO). All detected changes were assigned a year by the interpreter. The labeled years of change were aggregated in post-processing into the following periods: Pre-reference period (t0)–- 1983-2003, Reference Period (t1) – 2004-2014, First monitoring period (t2) – 2015-2017, Second monitoring period (t3) – 2018-2021, and Third monitoring period (t4) – 2022-2024. - Sampling unit: The Sampling Unit (SU) is a 70 x 70 meter plot. Inside the SU, a 3x3 point sub-grid (9 points) was created to estimate forest canopy cover percentage within each sampling unit. This grid was used to reduce subjectivity in quantification of the severity of degradation. - Number of Sampling Units: A total of 1,961 sampling points, selected via stratified random sampling, were visually assessed. - Interpretation key: Nepal produced an interpretation key that should be reused and updated as needed. Note, the first three types of forest land (intact, degraded, and very degraded) were indirectly labeled in post-processing using the number of tree-covered points out of a 9-point grid over each plot. A visual supplement was added to the interpretation key to better explain the use of the 9-point grid for landcover labeling. A majority rule was used for the final landcover label of the 70x70 meter plot. Area and uncertainty estimation: Nepal employs a sample-based approach to estimate the Activity Data for Deforestation, Forest gain, and Degradation. All 1,961 samples were used as the basis for calculating area estimates and their uncertainty. The estimation of Activity Data was done using the stratified random estimator based on the formulas described by Cochran (1977). Estimates are made for each of the land use categories considered (10 classes) and in terms of changes from one period to another representing a total of more than 26 effective combinations (Deforestation 14, Forest Gain 3, and Degradation 9). |
| QA/QC procedures applied: | QAQC manual Manual Reference data collection: Reference data compilation: To ensure accuracy, the data collected from the CEO was compiled in R for each period of the time series analysis (t0, t1, t2, t3, and t4 (t4 is the monitoring period)). The compilation process identifies land-use interpreted points with impossible transitions and interpreter disagreements, and these points are sent back to the interpreted for review, until the compilation process detects no inconsistencies. Area estimate: To ensure accurate Activity Data estimates, material errors are controlled through specific mechanisms in the estimation spreadsheet (Activity Data Tool). This includes matching forest-type sampling points with sample size to prevent double counting in the sample-based Activity Data estimate. The accuracy of deforested, forest gain, and degraded areas are checked in cells Deforestation: I41-N58, Forest_gain: E19-K47, and Degradation: F30-V50 respectively. Before reporting AD values, a quality assurance/quality control procedure is conducted to verify that all these cells are labeled “Ok”. |
| Uncertainty for this parameter: | The determination of the uncertainty Activity Data was not done using the standard deviation (SD) given that the formulas (Cochran (1977)) used to estimate the Activity Data do not allow for the calculation of SD. To determine the uncertainty for Activity Data, initially the country used the calculated half-width of the 90% confidence interval as a percentage of the estimated emissions and the normal probability density function (PDF). This calculation only takes sampling errors into account and does not consider the interpreter error. Later on, the Monte Carlo simulation was updated to utilize the standard error (SE) in the normal PDF functions when no standard deviation (SD) was available. Deforestation Initial; Final; 2022-2024. Area (ha); ±90% CI; %E. Intact Forest; Grasslands; 0; 0; 0%. Intact Forest; Other Land; 0; 0; 0%. Intact Forest; Settlements; 0; 0; 0%. Intact Forest; Unshaded Cropland (TCC 10% or less); 782; 900; 115%. Degraded Forest; Grasslands; 0; 0; 0%. Degraded Forest; Other Land; 1,607; 2,640; 164%. Degraded Forest; Settlements; 0; 0; 0%. Degraded Forest; Unshaded Cropland (TCC 10% or less); 0; 0; 0%. Very Degraded Forest; Grasslands; 391; 639; 164%. Very Degraded Forest; Other Land; 3,250; 3,772; 116%. Very Degraded Forest; Settlements; 391; 639; 164%. Very Degraded Forest; Unshaded Cropland (TCC 10% or less); 2,016; 2,745; 136%. secondary natural forest; other land; 0; 0; 0%. Forest gain Forest Type; 2022-2024. Area (ha); ±90% CI; %E. natural secondary forest gain; 19,156; 8,888; 46%. plantation forest gain; -; -; -. shaded cropland gain; -; -; -. Degradation Initial; Final; 2022-2024. Area (ha); ±90% CI; %E. Intact forest; Degraded forest; 14,233; 7,478; 57%. Intact forest; Very degraded forest; 0; 0; 0%. Degraded forest; Very degraded forest; 3,641; 3,826; 0%. Degraded forest; Intact forest; 9,907; 5,680; 53%. Very degraded forest; Intact forest; 0; 0; 0%. Very degraded forest; Degraded forest; 0; 0; 105%. |
| Any comment: |  |

# Quantification of Emission Reductions

## ER Program Reference level for the Monitoring / Reporting Period covered in this report

| Year of Monitoring/Reporting Period t | Average annual historical Emissions from deforestation over the Reference Period (tCO2-e/yr) | If applicable, average annual historical Emissions from forest degradation over the Reference Period (tCO2-e/yr) | If applicable, average annual historical enhanced Removals from afforestation/reforestation (A/R) (tCO2-e/yr) | If applicable, average annual historical enhanced Removals from other activities besides A/R (tCO2-e/yr) | Adjustment, if applicable (tCO2-e/yr) | Reference Level (tCO2-e/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| 2022 | 273,539 | 182,182 | -438,508 | -239,852 |  | -222,639 |
| 2023 | 273,539 | 182,182 | -465,261 | -239,852 |  | -249,392 |
| 2024 | 273,539 | 182,182 | -492,014 | -239,852 |  | -276,145 |
| Total | 820,617 | 546,546 | -1,395,783 | -719,556 |  | - 748,176 |

## Estimation of Emissions by Sources and Removals by Sinks included in the ER Program’s scope

The following table shows emissions by sources and removals by sinks from the ER Program during the Monitoring Period. The Emission Reductions calculation tool (Nepal_TAL_Integration_tool_2sdERMR.xlsx) is available at https://drive.google.com/drive/folders/17cJC67Ixpiv1ivgkVL21_ZVOwNXEV9Wx?usp=sharing . ER estimate tool provides sample calculations using the actual values from section 3 above. This tool also includes all formulas used for the ER estimate.

| Year of Monitoring/ Reporting Period t | Emissions from deforestation (tCO2-e/yr) | If applicable, Emissions from forest degradation (tCO2-e/yr) | If applicable, enhanced Removals from afforestation/ reforestation (A/R) (tCO2-e/yr) | If applicable, enhanced Removals from other activities besides A/R (tCO2-e/yr) | Net Emissions and Removals (tCO2-e/yr) |
| --- | --- | --- | --- | --- | --- |
| 2022 | 303,884 | 820,832 | -1,037,837 | -1,430,507 | -1,343,628 |
| 2023 | 303,884 | 820,832 | -1,101,008 | -1,430,507 | -1,406,799 |
| 2024 | 303,884 | 820,832 | -1,126,736 | -1,430,507 | -1,432,527 |
| Total | 911,652 | 2,462,496 | -3,265,581 | -4,291,521 | -4,182,954 |

## Calculation of Emission Reductions

The Reporting Period aligns with the Monitoring Period, which begins on January 1st, 2022, and concludes on December 31st, 2024. No pro-rata allocation was required.

ER Program measures other than A/R that are being considered to generate enhanced removals:

Forest degradation contributes roughly 2.7 times more emissions than deforestation. It is mainly caused by a supply-demand gap for forest products like fuelwood and fodder, along with illegal and unsustainable logging in government-managed forests. Uncontrolled grazing, especially outside community forests, worsens these issues and probably hinders forest regeneration and carbon storage in permanent forests areas.

The ER program envisioned a total of seven interventions to achieve emission reductions. They include (1) improving forest management; (2) localizing forest governance through the handover of forests to local user groups; (3) expanding private sector forestry; (4) expanding access to alternative energy with biogas and improved cookstoves; (5) scaling up pro-poor leasehold forestry; (6) improving integrated land-use planning to reduce forest conversion; and (7) strengthening the management of protected areas. Some of these interventions, specifically 1, 3, 4, and 7, are implemented to reduce forest degradation and consequently enhance carbon in remaining or permanent forest lands.

| SN | Intervention | Target | Progress 2022-2024 |
| --- | --- | --- | --- |
| 1 | Improve management practices on existing community and collaborative forests building on traditional and customary practices | 336,069 ha | 217,851.481 ha (total) Community Forests: 178251.19 ha. Collaborative Forests: 39600.291 ha |
| 2 | Expand private sector forestry operations through improved access to extension services and finance | 30,141 ha | 50 new private forests of 17.42 ha registered |
| 3 | a. Expand access to alternative energy with biogas | 60,000 units | 3674 units |
|  | b. Expand access to alternative energy with improved cookstoves | 60,000 units | 431 units |
| 4 | Improve management of existing Protected Areas (PAs) | 6 PAs | 3600 ha of grassland managed, 136 number of water hole maintenance, (Chitwan national park, Bardiya national park, Banke national park, Shuklaphanta national park, Parsa National Park and Krishnasar Conservation area) |

Total Emissions, Removals and Emission Reductions:

| T.1 | Deforestation | If applicable, forest degradation | If applicable, enhanced Removals from afforestation/ reforestation (A/R) | If applicable, enhanced Removals from other activities besides A/R* | Adjustment, if applicable | Total (tCO2-e) |
| --- | --- | --- | --- | --- | --- | --- |
| Emissions or Removals in the Reference Level (tCO2-e) | 820,617 | 546,546 | -1,395,783 | -719,556 | 0 | -748,176 |
| Emissions or Removals under the ER Program during the Reporting Period (tCO2-e) | 911,652 | 2,462,496 | -3,265,581 | -4,291,521 | 0 | -4,182,954 |
| Emission Reductions during the Reporting Period (tCO2-e) | -91,035 | -1,915,950 | 1,869,798 | 3,571,965 | 0 | 3,434,778 |

Annual Emissions, Removals and Emission Reductions:

| T.2 | Deforestation |  |  | If applicable, forest degradation |  |  | If applicable, enhanced Removals from afforestation/ reforestation (A/R) |  |  | If applicable, enhanced Removals from other activities besides A/R |  |  | Adjustment, if applicable |  |  | Total (tCO2-e) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 2022 | 2023 | 2024 | 2022 | 2023 | 2024 | 2022 | 2023 | 2024 | 2022 | 2023 | 2024 | 2022 | 2023 | 2024 | 2022 | 2023 | 2024 |
| Emissions or Removals in the Reference Level (tCO2-e) | 273,539 | 273,539 | 273,539 | 182,182 | 182,182 | 182,182 | -438,508 | -465,261 | -492,014 | -239,852 | -239,852 | -239,852 |  |  |  | -222,639 | -249,392 | -276,145 |
| Emissions or Removals under the ER Program during the Monitoring Period (tCO2-e) | 303,884 | 303,884 | 303,884 | 820,832 | 820,832 | 820,832 | -1,037,837 | -1,101,008 | -1,126,736 | -1,430,507 | -1,430,507 | -1,430,507 |  |  |  | -1,343,628 | -1,406,799 | -1,432,527 |
| Emission Reductions during the Reporting Period (tCO2-e) | -30,345 | -30,345 | -30,345 | -638,650 | -638,650 | -638,650 | 599,329 | 635,747 | 634,722 | 1,190,655 | 1,190,655 | 1,190,655 |  |  |  | 1,120,989 | 1,157,407 | 1,156,382 |

* Please list below which of the ER Program Measures other than A/R that are being considered to generate enhanced Removals:

Proportion of Emission Reductions per REDD+ activity:

| T.5 | Emission Reductions during the Reporting Period (tCO2-e) |  |  |  |
| --- | --- | --- | --- | --- |
| Year |  | 2022 | 2023 | 2024 |
| Total |  | 1,120,989 | 1,157,407 | 1,156,382 |
| Emission Reductions from enhanced Removals from afforestation/reforestation as a percentage of the total FCPF ERs (%) |  | 53.46430696% | 54.92856013% | 54.88860947% |
| If applicable, enhanced Removals from other activities besides A/R (%) |  | 106.21469078% | 102.87262821% | %02.96381300% |
| If applicable, forest degradation (%) |  | -56.97201311% | -55.17937942% | -55.22828961 % |
| Deforestation (%) |  | -2.70698464% | -2.62180892% | -2.62413286 % |
| Adjustment, if applicable (%) |  | 0% | 0% | 0% |

| T.6 | Year | 2022 | 2023 | 2024 |
| --- | --- | --- | --- | --- |
| Emission Reductions from enhanced Removals from afforestation/reforestation as a percentage of the total FCPF ERs (%) |  | 0% | 0% | 0% |

Uncertainty of the estimate of Emission Reductions

## Identification, assessment and addressing sources of Uncertainty

Table 5-1 shows the nature and level of uncertainty associated with Activity Data, Emission Factors, as well as the integration model.

Figure 5-1: Levels of Uncertainty in Activity Data, Emission Factors and Integration.

| Sources of uncertainty | Systematic | Random | Analysis of contribution to overall uncertainty | Contribution to overall uncertainty (High -H /Low - L) | Addressed through QA/QC? | Residual uncertainty estimate? |
| --- | --- | --- | --- | --- | --- | --- |
| Activity Data |  |  |  |  |  |  |
| Measurement | 🗹 | 🗹 | Activity Data is based on sampling. Systematic and random errors during the visual interpretation of land-use and land-use change in satellite imagery contribute to the overall uncertainty. Nepal has taken measures to address this issue by implementing QA/QC procedures for collecting reference data. This involves using the best available imagery and providing detailed interpretation keys. The interpreters have been trained to follow the correct land-use and land-use change interpretation procedures. To guarantee accuracy, the collected reference data is compiled in R for each period of the time series analysis (t0, t1, t2, and t3). During the compilation process, land-use interpretation points with impossible transitions are identified and sent back to the interpreter for review until the compilation process detects no inconsistencies. Using a requirement in R, a quality control search is used to find impossible transitions that do not meet the logic requirement. This refers to sequential events that do not make sense between forest changes indicated in the three time periods or do not make sense with the land covers (LC) indicated in each time-period. For example, a forest gain event could not have occurred in period 2 if period 1 already had a land cover of the forest. This requirement is using the answer to the survey question “Is this answer different from the label in (previous period)”, and the questions about “Forest in (current time period)” and “Forest in (previous time period)”. Logic requirements (all other transitions would be flagged as logically impossible and requiring correction); Explanation. An answer of “No - consistent land cover of forest or non-forest” requires the previous LC and the current LC to be the same.; It is impossible to have a stable forest or stable other LC type if the LC labels for period A and period B are different.. An answer of “Yes - forest was GAINED…” requires the previous LC to be non-forest and the current LC to be a forest.; Tree densification does not count as a forest gain; it must be a conversion of LC from non-forest to forest.. An answer of “Yes - forest was LOST…” requires the previous LC to be forest and the current LC to be non-forest.; A degradation of trees does not count as a forest loss, as it requires a change in land cover from a forest to a non-forest. If any of these requirements are not met, an ‘impossible transition’ has occurred, and the QA/QC team reviews the sample plot to make sure the marked events make sense with the marked LC types. | H (bias/random) | Yes | No |
| Representativeness | 🗹 | 🗷 | Sampling was carried out over the entire accounting area and all reference and monitoring periods. It can therefore be concluded that the impact of this source of uncertainty is low. | H (bias) | Yes | No |
| Sampling | 🗷 | 🗹 | To determine the number of points needed for the study, we must consider the area of each stratum. Once the total number of samples is calculated, they must be distributed across the strata proportionally. If any of the strata receive too few samples, they should have a minimum sample size requirement, and the remaining points should be proportionally distributed to the larger strata. However, changes in the study area are small, resulting in a high variance in some change categories. To select the estimator, we follow Cochran's (1977) recommendations. | H (random/bias) | Yes | Yes |
| Extrapolation | 🗹 | 🗷 | The estimates were made based on the samples collected for which the interpretation of the land cover classes is exhaustive and covers the whole reference and monitoring periods. This source of error is therefore unlikely to be present in the approach adopted. | L (bias) | Yes | No |
| Approach 3 | 🗹 | 🗷 | This source of uncertainty exists when there is no land monitoring or IPCC Approach 3 of monitoring, which is not valid for the Nepal ER program. Four non-independent surveys were conducted covering reference and monitoring periods (t0, t1, t2, and t3), conducting lands tracking. | Not apply | Yes | No |
| Emission Factors |  |  |  |  |  |  |
| DBH measurement | 🗹 | 🗹 | The permanent sample plots were selected from the National Forest Resource Assessment. The sampling design was adopted from Forest Resource Assessment Design 2011. In the selected sample plots, all trees with more than 5 cm diameter were measured. For more details, please refer to the FRA field manual. The height of every fifth tree was measured and for the remaining trees, their height was predicted using the model developed based on the height-diameter relationship of neighboring trees. The model prepared and used during the calculation is presented in Annex 2 of the report “Terai Forests of Nepal”. A strong QA/QC was carried out for all the above-mentioned processes using the QA/QC Manual approved by the FRTC. Comprehensive training was conducted for field staff to minimize field measurement errors. In addition, the continuous monitoring of the field personnel was carried out by the FRTC’s officials. As a result of the robust QA/QC process, the error for field measurement is below 5%. | L (bias) & L (random) | Yes | No |
| H measurement | 🗹 | 🗹 |  | L (bias) & L (random) | Yes | No |
| Plot delineation | 🗹 | 🗹 |  | L (bias) & L (random) | Yes | No |
| Wood density estimation | 🗹 | 🗹 | The species-specific wood density is referenced from Table 1 of Sharma and Pukkala, 1990, Master Plan for the Forestry Sector Nepal, and Devagiri et al., 2013. | H (bias) & L (random) | Yes | No |
| Biomass allometric model | 🗹 | 🗹 | The country used a local volume tree equation to estimate biomass. The volume of the tree, which is further converted into biomass and carbon, is calculated using the allometric equation developed by Sharma and Pukkala, 1990. [Table 2 of Sharma_and_Pukkala_1990_Volume_equations_and_biomass_prediction_of_forest_trees_of_Nepal.pdf]. Note that for the species not included in Table 2-7 of Sharma and Pukkala 1990, the parameters from the Miscellaneous Categories were used, depending on the sample plots’ locations (Terai or Hills). There are more than 21 species of trees with specific parameters and an additional two groups of species found in lower altitudes and higher altitudes with their respective parameters. The maximum and minimum standard error percentage of the regression model is 9.9 % and 5.8 % respectively. The R2 of the model for every species is higher than 95 % (Sharma and Pukkala, 1990). The country has not determined whether the uncertainty from this source is lower than the uncertainty from the sampling error. Since Nepal is unable to include this error source in the Monte Carlo simulation, due to the lack of a covariance table, the sampling uncertainty of carbon density in different land uses based in the NFI dataset has been increased by 10% at a 90% confidence level using the quadrature approach. The combined error was included in the Monte Carlo simulation. | L (bias) & L (random) | Yes | Yes |
| Sampling | 🗷 | 🗹 | The sampling is done based on the Inventory Design (ID) of the National Forest Inventory. The error of the Inventory Design is 7.34% at a 95 % confidence interval. [Please refer to page 40 of the report “State of Nepal’s Forests”. However, despite the low error of the Inventory Design, the sampling error of the carbon densities for the different land uses based on the NFI varies from 5% (Intact Forest) to 148% (Grassland). Therefore, this source of uncertainty is considered high. | H (random/bias) | Yes | Yes |
| Other parameters (e.g. Carbon Fraction, root-to-shoot ratios) | 🗹 | 🗹 | Other relevant parameters includes root-to-shoot ratio and carbon fraction. Booth parameters are taken from the 2006 IPCC guideline. The error provided by the IPCC guideline is also factored in while carrying out the Monte Carlo simulation for uncertainty estimation. | H (bias/random) | Yes | Yes |
| Representativeness | 🗹 | 🗷 | The carbon densities and removal rates used for the ER monitoring report are Tier 2 (country-specific data) and have been derived from the latest NFI (FRA) except the removal rates for forest plantation and shaded crops. The NFI (FRA) involved remeasurement in 2022 of the permanent sample plots established by the FRA Nepal Project (2010-2014) including an additional number of plots established and measured using the same methodology. Nepal is conducting NFI by re-measuring the permanent sample plots at an interval of every five years. The carbon densities of natural forests categorized as intact, degraded, and very degraded were estimated using the first measurement from NFI’s 388 plots. The determination of average carbon densities for non-forest lands was based on 21 NFI plots, which provided biomass estimates for grassland, other land, and unshaded cropland. These estimates were obtained during the initial measurement phase of the NFI. The forest regrowth removal rate calculation is based on a sample of sixteen NFI plots established in secondary forests. Two biomass measurements were taken in these plots, and the difference in biomass over the years between measurements was used to estimate the average removal rate. | L (bias) | Yes | No |
| Integration |  |  |  |  |  |  |
| Model | 🗹 | 🗷 | To ensure accurate Activity Data estimates, material errors are controlled through specific mechanisms in the estimation spreadsheets. This includes matching forest-type sampling points with sample size to prevent double counting in the sample-based Activity Data estimate. The accuracy of deforested, forest gain, and degraded areas are checked in cells Deforestation: I41-N58, Forest_gain: E19-K47, and Degradation: F30-V50 respectively. Before reporting AD values, a quality assurance/quality control procedure is conducted to verify that all these cells are labeled “OK”. | L (bias) | Yes | No |
| Integration | 🗹 | 🗷 | Activity Data and Emission Factors are comparable. Carbon densities have been estimated according to the forest types, and non-forest land uses interpreted in the visual assessment. To ensure consistency, the Emission Factors (EF) have been aligned with the estimates of land-use transition area (AD). To achieve this, the same time series analysis and data collection methods that were used in CEO were replicated for the NFI permanent plot’s locations. The NFI plots have been classified as Non-Forest land use (grassland, other land, unshaded cropland), Permanent Forest, or Secondary Forests. Additionally, the canopy cover of Permanent Forest plots was evaluated to determine whether they were intact (7-9 points), degraded (4-6 points), or very degraded forest (1-3 points). | L (bias) | Yes | No |

## Uncertainty of the estimate of Emission Reductions

### Parameters and assumptions used in the Monte Carlo method

Nepal’s ER Program applied Monte Carlo methods (IPCC Approach 2) for quantifying the Uncertainty of the Emission Reductions. Because the MC propagation analysis includes 53 parameter values, it has been provided access to the uncertainty calculation tool to see all parameter values used in the analysis. The sources of uncertainty propagated in the Monte Carlo (MC) analysis are provided in the following Table.

Figure 5-2: Parameters and assumptions used in the Monte Carlo simulation.

| Parameter included in the model | Parameter values | Error sources quantified in the model (e.g., Measurement error, model error, etc.) |  | Probability distribution function | Assumptions |
| --- | --- | --- | --- | --- | --- |
|  |  | Range | %Error |  |  |
| ratio R::S | 0.44 | 0.30 | 68% | Normal Mean 0.44 ,SD 1.184 | Normal distribution only values > 0. |
| ratio R::S Grassland | 1.887 | 0.499 | 26% | Normal Mean 1.887 ,SD 0.304 | Normal distribution |
| CF | 0.47 | 0.020 | 4% | Normal Mean 0.47 ,SE 0.0120 | Normal Distribution |
| CD-natural inctat forest | 203.80 | 9.78 | 5% | Bootstrapping | Only values > 0. |
| CD-natural degraded forest | 102.41 | 37.78 | 37% | Bootstrapping | Only values > 0. |
| CD-natural very degraded forest | 19.21 | 11.98 | 62% | Bootstrapping | Only values > 0. |
| CD-grassland | 3.97 | 5.88 | 148% | Bootstrapping | Only values > 0. |
| CD-other land | 39.95 | 53.09 | 133% | Bootstrapping | Only values > 0. |
| CD-settlements | 3.97 | 5.88 | 148% | Bootstrapping | Only values > 0. |
| CD-unshaded cropland | 48.31 | 36.53 | 76% | Bootstrapping | Only values > 0. |
| RF-natural secondary forest gain | 16.69 | 9.41 | 56% | Normal Mean 16.69 ,SD 8.97 | Normal distribution only values > 0. |
| RF-plantation forest gain | 13.79 | 9.41 | 68% | Normal Mean 13.79 ,SD 8.97 | Normal distribution only values > 0. It is assumed the same SD than Nat Sec forest gain removal rate |
| RF-shaded cropland gain | 10.23 | 2.46 | 24% | Normal Mean 391 ,SE 1.50 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Grasslands-2004-2014 | 391 | 639 | 163% | Normal Mean 391 ,SE 389 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Other Land-2004-2014 | 1,564 | 1,261 | 81% | Normal Mean 1564 ,SE 767 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Settlements-2004-2014 | 1,676 | 2,199 | 131% | Normal Mean 1676 ,SE 1337 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Unshaded Cropland-2004-2014 | 4,818 | 3,983 | 83% | Normal Mean 4818 ,SE 2422 | Normal distribution only values > 0. |
| AD-Defo_Degraded Forest-Other Land-2004-2014 | 3,506 | 3,672 | 105% | Normal Mean 3506 ,SE 2233 | Normal distribution only values > 0. |
| AD-Defo_Degraded Forest-Settlements-2004-2014 | 2,017 | 2,748 | 136% | Normal Mean 2017 ,SE 1670 | Normal distribution only values > 0. |
| AD-Defo_Degraded Forest-Unshaded Cropland-2004-2014 | 3,897 | 3,726 | 96% | Normal Mean 3897 ,SE 2265 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Other Land-2018-2021 | 1,627 | 2,674 | 164% | Normal Mean 1627 ,SE 1626 | Normal distribution only values > 0. |
| AD-Defo_Intact Forest-Unshaded Cropland-2022-2024 | 782 | 900 | 115% | Normal Mean 782,SE 547 | Normal distribution only values > 0. |
| AD-Defo_Degraded Forest-Other Land-2022-2024 | 1,607 | 2,640 | 164% | Normal Mean 1607,SE 1605 | Normal distribution only values > 0. |
| AD-Defo_Very Degraded Forest-Grasslands-2022-2024 | 391 | 639 | 164% | Normal Mean 391,SE 389 | Normal distribution only values > 0. |
| AD-Defo_Very Degraded Forest-Other Land-2022-2024 | 3,250 | 3,772 | 116% | Normal Mean 3250,SE 2293 | Normal distribution only values > 0. |
| AD-Defo_Very Degraded Forest-Settlements-2022-2024 | 391 | 639 | 164% | Normal Mean 391,SE 389 | Normal distribution only values > 0. |
| AD-Defo_Very Degraded Forest-Unshaded Cropland-2022-2024 | 2,016 | 2,745 | 136% | Normal Mean 2016,SE 1669 | Normal distribution only values > 0. |
| AD-Defo_Secondary natural forest 2007-other land-2015-2017 | 1,627 | 2,674 | 164% | Normal Mean 1627 ,SE 1626 | Normal distribution only values > 0. |
| AD-Defo_Secondary natural forest 2003 -Other Land-2022-2024 | 1,623 | 2,667 | 164% | Normal Mean 1623,SE 1621 | Normal distribution only values > 0. |
| AD-Defo_Secondary natural forest 2020 -Unshaded Cropland-2022-2024 | 1,623 | 2,667 | 164% | Normal Mean 1623,SE 1621 | Normal distribution only values > 0. |
| AD-Defo_Secondary natural forest 1995 -Other Land-2022-2024 | 1,623 | 2,667 | 164% | Normal Mean 1623,SE 1621 | Normal distribution only values > 0. |
| AD-Defo_Secondary natural forest 1990 -Grasslands-2022-2024 | 1,609 | 2,315 | 144% | Normal Mean 1609,SE 1407 | Normal distribution only values > 0. |
| AD-Deg_Inctact forest-Degraded forest -2004-2014 | 5,991 | 4,120 | 69% | Normal Mean 5991 ,SE 2505 | Normal distribution only values > 0. |
| AD-Deg_Inctact forest-Very degraded forest-2004-2014 | 391 | 639 | 163% | Normal Mean 391 ,SE 389 | Normal distribution only values > 0. |
| AD-Deg_Degraded forest -Very degraded forest-2004-2014 | 1,627 | 2,672 | 164% | Normal Mean 1627 ,SE 1625 | Normal distribution only values > 0. |
| AD-Deg_Degraded forest -Inctact forest-2004-2014 | 7,904 | 5,553 | 70% | Normal Mean 7904 ,SE 3376 | Normal distribution only values > 0. |
| AD-Deg_Very degraded forest-Degraded forest -2004-2014 | 3,254 | 3,780 | 116% | Normal Mean 3254 ,SE 2298 | Normal distribution only values > 0. |
| AD-Deg_Inctact forest-Degraded forest -2018-2021 | 391 | 639 | 163% | Normal Mean 391 ,SE 389 | Normal distribution only values > 0. |
| AD-Deg_Degraded forest -Very degraded forest-2018-2021 | 782 | 900 | 115% | Normal Mean 782 ,SE 547 | Normal distribution only values > 0. |
| AD-Deg_Degraded forest -Inctact forest-2018-2021 | 3,505 | 3,671 | 105% | Normal Mean 3505 ,SE 2232 | Normal distribution only values > 0. |
| AD-Deg_Very degraded forest-Degraded forest -2018-2021 | 1,627 | 2,672 | 164% | Normal Mean 1627 ,SE 1625 | Normal distribution only values > 0. |
| AD-Deg_Inctact forest-Degraded forest -2022-2024 | 9,907 | 5,680 | 57% | Normal Mean 9,907 ,SE 3453 | Normal distribution only values > 0. |
| AD-Deg_Degraded forest -Inctact forest-2022-2024 | 14,233 | 7,478 | 53% | Normal Mean 14233 ,SE 4567 | Normal distribution only values > 0. |
| AD-Deg_Very degraded forest-Degraded forest -2022-2024 | 3,641 | 3,826 | 105% | Normal Mean 3641 ,SE 2326 | Normal distribution only values > 0. |
| AD-ForestGain_Natural Forest_1983-2003 | 7,996 | 5,869 | 73% | Normal Mean 7996 ,SE 3570 | Normal distribution only values > 0. |
| AD-ForestGain_Natural Forest_2004-2014 | 17,136 | 8,467 | 49% | Normal Mean 17136 ,SE 5150 | Normal distribution only values > 0. |
| AD-ForestGain_Natural Forest_2015-2017 | 3,114 | 3,615 | 116% | Normal Mean 3114 ,SE 2199 | Normal distribution only values > 0. |
| AD-ForestGain_Natural Forest_2018-2021 | 19,156 | 8,888 | 46% | Normal Mean 19156 ,SE 5406 | Normal distribution only values > 0. |
| AD-ForestGain_Natural Forest_2022-2024 | 9,700 | 6,271 | 65% | Normal Mean 9700 ,SE 3812 | Normal distribution only values > 0. |
| AD-ForestGain_Plantation Forest_1983-2003 | 1,627 | 2,674 | 164% | Normal Mean 1627 ,SE 1627 | Normal distribution only values > 0. |
| AD-ForestGain_Plantation Forest_2015-2017 | 1,627 | 2,674 | 164% | Normal Mean 1627 ,SE 1627 | Normal distribution only values > 0. |
| AD-ForestGain_Shaded cropland_2015-2017 | 391 | 639 | 163% | Normal Mean 391 ,SE 389 | Normal distribution only values > 0. |
| AD-ForestGain_Shaded cropland_2018-2021 | 1,627 | 2,674 | 164% | Normal Mean 1627 ,SE 1627 | Normal distribution only values > 0. |

### Quantification of the Uncertainty of the estimate of Emission Reductions

|  |  | Reporting Period | Crediting Period |
| --- | --- | --- | --- |
|  |  | Total Emission Reductions* | Total Emission Reductions* |
| A | Median | 4,829,807 | 9,043,384 |
| B | Upper bound 90% CI (Percentile 0.95) | 11,080,325 | 16,748,345 |
| C | Lower bound 90% CI (Percentile 0.05) | -340,822 | 2,619,661 |
| D | Half Width Confidence Interval at 90% (B – C )/ 2 | 5,710,573 | 7,064,342 |
| E | Relative margin (D / A) | 118% | 78% |
| F | Uncertainty discount | 15% | 12% |

Tools for calculating uncertainty can be accessed at the following link: https://drive.google.com/drive/folders/17cJC67Ixpiv1ivgkVL21_ZVOwNXEV9Wx

## Sensitivity analysis and identification of areas of improvement of MRV system

The following table shows each parameter's contribution to the uncertainty of the Emission Reductions. Four parameters represent 63% of the total ER’s uncertainty: i.  AD-Deg_Degraded forest -Intact forest-2022-2024 (31.2%), ii. AD-Deg_Inctact forest-Degraded forest -2022-2024 (18.0%), iii. Carbon Density of other land ( CD-other land 9.0 %), and iv.  AD-Deg_Very degraded forest-Degraded forest -2022-2024 (5.1%).

Figure 5-3: Sensitivity analysis.

| Input Variable | Corresponding Input Value |  |  | Swing | Percent Swing^2 |
| --- | --- | --- | --- | --- | --- |
|  | Low Output | Base Case | High Output |  |  |
| AD-Deg_Degraded forest -Inctact forest-2022-2024 | 6,755 | 14,233 | 21,712 | 3,717,627 | 31.2% |
| AD-Deg_Inctact forest-Degraded forest -2022-2024 | 15,587 | 9,907 | 4,227 | 2,823,507 | 18.0% |
| CD-other land | 0.00 | 27.37 | 59.69 | 1,995,687 | 9.0% |
| AD-Deg_Very degraded forest-Degraded forest -2022-2024 | 0 | 3,641 | 7,281 | 1,507,637 | 5.1% |
| AD-Defo_Secondary natural forest 2003 -Other Land-2022-2024 | 4,290 | 1,623 | 0 | 1,477,922 | 4.9% |
| AD-Defo_Secondary natural forest 1995 -Other Land-2022-2024 | 4,290 | 1,623 | 0 | 1,477,922 | 4.9% |
| AD-Defo_Secondary natural forest 1990 -Grasslands-2022-2024 | 3,924 | 1,609 | 0 | 1,477,222 | 4.9% |
| AD-ForestGain_Natural Forest_2018-2021 | 10,268 | 19,156 | 28,044 | 1,281,746 | 3.7% |
| AD-Defo_Very Degraded Forest-Unshaded Cropland-2022-2024 | 0 | 2,016 | 4,761 | 1,144,492 | 3.0% |
| ratio R::S | 0.1373476 | 0.44 | 0.7426524 | 858,444 | 1.7% |
| AD-Deg_Degraded forest -Inctact forest-2004-2014 | 13,457 | 7,904 | 2,351 | 752,855 | 1.3% |
| AD-Defo_Secondary natural forest 2020 -Unshaded Cropland-2022-2024 | 0 | 1,623 | 4,290 | 704,571 | 1.1% |
| AD-Defo_Very Degraded Forest-Other Land-2022-2024 | 0 | 3,250 | 7,022 | 634,134 | 0.9% |
| AD-ForestGain_Natural Forest_1983-2003 | 13,865 | 7,996 | 2,127 | 604,556 | 0.8% |
| AD-ForestGain_Natural Forest_2022-2024 | 3,429 | 9,700 | 15,971 | 602,868 | 0.8% |
| CD-unshaded cropland | 28.87 | 57.54 | 86.21 | 583,074 | 0.8% |
| RF-natural secondary forest gain | 27.05 | 16.69 | 6.33 | 576,879 | 0.8% |
| ratio R::S Grassland | 1.3869656 | 1.887 | 2.3870344 | 565,797 | 0.7% |
| AD-Deg_Inctact forest-Degraded forest -2004-2014 | 1,871 | 5,991 | 10,111 | 558,629 | 0.7% |
| AD-ForestGain_Natural Forest_2004-2014 | 8,670 | 17,136 | 25,603 | 517,994 | 0.6% |
| AD-Defo_Degraded Forest-Other Land-2022-2024 | 4,247 | 1,607 | 0 | 495,870 | 0.6% |
| CD-natural very degraded forest | 29.17 | 18.49 | 7.81 | 495,626 | 0.6% |
| AD-ForestGain_Natural Forest_2015-2017 | 0 | 3,114 | 6,729 | 485,184 | 0.5% |
| AD-Defo_Intact Forest-Unshaded Cropland-2004-2014 | 835 | 4,818 | 8,801 | 467,575 | 0.5% |
| AD-Defo_Intact Forest-Settlements-2004-2014 | 0 | 1,676 | 3,875 | 419,873 | 0.4% |
| AD-Deg_Very degraded forest-Degraded forest -2004-2014 | 7,034 | 3,254 | 0 | 397,220 | 0.4% |
| AD-Defo_Intact Forest-Unshaded Cropland-2022-2024 | 1,682 | 782 | 0 | 361,943 | 0.3% |
| CD-grassland | 2.98 | 20.95 | 38.91 | 338,334 | 0.3% |
| CF | 0.450401 | 0.47 | 0.489599 | 335,163 | 0.3% |
| AD-Defo_Secondary natural forest 2007-other land-2015-2017 | 4,301 | 1,627 | 0 | 310,121 | 0.2% |
| CD-natural inctat forest | 192.31 | 202.08 | 211.86 | 263,611 | 0.2% |
| AD-ForestGain_Plantation Forest_2015-2017 | 0 | 1,627 | 4,301 | 256,160 | 0.1% |
| AD-Defo_Intact Forest-Other Land-2004-2014 | 303 | 1,564 | 2,825 | 251,231 | 0.1% |
| AD-Deg_Degraded forest -Very degraded forest-2004-2014 | 0 | 1,627 | 4,299 | 242,758 | 0.1% |
| AD-Defo_Degraded Forest-Other Land-2004-2014 | 0 | 3,506 | 7,178 | 228,556 | 0.1% |
| AD-Defo_Degraded Forest-Settlements-2004-2014 | 0 | 2,017 | 4,765 | 193,275 | 0.1% |
| AD-ForestGain_Shaded cropland_2018-2021 | 0 | 1,627 | 4,301 | 190,076 | 0.1% |
| AD-ForestGain_Plantation Forest_1983-2003 | 4,301 | 1,627 | 0 | 170,773 | 0.1% |
| AD-Deg_Inctact forest-Very degraded forest-2004-2014 | 0 | 391 | 1,030 | 128,011 | 0.0% |
| AD-Defo_Intact Forest-Grasslands-2004-2014 | 0 | 391 | 1,030 | 111,619 | 0.0% |
| CD-settlements | 38.91 | 20.95 | 2.98 | 110,184 | 0.0% |
| AD-Defo_Degraded Forest-Unshaded Cropland-2004-2014 | 7,623 | 3,897 | 170 | 67,758 | 0.0% |
| CD-natural degraded forest | 63.99 | 101.93 | 139.86 | 66,746 | 0.0% |
| AD-Defo_Very Degraded Forest-Grasslands-2022-2024 | 0 | 391 | 1,030 | 60,101 | 0.0% |
| AD-Defo_Very Degraded Forest-Settlements-2022-2024 | 0 | 391 | 1,030 | 60,101 | 0.0% |
| RF-plantation forest gain | 3.43 | 13.79 | 24.15 | 48,554 | 0.0% |
| AD-ForestGain_Shaded cropland_2015-2017 | 0 | 391 | 1,030 | 45,528 | 0.0% |
| RF-shaded cropland gain | 7.77 | 10.23 | 12.69 | 42,812 | 0.0% |
| AD-Defo_Intact Forest-Other Land-2018-2021 | 0 | 1,627 | 4,301 | - | 0.0% |
| AD-Deg_Inctact forest-Degraded forest -2018-2021 | 0 | 391 | 1,030 | - | 0.0% |
| AD-Deg_Degraded forest -Very degraded forest-2018-2021 | 0 | 782 | 1,682 | - | 0.0% |
| AD-Deg_Degraded forest -Inctact forest-2018-2021 | 0 | 3,505 | 7,176 | - | 0.0% |
| AD-Deg_Very degraded forest-Degraded forest -2018-2021 | 0 | 1,627 | 4,299 | - | 0.0% |

# Transfer of Title to ERs

## Ability to transfer title

The Forest Carbon Partnership Facility (FCPF) Carbon Fund requires the Emission Reductions Program Entity to demonstrate its ability to transfer Title to Emission Reductions (ERs). Under the Methodological Framework, the term "Title to ERs" refers to the “full legal and beneficial title and exclusive right to ERs contracted for under the ERPA.” Indicator 36.2 of the Framework requires that the Program Entity “demonstrates its ability to transfer to the Carbon Fund Title to ERs, while respecting the land and resource tenure rights of the potential rights-holders, including Indigenous Peoples (i.e., those holding legal and customary rights, as identified by the assessment conducted under Criterion 28), in the Accounting Area.” Indicator 36.3 of the Framework also specifies that the Program Entity needs to demonstrate its ability to transfer Title to ERs before signing the ERPA, or at the latest, before transferring of ERs to the Carbon Fund.

In addition, Section 15.01(a) of the General Conditions Applicable to ERPAs for FCPF Emission Reductions Programs (November 1, 2014) provides that “the Program Entity shall ensure throughout the [term of the ERPA] and in accordance with the Methodological Framework that the Program Entity has the ability to transfer Title to ERs to the Trustee, free of any interest, encumbrance or claims of a Third Party other than in accordance with the ERPA.”

While the Emission Reductions Program Document ERPD (p.194) of this ER Program has established the MoFE as the national authority and REDD Implementation Center (and its subsequent version National REDD+ Center) as the duly recognized legal entity to transfer the Title to ER, further opinion was sought from the Attorney General of Nepal on this matter in view of the changes in some legislation.  The opinion from Nepal’s Attorney General concludes that “the Government of Nepal has the ability to transfer Title to Emissions Reductions as required to the Carbon Fund Methodological Framework of the Forest Carbon Partnership Facility.” The following constitutional and legal instruments define the Ministry of Forests and Environment (MoFE) as the national authority on forests and REDD+, and as the legal entity with the ability to transfer Title of ERs, including to the Carbon Fund under the Emission Reductions Program Documents (ER-PD).

Constitution of Nepal: The Constitution of Nepal Schedule 5 (27) has identified carbon as a service. According to article 57(1) and Schedule 5 (27) of the Constitution, national forest policy and carbon services shall be regulated by the Federal government in accordance with federal laws. The Government of Nepal, in February 2017, approved an unbundling report by detailing the list of exclusive and concurrent powers of the Federation, the Province and the Local Level. This report specified carbon service as the jurisdiction of the federal level and clearly stated that authority on the enhancement of carbon stocks, as well as fiscal management of the carbon service will be under federal jurisdiction.

Forest Act 2019: According to the Forest Act 2019, carbon stocks are not included under forest products and not counted as forest products/goods, but included under or counted as an environmental service, which will be managed and utilized based on Forest Regulation. Section 2 of the Forest Act defines forest carbon stocks as environmental services. Section 44 (1.a) of the Act authorizes the Government of Nepal to make arrangements for the management, utilization and distribution of benefits arising out of the environmental services, including carbon stock from emission reduction program. Similarly, the sub section (1) of section 3 states that the land ownership of the national forest lies with the Government of Nepal. As such, this Act defines forest carbon as an environmental service, and gives resource rights to communities on the products or goods produced in the forest but not to the land, intangible property, nor environmental services (e.g., carbon stocks).

FUGs prepare forest management plans and get them approved by DFOs as per Section 18 of the Forest Act 2019. According to the Community Forest Guidelines 2025 (revised), Community Forest User Groups (CFUGs) shall identify the drivers of deforestation and forest degradation and implement activities incorporated in their operational plans to address these drivers for reducing emissions from the forest sector. Section 44 (1.b) of the Act empowers the FUG to manage, use and distribute benefits arising out of the environmental services other than carbon stock and emission reduction. The use rights of communities and individuals are also established for PA Buffer Zones and Conservation Areas, according to National Parks and Wildlife Conservation (NPWC) Act, 1973. Thus, the forestry and protected area legislation provide resource rights to communities on the products or goods produced in the forest but not to the land, intangible property, nor environmental services (e.g., carbon stocks).

Environment Protection Act 2019: The Act’s Section 28 of the Act authorizes the GoN to participate in carbon trading for emission reduction and carbon stock enhancements with international mechanism, foreign Governments or entity or professional entity or private sector established under international conventions.

Carbon Trading Regulation 2025: This regulation provides a structured framework for the development, approval, implementation, and management of carbon trading projects. Further, it addresses the development and implementation of the Non market based Carbon Projects, and introduces the framework for domestic and international carbon trading mechanism in order to participate in both international and national carbon market.

Government of Nepal (Business Allocation) Regulation 2018: This Regulation authorizes MoFE to develop and approve plans and programs related to carbon service and carbon stock. The Ministry of Finance signed the ERPA with the FCPF Carbon Fund upon the request of REDD IC as the Secretariat of the NRSC, as forwarded through MoFE, pursuant to Section 2(9) of the Government of Nepal (Business Allocation) Regulation 2018. The Regulation authorizes the Ministry of Finance to sign an Emission Reductions Payment Agreement with an international entity, including FCPF's Carbon Fund.

Assessment of carbon rights

The Constitution of Nepal (2015) Schedule 5, No 27, puts the following matter under the sole jurisdiction of federal power. The following matter is dealt with by the Federal Government, as opposed to the provincial or local government: “National and international environment management, national parks, wildlife reserves and wetlands, national forest policies, carbon services.” Therefore, it is clear that the Government of Nepal has the right to transfer the Title to ER to international entities on the basis of this jurisdiction. However, it may transfer the Title to ER generated by private person or entity only when there is equitable benefit sharing, as the Constitution’s commitment in its State Policy and Principles imply that the benefits to natural resources, including the benefits from carbon services, are equitably distributed.

Carbon rights and emission reduction title

REDD+ comes under both national and international environment management. On the one hand, the activities and interventions for implementing Emission Reduction Programs, or any other programs in the National REDD+ Strategy, are guided by national laws, policies and the annual budget/program of the government. On the other hand, REDD+ is also guided by UNFCCC and any contract made by the Government of Nepal with any international entities such as the World Bank’s Carbon Fund and UN-REDD.  Therefore, carbon rights and Emission Reduction Title directly belong to national and international environment management and carbon services. They are also a matter of national forest policies as defined by the constitution.

It is clear that the Federal Government (that is, the Government of Nepal) has the right to transfer Title of Emission Reduction to international entities based on its right over national and international environment management and carbon services, as well as the right to issue national forest policies. There has been no contestation on the ability and jurisdiction of the Government of Nepal to transfer the Title of ER in the program area. The Federal Government has power over carbon services as well as the land ownership of national forests. The land under national forests, including community forests, collaborative forest, block forest or protected areas is owned by the Federal Government and authority over carbon services is vested in the Federal government (Constitution Schedule 5 (27)), no person (biological or legal) can transfer Title to ERs apart from the Federal Government. The Federal Government pursuant to other legislation (Article 25) and equitable benefit-sharing plans (Article 51) can transfer Title to ERs to any entity.

## Implementation and operation of Program and Projects Data Management System

Existing REDD Implementation Centre, under the Ministry of Forests and Environment (MoFE), is the program entity for ER program. It has been mandated as the entity for the development, implementation and management of ER programs on behalf of the Government of Nepal. It is a specialized body within MoFE and is dedicated to the implementation of the National REDD+ Strategy and associated implementation plan. Its main function is to coordinate with all stakeholders, including government agencies, civil society, academia and practitioners for the development and implementation of REDD+ in Nepal. It also serves as the operating entity for the Forest Carbon Partnership Facility (FCPF), the Forest Investment Program (FIP) and the UN-REDD Program. The following are key responsibilities of REDD IC:

- Identification of and access to national and international funds for results-based REDD+ payments. Coordination with different sectors and stakeholders for the development of REDD policies and decision-making

- Coordination for the regulation of greenhouse gas emissions from the forest sector

- Coordination for the sharing of benefits arising from REDD+

- Coordination of the implementation and monitoring of REDD+ safeguards

- Establishment and operation of national forest information system

- Coordination for the implementation of National REDD+ strategy and emissions reduction program Studies, research, and publication on REDD issues and their dissemination

- Establishment forest carbon registry and national forest monitoring system and coordination with Forests Research and Training Center for carbon measurement, report preparation and verification

- Development of a system/procedure for entering into agreements with private forest owners for encouraging them in emissions reduction program.

Projects Data Management System

REDD IC has developed National Forest Database (NFD) and National Forest Information System (NFIS, website: http://nfis.redd.gov.np/nfis) as web-based platforms for the entry and visualization of data on forest and ER projects. The REDD IC is responsible for the development and implementation of ER projects across the country’s national forests, ensuring a centralized approach that prevents multiple claims to the same ER credits. The NFIS system has been designed with a clear administrative procedure for data management, allowing for input and updates at both district and central levels. Forest officers in each district can enter management level data, while the REDD IC has appointed a focal person in every forestry directorate and district to load primary information into the system. The REDD IC plays a key role in monitoring, controlling, and managing the data, with stakeholders having access to major open-source data through NFIS. Because REDD IC (and subsequent NRC) will be responsible for ER project development and implementation for all of country’s government-owned forests, this arrangement forestalls and avoids multiple claims to ER.  It requires updating and consolidation to account for and updating project data, by including the following information:

- The proponent of the ER program or project

- Geographic boundaries of the ER program or project

- Scope of REDD+ activities and carbon pools

- The reference level used

- MRV data to specific REDD+ projects or programs

- Safeguard plans of specific REDD+ projects or programs.

Further, the Forest Research and Training Centre (FRTC) had developed Measurement, Reporting and Verification (MRV) Portal (https://mrv.frtc.gov.np/ ). Users can access the activity data and emission data from this portal.

## Implementation and operation of ER Transaction Registry

“People and Forests - A Sustainable Forest Management - Based Emission Reduction Program in the Terai Arc Landscape” is the first ER program primarily implemented in Nepal’s government-owned forests, with a minor presence (2-3%) in privately-owned forests. Accordingly, Nepal has launched its carbon registry   (https://carbonregistry.mofe.gov.np/). Nepal’s REDD entity will use its own carbon registry if the registry is fully operational. Until REDD + Registry is fully operationalized, it has been agreed that the Government of Nepal deploys the World Bank’s Carbon Assets Tracking System (CATS).

## ERs transferred to other entities or other schemes

“People and Forests- A Sustainable Forest Management -Based Emission Reduction Program in the Terai Arc Landscape” is the first REDD+ initiative implemented in Nepal within government-owned forests. It is important to note that Nepal's LEAF ER Program is another jurisdictional initiative. The accounting areas of LEAF and FCPF overlap. The crediting periods of the FCPF ER program coincide with those of the LEAF ER Program for 2022, 2023, and 2024. The volume of FCPF credits from the eight overlapping districts will be deducted from TREES issuance volume.

After the approval of this ER Reporting Report and according to the ERPA, the contract ER units will be transferred to FCPF Carbon Fund on a 100% basis. No ERs will be transferred from this program to other entities during the crediting period.

Since the Emission Reduction Purchase Agreement (ERPA) between the World Bank and the Forest Carbon Partnership Facility (FCPF) has already outlined the boundary area and the activities for emission reductions, the national authority will ensure that there is no double-counting of the emission reductions. Meaning that Nepal’s Ministry of Forests and Environment (MoFE) is the national authority that will ensure there is no double counting of emission reductions.

Rights to carbon emissions reductions/removals generated on private land.

The ER Program does not have rights to carbon emissions reductions/removals generated on private land. The ER program includes the area of national forest (Community managed and government managed forest only). However, carbon service is the sole authority of the federal government. The government has not managed the ER rights for the private sector until now. But private sector can develop the ER projects with the consent from National Designated Authority (NDA). There are no projects designed, registered, and running in the ERP area. In our record there are no projects that have taken the consent from the NDA for ER program on private land. We have published public notification related to ER program on private land (https://drive.google.com/file/d/1LEeCkQQP1o4MY7iPX5BVs2RecD0cGPS_/view?usp=sharing). The Benefit Sharing Plan (BSP) has ensured 5 % of the benefits to the private forest.

Projects using the AMS-I.C CDM and Gold Standard methodologies for Biogas and Improved Cook Stoves.

Use of the methodology AMS-I.C in Nepal.

A total of seven projects have been registered within the CDM Registry of Programs of Activities for Nepal (please refer to the table below). Among these, only project 0139, the Biogas Support Program - Nepal (BSP-Nepal) Activity-2, is included in the official project list (https://cdm.unfccc.int/Projects/projsearch.html), indicating its association with Methodology AMS-I.C. version 6. However, it is important to clarify that, despite the designation of project 0139 with methodology AMS-I.C in this list, the detailed project description (https://cdm.unfccc.int/Projects/DB/DNV-CUK1132671435.09/view ) actually specifies the methodology as AMS-I.E. version 9. Consequently, no projects utilizing the AMS-I.C. methodology have been implemented in Nepal.

Double-counting risk analysis:

An analysis of double-counting risk between biogas/improved stove projects and the FCPF Carbon Fund ER Program concluded there's no overlap. Nepal found no double counting with projects using Gold Standard methods for Black Carbon, animal manure, or biogas for thermal energy, as these methods focus on different emissions than those considered by the ERP. The Gold Standard for cook stove efficiency specifically addresses non-CO2 gases from biomass burning, which the ERP does not account for in forest degradation. Likewise, for animal manure and biogas projects, no double-counting risk exists since the ERP excludes manure emissions management.

Nepal's biogas and stove projects use AMS-I.E and AMS-II.G methods, risking double-counting between FCPF ERP, which reduces forest degradation from fuelwood demand, and CDM projects, which lower firewood demand through efficient stoves and biogas digesters. These efforts cut GHG emissions by displacing firewood and fossil fuels. However, CDM methods differ in how they calculate ERs, assuming fossil fuels would meet thermal needs without the project. ERs are then based on reducing fossil fuel use, not on decreased fuelwood demand from natural forests.

The full deduction of all CERs may be unjustified due to a lack of one-to-one equivalence among emission reductions. Also, it's uncertain if all non-renewable biomass in the CDM project comes from within the ERP accounting area.

| Parameter | Gold Standard Quantification of climate-related emission reductions of Black Carbon and Co-emitted Species [1] | Gold Standard animal manure management and biogas for thermal energy [2] | AMS-I.E. Switch from non-renewable biomass for thermal applications by the user [3] | AMS-II.G. Energy efficiency measures in thermal applications of non-renewable biomass [4] | Avoided degradation in the FCPF Carbon Fund ER Program |
| --- | --- | --- | --- | --- | --- |
| Project Boundary | The project boundary is to be the physical, geographical site of the baseline and the proposed cookstoves project and fuel collection area. | The project boundary is the physical, geographical sites of •The livestock •Animal manure management system •Biogas •Digestate treatment, usage, and or disposal | Geographical site of the use of biomass or renewable energy | The project boundary is the physical, geographical site of the efficient devices that utilize biomass. | Permanent Natural Forest Areas degradation, driven by demand for fuelwood and fodder, contributes significantly to land-based emissions. |
| Methodology Scope / Project Activity | This methodology applies to project activities that introduce efficient cookstove technologies and/or practices or switch from non-renewable to renewable biomass for meeting thermal energy requirements for cooking regimes. | This methodology applies to activities that involve the recovery and use of methane from manure and agricultural wastes that would be decaying anaerobically, emitting methane to the atmosphere in the absence of the implemented activity | The project activity will reduce GHG emissions by displacing conventionally used fuel sources for cooking, such as non-renewable woody biomass (firewood) and/or fossil fuels (kerosene and/or LPG) | The use of fuel-efficient improved cooking stoves would lead to less consumption of fuel-wood, which would thus reduce the emissions from the stove. | Degradation, driven by a forest product supply-demand gap, contributes significantly to land-based emissions. The ER Program aims to combat deforestation and degradation in Nepal by enhancing forest management and productivity. |
| Baseline Carbon Pool | Non-Renewable Woody Biomass | •Methane from manure, agricultural, and organic waste. •Delivery of thermal energy •Production of fuel, transport of fuel | Non-Renewable Woody biomass | Projected fossil fuel | AGB and BGB in permanent natural forest |
| Gases | Black Carbon (BC), Organic Carbon (OC), CO, Nox, Non-methane volatile organic compounds (NMVOCs) and Sulfates | CO2, CH4, N2O | CO2 | CO2 from fossil fuel | CO2 (Non-CO2 gases are excluded due to low accurate activity data) |

[1] Gold Standard cookstoves improved efficiency:

https://globalgoals.goldstandard.org/standards/412_V1.1_ICS_SLCP_Black-Carbon-and-Co-emitted-Species-due-to-the-replacement-of-less-efficient-cookstoves-with-improved-efficiency-cookstoves.pdf

[2] Gold Standard animal manure management and biogas for thermal energy:

https://globalgoals.goldstandard.org/433-ee-ics-methodology-for-animal-manure-management-and-biogas-use-for-thermal-energy-generation/

[3] AMS-I.E ver 9. Thermal energy production with or without electricity:

https://cdm.unfccc.int/methodologies/DB/JB9J7XDIJ3298CLGZ1279ZMB2Y4NPQ

[4]Energy efficiency measures in thermal applications of non-renewable biomass

https://cdm.unfccc.int/UserManagement/FileStorage/SG39AKEL4B2H5UJVPD86XN0I7OR1ZQ

CER deduction calculation

The double-counting analysis shows that no CER units were issued in the CDM projects, so there are no deductions for 2022, 2023, and 2024. According to the registry, no CERs were issued in this Reporting Period. Refer to the Double Counting worksheet of the Nepal TAL Integration Tool 2sd ERMR.xlsx: https://docs.google.com/spreadsheets/d/1rS0Dq_cf11jbtafWWuq-tFdnGlqSEQms/edit?usp=share_link&ouid=100991295489415488908&rtpof=true&sd=true

| Number | Project | Crediting period | CERs requested | CER requested dates | Project Boundary | FCPF ERP boundary | Project boundary overlapping | Crediting period overlapping with 2nd ERMR - 01/1/2022-31/12/2024 31/12/2021 | Deduction tCO2 [5] [6] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Improved Cook Stove Programme with Carbon Finance (ICF), Nepal 9811 | 22/05/2013 - 23/05/2023 | 129,132 | 19 Dec 2013 - 01 Apr 2015 02 Apr 2015 - 01 Apr 2017 | Districts Doti, Dadeldhura, Baitadi, Achham, Darchula, Bajhang & Bajura | Rautahat, Bara, Parsa, Chitwan, Nawalparasi, Rupandehi, Kapilbastu, Dang, Banke, Bardia, Kailali, Kanchanpur | No | No | 0 |
| 2 | Promotion of the Improved Cooking Stove (ICS) 9902 | 2022-2029 | - | - | All 75 districts of Nepal |  | Yes | No | 0 |
| 3 | Biogas Support Program - Nepal (BSP-Nepal) Activity-1 | 01/08/2018-31/07/2025 | 109,838 | 01 Aug 2018 - 31 Jul 2019 01 Aug 2019 - 31 Dec 2020 01 Jan 2021 - 31 Dec 2021 | 57 districts of Nepal |  | Yes | No | 0 |
| 4 | Biogas Support Program - Nepal (BSP-Nepal) Activity-2 | 01/08/2019-31/07/2026 | 69,973 | 01 Aug 2019 - 31 Dec 2020 01 Jan 2021 - 31 Dec 2021 |  |  | Yes | No | 0 |
| 5 | Biogas Support Program - Nepal Activity-3 | 13/12/2018 - 12/12/2025 | 204,190 | 13 Dec 2018 - 31 Jul 2019 01 Aug 2019 - 31 Dec 2020 01 Jan 2021 - 31 Dec 2021 | Districts in three regions: i)Remote/High Hills, ii) Hills and iii) Terai regions. |  | Yes | No | 0 |
| 6 | Biogas Support Program - Nepal Activity-4 |  | 197,485 | 13 Dec 2018 - 31 Jul 2019 01 Aug 2019 - 31 Dec 2020 01 Jan 2021 - 31 Dec 2021 |  |  | Yes | No | 0 |
| 7 | Efficient Fuel Wood Cooking Stoves Project in Foothills and Plains of Central Region of Nepal 4530 | 2011-2020 | 27,053 | 01/01/2013-31/12/2020 | Parsa, Bara, Rautahat, Sarlahi, Mahottari, and Dhanusa |  | Yes | No | 0 |

VCM Projects listed in the Verra registry and issue of double counting.

The MRV team checked the Verra database and when filtered for Nepal, the Verra database shows no entries registered under the category 'Agriculture, Forestry and Land Use. (https://registry.verra.org/app/search/VCS/Registered) When the team filtered the database to include all projects within Nepal on AFOLU, there is only one (ID: 4046) but has the status "Registration and verification approval requested". The Government of Nepal, through the NDA for carbon trade, the Ministry of Forests and Environment, has issued a public notice informing all concerned parties that any initiative to implement carbon trading is against the prevailing law and subject to penalties (https://drive.google.com/file/d/1LEeCkQQP1o4MY7iPX5BVs2RecD0cGPS_/view?usp=sharing). Given this context, there is no potential for double counting from the mentioned VCS projects.

# Reversals

## Occurrence of major events or changes in ER Program circumstances that might have led to the Reversals during the Reporting Period compared to the previous Reporting Period

The section has been left blank because there have been no major events or changes in ER program circumstances that might have led to the Reversals during the Reporting Period compared to the previous Reporting Period(s).

## Quantification of Reversals during the Reporting Period

INTENTIONALLY LEFT BLANK

## Quantification of Pooled Reversal Buffer replenishments

Not applicable

## Reversal Risk assessment

The first ER-MR development team estimated that the risk of reversal, both natural and human-induced, was 11%. However, this estimate did not account for the 10% default risk. Therefore, the risk of reversal was reviewed and revised. The FCPF Buffer Guidelines were used to determine acceptable values, and the discounts were applied accordingly. The updated estimate of the overall risk due to reversals is 16%.

| Risk Factor | Risk Indicators | Default Reversal Risk Set- Aside Percentage | Discount | Resulting Reversal Risk set-aside percentage |
| --- | --- | --- | --- | --- |
| Default risk | N/A | 10% | N/A | 10% |
| Lack of broad and sustained stakeholder support | Stakeholders have been engaged throughout the REDD+ process with multiple consultations at all levels. There is broad support for the ER Program across stakeholder groups. There is a low risk of land conflict with the handover of forests to the communities, and the chances of internal migrations and forest land encroachments are also low. At places in the ERPA area, localized instances of forest land encroachment were observed. To address this, the government has put in place an institutional mechanism that includes the Ministry of Home Affairs, Ministry of Forests and Environment, Nepal Police and Provincial governments. Furthermore, handing over of the national forests to CBFM groups will result in management and protection of forest areas. Nepal has recently formulated National Land Use Act 2019 and Land Use Regulation 2022. The Land Use Act has provision to safeguard land use classes and it requires a rigorous process to change and transform land use from one class to another. | 10% | 10% | 0% |
| Lack of institutional capacities and/or ineffective vertical/cross sectorial coordination | The forests are managed by 84 Division Forest Offices (DFOs) and 528 sub-division Forest Offices which are under the jurisdiction of the Provinces since 2018. This has enhanced the capacity of the provincial ministry and institutions to effectively carry out the mandates stipulated in the Nepal's constitution, Forest Act and Forest Regulation. The local levels have started establishing 'Forest and Environment Section' to implement the responsibilities as per the Forest Act and the Forest Regulation. These agencies are responsible for horizontal and vertical coordination. Since forest management is a concurrent power between federal, provincial and local governments, the Federation, Province and Local Level (Coordination and Inter-relation) Act, 2020, ensures collaboration between the three levels. In addition, Forest Act, pursuant to clause 121, has mandated that province and federal forestry sector bodies coordinate and collaborate on forest and forestry issues. This risk has been classified as low. There is a broad buy-in for the ERPD process, and Annex 10 shows the strong level of commitment across ministries for the ER Program. However, the evolving process of devolution represents a risk in terms of the vertical collaboration between the federal, provincial, local l and the community levels. The government is mitigating this risk through the development of a risk matrix that provides options for ERPD implementation under the different institutional arrangements that might emerge. | 10% | 10% | 0% |
| Lack of long term effectiveness in addressing underlying drivers | Several factors may impact the risk of reversals due to a lack of long-term effectiveness in addressing the underlying drivers. These have overall been assessed to be low risk. Infrastructure: The demand for infrastructure will keep growing with the growth in population. This risk is mitigated through land use planning intervention, which will help to minimize deforestation that may result from infrastructure development. Trade disruption and road blockages resulting in demand for fuelwood: Nepal is a landlocked country and is dependent on neighboring countries for trade. Any delay in trade and transit arrangements may exert undue pressure on forests for various issues including fuel. Nepal has substantially increased hydropower capacity and is diversifying its energy mix by expanding biogas and solar programs, which will provide alternative energy sources and minimize the pressure on forests for fuelwood. Uncontrolled grazing due to increased stray cattle: The handover of forests to communities will reduce the risks of uncontrolled grazing , but the stray cattle, especially oxen, may lead to uncontrolled grazing to some extent. Nepal has developed government policies aimed at addressing climate change impacts. These include progressive NDC with sectoral goals, NDC implementation plans, long-term strategies, and environmental regulations addressing issues such as carbon trading, reducing deforestation, forest degradation, and environmental preservation. Moreover, activities such as forest clearance by ranchers are not permitted in Nepal. The community forests are primarily focused on forest conservation, and therefore, grazing is strictly regulated. Most community forests do not allow cattle grazing, and to enforce this, many have implemented fencing. Divisional Forest Offices (DFOs) regularly assist Community Forest User Groups (CFUG) in installing fences to protect the forests from uncontrolled grazing. The Forest Operation Plans of CFUGs include a grazing management scheme and the practice of stall feeding for cattle. Additionally, communities receive support from DFOs to plant fodder trees in their farmlands. | 5% | 2% | 3% |
| Exposure and vulnerability to natural disturbances | Several factors affect the risk due to climate-related and non- anthropogenic impacts. Overall, these have been given a low risk Increased demand for timber due to non-climatic hazards such as earthquakes: Nepal lies in a seismic zone and there is potential of a big earthquake in the western region. The earthquake that struck Nepal in April 2015 saw an increased demand for home construction which caused limited pressure on forests for timber. The interventions proposed here should significantly increase the supply of timber, and this risk is considered to be low. Floods, soil erosion, and landslides in riverine forest areas: Though there is the possibility of floods and soil erosion, the impact on forest loss has historically been low. Similarly, there is a chance of forest degradation on hill slopes, but generally, areas that have less vegetation bear the brunt of landslides compared to forested areas that hold the soil. Climate change and droughts: Nepal faced acute droughts in 2009 and during winter and summer periods in 2016. The droughts have not impacted forest areas because there has been no demand for expansion of agricultural land and direct impact on tree mortality due to droughts. The ERPD also proposes several interventions to increase understanding of climate vulnerability and to address climate change impacts through improved tree species selection. Forest Fire: the frequency and intensity of forest fires are increasing due to the impact of climate change. However, the majority of forest areas are under the CBFM and this has ensured timely action to mitigate the impact and losses from the forest fires. Further, the DFOs are well-equipped and are effectively managing forest fire incidents in collaboration with local communities. Despite Nepal’s anticipation of several potential risks to its forests and changes in product usage patterns following a disaster, they are currently unable to address these risks. This is due to the absence of data from reliable monitoring systems or studies, as well as a lack of long-term, strategic approaches to managing forests' vulnerability to climate change. There are still information, planning, and program design gaps that need to be filled to effectively address this challenge. In the context of addressing the risk of forest fires in Nepal, it is important to note that previous assessments did not consider fires to be a significant source of historical emissions. Additionally, there is insufficient information to fully assess the impact of fires in the Terai on forests or emissions. While it's difficult to definitively determine whether fires pose a substantial risk of reversal based on existing information, it is worth noting that there has not been a documented case of stand-replacing fire in Nepal. This suggests that fire impacts on Nepal’s forests are generally of low intensity, primarily affecting surface fuel while leaving the canopy mostly intact. | 5% | 2% | 3% |
|  |  | Total Reversal Risk set-aside percentage |  | 16% |
|  |  | Total Reversal Risk set-aside percentage from ER-PD or previous monitoring report (whichever is more recent) |  | 16% |

# Emission Reductions available for transfer to the Carbon Fund

| Year |  |  | 2022 | 2023 | 2024 | Total |
| --- | --- | --- | --- | --- | --- | --- |
| A. | Emission Reductions during the Reporting period (tCO2-e) | from section 4.3 | 1,120,989 | 1,157,407 | 1,156,382 | 3,434,778 |
| B. | If applicable, number of Emission Reductions from reducing forest degradation that have been estimated using proxy-based estimation approaches (use zero if not applicable) |  | 0 | 0 | 0 |  |
| C. | Number of Emission Reductions estimated using Measurement approaches (A-B) |  | 1,120,989 | 1,157,407 | 1,156,382 | 3,434,778 |
| D. | Percentage of ERs (A) for which the ability to transfer Title to ERs is clear or uncontested | from section 6.1 | 100% | 100% | 100% | 100% |
| E. | ERs sold, assigned or otherwise used by any other entity for sale, public relations, compliance or any other purpose including ERs accounted separately under other GHG accounting schemes or ERs that have been set-aside to meet Reversal management requirements under other GHG accounting schemes | from section 6.4 | 0 | 0 | 0 | 0 |
|  | If applicable, any buffer replenishments | section 7.3 P | 0 | 0 | 0 | 0 |
| F. | Total ERs [(B+C)*D-E] minus, if applicable, any replenishments as per section 7.3, P |  | 1,120,989 | 1,157,407 | 1,156,382 | 3,434,778 |
| G. | Conservativeness Factor to reflect the level of uncertainty from non-proxy based approaches associated with the estimation of ERs during the Crediting Period | from section 5.2 | 12 % | 12 % | 12 % | 12 % |
| H. | Quantity of ERs to be allocated to the Uncertainty Reversal Buffer (0.15*B/A*F)+(G*C/A*F) |  | 134,518 | 138,888 | 138,765 | 412,171 |
| I. | Total Reversal Risk set-aside percentage applied to the ER program | from section 7.4 | 16% | 16% | 16% | 16% |
| J. | Quantity of ERs to be allocated to the Pooled Reversal Buffer (F-H)*I |  | 157,835 | 162,963 | 162,818 | 483,616 |
| K. | Number of FCPF ERs (F- H – J) |  | 828,636 | 855,556 | 854,799 | 2,538,991 |
| L. | Percentage of Emission reductions from enhanced Removals from afforestation/reforestation as a percentage of the total FCPF ERs [Optional if the country wishes to generate enhanced Removals] | From section 4.3 | 0.00000000% | 0.00000000% | 0.00000000% | 0.00000000% |
| M | Number of FCPF ERs from enhanced Removals from afforestation/reforestation (L * K) [Optional if the country wishes to generate enhanced Removals] |  | 0 | 0 | 0 | 0 |

# Annex 2: Information on the implementation of the Benefit-Sharing Plan

- Requirements of FCPF on Benefit Sharing Plans

Programmatic Element 5: Benefit sharing

The ER Program uses clear, effective and transparent benefit-sharing mechanisms with broad community support and support from other relevant stakeholders.

See Criterion 29; 30; 31; 32; 33 of FCPF Methodological Framework

- The General Conditions Applicable to Emission Reductions Payment Agreements (EPRAs), Section 5.01(b)(i), requires the Program Entity to “provide evidence satisfactory to the Trustee . . . that the Benefit Sharing Plan has been implemented in accordance with its terms” as an annex to the ER Monitoring Report.

- The General Conditions Applicable to ERPAs, Section 16.01(vii), also provides that “failure to observe, implement and meet all requirements contained in . . . the Benefit Sharing Plan . . . provided for under the ERPA (including any Feedback and Grievance Redress Mechanism provided for under the ER program, the Benefit Sharing Plan and/or a Safeguards Plan)” is considered an Event of Default on the part of the Program Entity.

- The information provided on the readiness of the implementation of the Benefit Sharing Plan should be up to date (not only related to the specific Reporting Period).

- The Methodological Framework, Criterion 32, requires that information on the implementation of the BSP is disclosed publicly.

- The ERPAs include an additional covenant requiring the Program Entity to “monitor and report to the Trustee on the implementation of (…) the Benefit Sharing Plan during Reporting Periods (…) The Program Entity shall first monitor and report to the Trustee on the implementation of the Benefit Sharing Plan six (6) months after receipt of the first Periodic Payment and annually thereafter. The Program Entity may coordinate the annual monitoring and reporting of the Safeguards Plans and the Benefit Sharing Plan, provided that the Program Entity notifies the Trustee and the Trustee accepts such coordinated timelines. The Trustee reserves the right to initiate a separate monitoring of the implementation of (…) the Benefit Sharing Plan annually after the date of this [ERPA] by an independent Third Party monitor.”

- Annex 2 is the primary tool for the Program Entity to provide evidence on whether the BSP has been implemented in accordance with the terms of the BSP.

- The specific content of Annex 2 should be determined based on the terms of the BSP. In general, Annex 2 should address: (i) what the agreed commitments in the BSP are; (ii) To what extent have these commitments been met; (iii) whether the agreed benefit sharing arrangements in the BSP are effective; and (iv) whether any aspects of the BSP should be changed to ensure that the agreed commitments will be achieved.

- Annex 2 should provide a synthesis of existing monitoring data collected as part of the implementation of the BSP. It is based on regular self-reporting of the Program Entity as supplemented from time to time by findings of World Bank supervision missions and independent third party monitoring initiatives including field visits, key informant interviews or periodic performance audits.

- This report should include core information and data that Program Entities are required to report, as specified in this template. Any additional information can be provided in an annex to the report depending on the specific content of the BSP. The Program Entity should submit the report six months after receiving the first payment and every year thereafter. The report will be considered valid and accepted only after all the core sections of the report using this template are filled with relevant details.

II. Monitoring and Reporting Requirements

- Overall fund disbursement: [information below is extracted from the detailed Excel Table 1 from Tab 2 prepared and submitted with this report]

Table 0-1: Summary of the funds distributed by the Program Entity (PE) as per the BSP (includes advance payments)

| Source of funds | Total commitment (contract value) | Total received by PE - Payment 1 | Total received by PE – Payment 2 | Total disbursed by PE | % Disbursed | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | (1) | (2) | (3) | (4) | (5) = (4)/((2)+(3)) |  |
| FCPF ERPA |  |  |  |  |  |  |

- BSP revision: were there any changes made to the BSP during the Reporting Period (as specified above in section II): ☐ Yes   ☐ No

If yes, please explain what changes have been made, why, and how they were consulted with the stakeholders.

- Overall summary of the BSP implementation during the Reporting Period.

Please include a brief overview of the status of BSP implementation. Detailed information can be included in an annex to this report. Please avoid repeating the same information from the BSP and provide links to external documents when possible.

  - Have there been any challenges faced in the distribution of funds to beneficiaries? If yes, please describe them and how they will or have been addressed.

  - Are the governance and funds flow arrangements as described in the BSP and operations manuals (if relevant) functioning well? Please provide as an annex a summary of meetings, consultations with stakeholders and decisions undertaken during the Reporting Period to implement and monitor the Benefit Sharing Plan.

  - Is the FGRM functioning and accessible to people in the target areas with uptake channels? Is there evidence on the number and types of feedback and grievances, and how they were addressed? Please briefly describe them here and as relevant please also include links.

  - In cases where capacity building initiatives are ongoing, confirm whether the Program Entity has completed required capacity building measures to ensure system effectiveness. What other measures are still outstanding?

- Status of Benefit Distribution [Indicate the payment period, e.g., first ER payment, etc.]. If there has been any delay in overall disbursement of funds, please explain.

  - Indicate the overall number of beneficiaries who received benefits, and please provide this information as per the arrangements described in the BSP. For instance, number of beneficiaries should be disaggregated as follows:

- type of benefits: monetary and non-monetary

- gender

- type of beneficiaries: (public organizations, private sector, IPs, communities, CSOs, etc.). geographic location of the beneficiaries (e.g., village, province)

  - Following the BSP and operations manuals (if relevant), please provide information on the mechanisms for benefit distribution. For example, a BSP may include the submission of project proposals or selected investments, community action plans, institutional work plans etc. for beneficiaries to receive benefits. For each type of benefit to be distributed, summarize the status of approval and allocation.

  - Are the eligibility Criteria to access benefits as described in the latest BSP still relevant? If there have been any revisions to the Criteria during the Reporting Period, please describe.

  - Provide information on the processes and timeline for distributing the benefits (e.g., whether the benefits are distributed one-time or continuous/periodic).

  - Provide information on any specific agreements signed with the beneficiaries for them to receive the benefits, and the key terms of such agreements.

  - Describe the mechanisms that are in place to verify how benefits are used and whether those payments provide incentives to participate in the ER program activities.

  - Describe the financial management arrangements and financial control mechanisms that are in place for recording the distribution of benefits, tracking payments, and maintaining accounting and internal controls.

4.8 Provide information on how beneficiaries are using and reporting the benefits received. Are the systems in place able to track the use of the funds? Please describe any challenges encountered.

Table 3. Total monetary benefit distribution breakdown. The table below is a generic template, please adjust as per the BSP. [information below is extracted from the detailed excel Table 3 from Tab 2 prepared and submitted with this report]

| Total monetary benefits distributed per beneficiary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Subcategory | Amount allocated |  | Amount distributed |  | Balance |  |
|  |  | (US$) | % | (US$) | % | (US$) | % |
| Government | National |  |  |  |  |  |  |
|  | Regional |  |  |  |  |  |  |
|  | Municipal |  |  |  |  |  |  |
| Private Sector |  |  |  |  |  |  |  |
| CSOs |  |  |  |  |  |  |  |
| IPs |  |  |  |  |  |  |  |
| Local Communities |  |  |  |  |  |  |  |
| Other (please specify) |  |  |  |  |  |  |  |
| TOTAL |  | 0 | 0 | 0 | 0 | 0 | 0 |

Note: Categories and subcategories should follow the BSP and as relevant for the program.

Table 4. Total non-monetary benefit distribution breakdown. The table below is a generic template, please adjust as per the BSP. [information below is extracted from the detailed excel Table 4 from Tab 2 prepared and submitted with this report]

| Total non-monetary benefits distributed per beneficiary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Subcategory | Amount allocated |  | Amount distributed |  | Balance |  |
|  |  | (US$) | % | (US$) | % | (US$) | % |
| Government | National |  |  |  |  |  |  |
|  | Regional |  |  |  |  |  |  |
|  | Municipal |  |  |  |  |  |  |
| Private Sector |  |  |  |  |  |  |  |
| CSOs |  |  |  |  |  |  |  |
| IPs |  |  |  |  |  |  |  |
| Local Communities |  |  |  |  |  |  |  |
| Other (please specify) |  |  |  |  |  |  |  |
| TOTAL |  | 0 | 0 | 0 | 0 | 0 | 0 |

Note: Categories and subcategories should follow the BSP and as relevant for the program.

Table 5. Benefit distribution to local communities divided by monetary and non-monetary and gender. The table below is a generic template, please adjust as per the BSP. [information below is extracted from the detailed excel Table 5 from Tab 2 prepared and submitted with this report]

|  | Total monetary |  |  |  | Total non-monetary |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No. of people | US$ amount disbursed | % total | TOTAL | No. of people | US$ amount disbursed | % total | TOTAL |
| Men |  |  |  |  |  |  |  |  |
| Women |  |  |  |  |  |  |  |  |
| TOTAL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Key issues and actions

  - Based on experience with the implementation of the BSP during the Reporting Period, identify and explain other key issues encountered not already described above, and how and when they are going to be addressed.

  - Are there any other emerging risks that may affect implementation, sustainability or effectiveness of the BSP? Please describe.

  - Is the Benefit Sharing distribution contributing to the core objectives and legitimacy of the ER Program and effectively rewarding/incentivizing the adoption or sustainability of Emission Reduction measures? Please describe as relevant.

  - Describe any lessons learned and, if relevant, recommendations for BSP improvement or modifications.

- If relevant, please provide information on any other Indicators as contained in the BSP and operational manuals (if relevant), including the explanation of un-met Indicators.

# Annex 3: Information on the generation and/or enhancement of priority Non-Carbon Benefits

ER programs should review potential Non-Carbon Benefits, identifying a set of priority Non-Carbon Benefits and report on the generation or enhancement of such priority Non-Carbon Benefits.  The priority Non-Carbon Benefits should culturally appropriate, and gender and inter-generationally inclusive, as relevant.

The information provided on Non-Carbon Benefits should be up to date (not only related to the specific Reporting Period).

Refer to criterion 34 and 35 of the Methodological Framework

Priority Non-Carbon Benefits

- List the set of priority Non-Carbon Benefits that were identified in the ERPD and provide necessary details on activities for generation and enhancement of these Non-Carbon Benefits. (See questions in sections 2 and 3 below for examples of details on potential specific Non-Carbon Benefits identified).

| Priority Non-Carbon Benefit | - Details on activities for generation and enhancement - Relevant Indicators as defined in ERPD - Results of Indicators (information to justify the generation and enhancement of Non-Carbon Benefits |
| --- | --- |
| … | … |

Other Non-Carbon Benefits and additional information as linked to Monitoring and Evaluation Framework

The following Indicators are to meet the monitoring requirements within the revised M&E Framework as endorsed at PC25 to be measured through the ER-Monitoring template.

Refer to Annex 4 of the FCPF Monitoring and Evaluation Framework March 2018

- Provide additional details for the following Non-Carbon Benefits as linked to Monitoring and Evaluation Framework:

Livelihood enhancement and sustainability

  - Is your CF program testing ways to sustain and enhance livelihoods (e.g. one of your program objective/s is explicitly targeted at livelihoods; your approach to Non-Carbon Benefits explicitly incorporates livelihoods)? Please provide detailed information to justify.

Biodiversity

  - Is your CF program testing ways to conserve biodiversity (e.g. one of your program objective/s is explicitly targeted at biodiversity conservation; your approach to Non-Carbon Benefits explicitly incorporates biodiversity conservation)? Please provide detailed information to justify.

Protected/conserved areas

  - What amount (in ha) of protected or conserved areas are included in your CF program area?

Has this amount increased or decreased in the last year? If so, by how much?

Re/afforestation and restoration

  - Total forest area re/afforested or restored (in ha) through the program

Finance and Private Sector partnerships

  - Update on CF program budget (as originally presented in ERPD), with updated detail on secured (i.e. fully committed) finance, in US$

    - Detail the amount of finance received (including ER payments) in support of development and delivery of your CF program. Figures should only include secured finance (i.e. fully committed): ex ante (unconfirmed) finance or in-kind contributions should not be included:

| Amount (US$) | Source (e.g. FCPF, FIP, or name of gov’t department) | Date committed (Month/Year) | Public or private finance? | ERP, grant, loan, equity or other? |
| --- | --- | --- | --- | --- |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |
| $ |  |  | Public / Private | ERP / Grant / Loan / Equity / Other |

    - Not including ER payments from the FCPF Carbon Fund, what is the value of REDD+ ER payments that your CF projects have received, and that your country has received overall? If no ER payments were received, please indicate by entering '0' in the table below.

|  | Total REDD+ ER payments received to date ($US) |
| --- | --- |
| Carbon Fund project/s (i.e. ER payments from sources other than the FCPF Carbon Fund) | $ |
| All other national REDD+ projects | $ |

    - How many formal partnerships have been established between your CF program and private sector entities? If no formal partnerships were established, please indicate by entering '0' in the table below.

Formal partnerships are defined as:

- The partnership is based on a written MoU (or equivalent), and/or

- The partnership involves tangible financial exchange/s, and/or

- The partnership involves tangible non-financial exchange/s (e.g. in-kind contributions)

|  | Established in recent Reporting Period | Total to date |
| --- | --- | --- |
| Number of private sector partnerships involving financial exchange |  |  |
| Number of private sector partnerships involving non-financial exchange |  |  |

- Other Non-Carbon Benefits and additional information

Any other activities that generate or enhance Non-Carbon Benefits in addition to those listed as earlier priority or those that are required for the Monitoring and Evaluation Framework

Policy development

  - Is your CF program engaged in contributing to the development, reform and/or implementation of policies to help institutions/people/systems/sectors? Please provide information on the approach taken and any other relevant Indicators or outcomes.

Capacity building

  - Is your CF program involved in training, education or provision of capacity building opportunities to increase the capacity of institutions/people/systems? Please provide information on the approach taken and any other relevant Indicators or results.

Safeguards Plans and Environmental and Social Management Framework (ESMF)

  - Has your CF program realized any Non-Carbon Benefits as a result of implementing the safeguards plans and ESMF? Please provide details on any relevant Indicators or outcomes.

# Annex 5: DETAILED DESCRIPTION of the application of the Reversal risk assessment tool

Please include here the detailed description of the assessment conducted to estimate the Reversal Risk reported in section 7.4 of the Monitoring Report. ER Programs shall follow the guideline on the application of the Reversal Risk Assessment Tool and the Validation and Verification of its outcomes and present the background information and results in this annex. Refer to criterion 19 of the Methodological Framework and the FCPF ER Program Buffer Guidelines

>>

Document history

| Version | Date | Description |
| --- | --- | --- |
| 3.2 | September 2025 | Tables in sections 4, 8 and 8.4 (annex 4) have been adjusted to reflect the latest updates to the FCPF ER-MR template tool. |
| 3.1.2 | March 2025 | Requirements for HFLD labelling were added to the template. |
| 3.1.1 | July 2024 | - The frontpage table and Sections 7 and 8 have been revised to reflect the provisions of the Buffer Guidelines version 4.2, namely: - The changes made to the equation applied to estimate the amount of Pooled Reversal Buffer ERs that should be cancelled in case of a Reversal; - The merge of Reversal Buffers and the Pooled Reversal Buffer; and - The recognition that not only the ERs transferred to the Carbon Fund may suffer Reversals. - Section 1.3 has been included to allow the reporting of approved material or non-material methodological deviations. - Section 2.2 has been included to allow ER Programs to report any updates to the validated monitoring plan. |
| 3.1 | July 2024 | - Annex 2 was updated for the reporting of the BSP implementation, for countries that have received ER payments. Countries should provide evidence on whether the BSP has been implemented in accordance with the terms of the agreed BSP. - Annex 3 was adjusted for a better guidance on the reporting of Non-Carbon Benefits. |
| 3 | February 2024 | Version approved virtually by Carbon Fund Participants. Changes made: - Sections 4.3 and section 8 were adjusted to be able to report ERs from Removals separately - Annex 5 was included to provide a detailed report on the application of the Reversal Risk Assessment Tool |
| 2.5 | May 2023 | - Section 4.3 has been revised to provide guidance on how to consider non-performance or Reversals from previous periods - Section 5.2 has been revised to clarify that the cumulative Uncertainty during the Crediting Period may be calculated based on propagation of errors, not montecarlo |
| 2.4 | May 2022 | - Page 1 and section 8 have been adjusted to reflect the definition of Total ERs |
| 2.3 | December 2021 | - Section 5.2 was adjusted to allow the reporting of the Uncertainty estimates for both the Reporting Period and the Crediting Period. - Section 8 has been adjusted to clarify that countries can also report ERs jointly and not only in separate calendar years. |
| 2.2 | August 2021 | - Cross-references have been corrected - Information about the start date of the Crediting Period has been requested in annex 4. |
| 2.1 | November 2020 | Aspects on Uncertainty analysis were revised based on the Guidelines on Uncertainty analysis. |
| 2 | June 2020 | Version approved virtually by Carbon Fund Participants. Changes made: - Update to consider the changes made to the Methodological Framework (Version 3.0) and Buffer Guidelines (Version 2.0) - Update to consider the changes made to the Validation and Verification Guidelines |
| 1 | January 2019 | The initial version approved by Carbon Fund Participants during a three-week non-objection period. |

https://www.worldbank.org/en/news/press-release/2025/11/16/nepal-receives-9-4-million-for-forest-carbon-credits-under-the-forest-carbon-partnership-facility

Verification report of the 1st ER-MR is available at: https://forestcarbonpartnership.org/sites/default/files/documents/23057.50_nepal_fcpf_verification_report_final_v1.4.pdf

https://www.forestcarbonpartnership.org/system/files/documents/Nepal%20ERPD%2024May2018final_CLEAN_0.pdf

While ERPD mentions 12 districts for the ER program area, presently it has 13 districts, as one of the districts (Nawalparasi) was split into two under the federal structure of the country.

ERPAs for Tranche A and B are available at: https://forestcarbonpartnership.org/system/files/documents/FCPF%20Carbon%20Fund%20ERPA-Nepal%20Tranche%20A.pdf;

https://forestcarbonpartnership.org/system/files/documents/FCPF%20Carbon%20Fund%20ERPA-Nepal%20Tranche%20B.pdf

https://forestcarbonpartnership.org/system/files/documents/Nepal%20National%20REDD%2B%20Strategy.pdf

https://giwmscdntwo.gov.np/media/pdf_upload/REDD_Strategy_Nepal_2025_bmhkz0j.pdf

https://giwmscdntwo.gov.np/media/pdf_upload/REDD_Strategy_Nepal_2025_bmhkz0j.pdf

https://www.forestcarbonpartnership.org/system/files/documents/R-PP_Nepal_revised_October.pdf

Available at https://www.forestcarbonpartnership.org/system/files/documents/Nepal%20ERPD%2024May2018final_CLEAN_0.pdf

Field Manual, 2022 is available at: https://drive.google.com/file/d/1EvrStTmfNAZVW7ewAqEXuKiUm-OzqUVB/view?usp=drive_link

Go to: https://frtc.gov.np/uploads/files/Final_FRA_data_analysis_manual_2021.pdf

https://drive.google.com/file/d/1J89y22JPS0Qfh-DSgQ4ukLTTon704Qo6/view?usp=sharing

Refer to Forest Resource Assessment in Nepal [Re-Measurement of Permanent Sample Plots] Field Manual, 2022: page 58, Chapter 5.

QUALITY ASSURANCE AND QUALITY CONTROL (QA/QC). Available at https://drive.google.com/file/d/1EvrStTmfNAZVW7ewAqEXuKiUmOzqUVB/view?usp=drive_link

Available at https://drive.google.com/file/d/19Dd_AYZMQ0iOBi33Fj-eCXJ_LVZV7uMA/view?usp=drive_link

Available at https://drive.google.com/file/d/1EvrStTmfNAZVW7ewAqEXuKiUm-OzqUVB/view?usp=sharing

Go to https://drive.google.com/file/d/1YmbHZSOIxfsnfotBbb3elCBSemh4cA8h/view?usp=drive_link

Go to: https://drive.google.com/file/d/19Dd_AYZMQ0iOBi33Fj-eCXJ_LVZV7uMA/view?usp=drive_link

Procedure document of CCDCSMA can be accessed at the following link  https://github.com/shijuanchen/forest_degradation_georgia

Tools CODED of the GEE repository can be accessed at the following link https://code.earthengine.google.com/?accept_repo=users/bullocke/coded    24 Procedure document of LandTrendr can be accessed at the following link

https://docs.google.com/document/d/1GfdMSSaU4tiDv1Sf2L8S4k2144ptpU9seB1UkPURDCA/edit

Procedure document of MTDD can be accessed at the following link https://docs.google.com/document/d/1TukNQOuEqw9OoeZgcHWUrv-ER-87TkhU9HVuV_x6HZA/edit

The Reference Data collected for the 1961 can be accessed at the following link: https://drive.google.com/drive/folders/1E11B6CfnirxruXKvdSPvV37miHgVDfw0?usp=share_link

CEO is a custom built, open-source, satellite image viewing and interpretation system. Collect Earth Online promotes consistency in locating, interpreting, and labeling reference data plots for use in classifying and monitoring land cover / land use change (see https://app.collect.earth ).

Cochran, W.G. (1977) Sampling Techniques. 3rd Edition, John Wiley & Sons, New York.

Kleinn, C. 1994. Forest Resources Inventories in Nepal: Status Quo, Needs, Recommendations. FRISP. His Majesty’s Government of Nepal, Kathmandu, Nepal.

https://drive.google.com/file/d/1yD2AuvJjAtptFTzorisLAfJWAEzY0W-D/view?usp=drive_link

DFRS, 2014 https://drive.google.com/file/d/1RhxjEkT5veJb4uvo8tsIaqoiOzO-obth/view?usp=sharing

Page 44, GFOI (2013) Integrating remote-sensing and ground-based observations to estimate emissions and removals of greenhouse gases in forests: Methods and Guidance from the Global Forest Observations Initiative: Pub: Group on Earth Observations, Geneva, Switzerland, 2014.

Page 48, GFOI (2013) Integrating remote-sensing and ground-based observations for estimation of emissions and removals of greenhouse gases in forests: Methods and Guidance from the Global Forest Observations Initiative: Pub: Group on Earth Observations, Geneva, Switzerland, 2014.

Kleinn, C. 1994. Forest Resources Inventories in Nepal Status, Qou, Needs, Recommendations. FRISP, HMGN/FINNIDA

https://drive.google.com/file/d/1YmbHZSOIxfsnfotBbb3elCBSemh4cA8h/view?usp=drive_link

Kleinn, C. 1994. Forest Resources Inventories in Nepal Status, Qou, Needs, Recommendations. FRISP, HMGN/FINNIDA

https://drive.google.com/file/d/1RhxjEkT5veJb4uvo8tsIaqoiOzO-obth/view?usp=sharing

https://drive.google.com/file/d/1YmbHZSOIxfsnfotBbb3elCBSemh4cA8h/view?usp=drive_link

CEO is a custom built, open-source, satellite image viewing and interpretation system. Collect Earth Online promotes consistency in locating, interpreting, and labeling reference data plots for use in classifying and monitoring land cover / land use change (see https://app.collect.earth ).

Cochran, W.G. (1977) Sampling Techniques. 3rd Edition, John Wiley & Sons, New York.

https://docs.google.com/spreadsheets/d/1G3ToJYNA-n8kl12GfFBurQ-m9C1bcVyr/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true

(Please read this file “READ” before accessing it)

CEO is a custom built, open-source, satellite image viewing and interpretation system. Collect Earth Online promotes consistency in locating, interpreting, and labeling reference data plots for use in classifying and monitoring land cover / land use change (see https://app.collect.earth ).

Cochran, W.G. (1977) Sampling Techniques. 3rd Edition, John Wiley & Sons, New York.

https://docs.google.com/spreadsheets/d/1G3ToJYNA-n8kl12GfFBurQ-m9C1bcVyr/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true

(Please read this file “READ” before accessing it)

(Please read this file “READ” before accessing it)

https://drive.google.com/file/d/1EvrStTmfNAZVW7ewAqEXuKiUm-OzqUVB/view?usp=drive_link

https://drive.google.com/file/d/1Van7Ir2w6qWbZgy51SNd8rSpXUyaWNJi/view?usp=sharing

https://drive.google.com/file/d/1SRAc-nT7xhc8Sf3UjZGe2zhDJi2NX5d9/view?usp=drive_link

https://drive.google.com/file/d/1zaQKB5Xl_qIqB8SXPTQ2yf50XgSH4L1n/view?usp=drive_link

Uncertainty calculation tool can be accessed at the following link: https://docs.google.com/spreadsheets/d/1AOT_hd6qw4yVZsGWiQ5MwiwnnZH4I-V4/edit?usp=sharing&ouid=100991295489415488908&rtpof=true&sd=true

Attorney General of Nepal. 2020. Note on the Ability of Program Entity to Transfer Title to Emission Reductions. October 21, 2020. Letter from Attorney General to Ministry of Forests and Environment, Ref 48-077/078.

Government of Nepal, 2017. Unbundling/Detailing of List of Exclusive and Concurrent Powers of the Federation, the State and the Local Level Provisioned in the Schedule 5,6,7,8,9 of the Constitution of Nepal (report), Federalism Implementation and Administration Restructuring Coordination Committee, February, 2017

Community Forest Guidelines 2025,  https://dofsc.gov.np/content/100/guidelines-for-community-forestry--2081/

ToR of REDD Implementation Centre, in REDD IC. 2026 April. Proactive Disclosure Magh to Chaitra 2082. Available at https://giwmscdnone.gov.np/media/pdf_upload/Proactive-disclosure%20%E0%A5%A8%E0%A5%A6%E0%A5%AE%E0%A5%A8%20%E0%A4%AE%E0%A4%BE%E0%A4%98%20%E0%A4%A6%E0%A5%87%E0%A4%96%E0%A4%BF%20%E0%A4%9A%E0%A5%88%E0%A4%A4%E0%A5%8D%E0%A4%B0%20%E0%A4%B8%E0%A4%AE%E0%A5%8D%E0%A4%AE_mawjxod.pdf   The text is the translation from original Nepali version.

The first BSP Implementation Status Report should be submitted six months after the Program Entity receives the first payment and every year thereafter (as of June 30).

You can find the Excel Template here.

Any revisions to the BSP whether major and/or minor changes should be documented in this report, and the revised BSP should be submitted to the World Bank including the FCPF Facility Management Team (FMT).